3GPP TSG-WG SA2 Meeting #167	S2-2501902

Athens, 17th Feb 2025 - 21st Feb 2025	(revision of S2-24xxxxx)

Source:	Nokia

Title:	pCR for KI#1 on the aggregation aspects (Topic#3)

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	Dummy / Rel-19

Abstract: draft normative work for KI#1 on the aggregation aspects.

# 1. Introduction/Discussion

This contribution is prepared to draft the normative work to address the aggregation aspects based on the conclusion proposed in S2-2501901.

# 2. Text Proposal

It is proposed to approve the following changes in the draft AIoT TS v0.1.0.

FIRST CHANGE

### 4.5.2	AIoT Readers

#### 4.5.2.2	RAN reader

The RAN reader may aggregate data collected from multiple AIoT devices in accordance with the aggregation assistance information provided by the AIOTF through the AIoT RAN.

SECOND CHANGE

### 4.5.3	AIOTF

The AIOTF determines the aggregation assistance information based on operator policy and the requirements provided by the AF, as specified in clause 4.5.5. The determined aggregation assistance information is then provided to the AIoT RAN to assist in the aggregation of AIoT data received from multiple AIoT devices, which may subsequently forward it to the BS reader(s), which includes:

- 	Aggregation per AIoT device type (e.g., sensor, tag)

- 	Aggregation per time window (e.g., repeatedly aggregate and report data per a time period)

-	Aggregation per reader (e.g., AIoT RAN node aggregates data per BS reader)

-	Aggregation per priority level (e.g., if particular AIoT devices are requested with priority by the AF)

-	Aggregation per number of AIoT devices (e.g., for inventory, threshold for the maximum number of AIoT device IDs required for reporting)

-	Any combination of above (if needed)

NOTE X: The AIoT service operations being aggregated from the AIoT devices share the same service identifier.

NOTE X:	If the AIOTF does not provide aggregation assistance information, the aggregation process in the AIoT RAN becomes implementation-dependent. Consequently, how AIOTF or AF interprets the aggregated data is also left to individual implementation decisions.

THIRD CHANGE

### 4.5.5	AF

The AF may provide an aggregation indication with particular requirements to the AIOTF, which include:

- 	Aggregation per AIoT device type (e.g., sensor, tag)

- 	Aggregation per time window (e.g., repeatedly aggregate and report data per a time period)

-	Aggregation per priority level (e.g., if particular AIoT devices are requested with priority by the AF)

-	Aggregation per number of AIoT devices (e.g., for inventory, threshold for the maximum number of AIoT device IDs required for reporting)

-	Any combination of above (if needed)

End