3GPP TSG-WG SA2 Meeting #167	S2-2501931

# Athens, Greece, February 17 – 21, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1561 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of mobility functionality for Support of UE-Satellite-UE communication in IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [NTT DOCOMO, Nokia, China Telecom,] CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the approved 5GSAT_Ph3-ARC work item and the conclusion of FS_5GSAT_Ph3-ARC captured in clause 8.3 of TR 23.700-29, this CR adds description on mobility functionality for Support of UE-Satellite-UE communication in IMS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of mobility functionality for Support of UE-Satellite-UE communication in IMS. Modifications in this revision (highlighted): - Modify the description of optimized media routing activation upon satellite change. - Modify the description regarding IMS AGW change. - Clarify the message used and references for functional description. - Editorial modifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An objective of the 5GSAT_Ph3-ARC work item is not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AE.2.2.1, AE.2.2.2 (new), AE.3.3 (new), AE.4.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

### AE.2.2.1		P-CSCF

The P-CSCF used for this feature is enhanced to support the following functionalities:

-	the P-CSCF determines the activation of optimized media routing and interacts with 5GS, as described in clause AE.3.

-	the P-CSCF interacts with IMS AGW, as described in clause AE.4.

-	the P-CSCF requests IMS AS to initiate SIP re-INVITE to complete IMS AGW relocation, as described in clause AE.4.

## ---Start of the 2nd Change---

AE.2.2.2	IMS AS

The IMS AS used for this feature is enhanced to support the following functionalities:

-	the IMS AS, requested by P-CSCF, initiates SIP re-INVITE to complete IMS AGW relocation, as described in clause AE.4.

## ---Start of the 3rd Change---

AE.3.3	At change of satellite

The P-CSCF is notified of the identifier of the target satellite from the PCF when the serving satellite of the UE changes as per clause AE.5.2.1. The P-CSCF determines whether optimized media routing continues to be possible between the target satellite and the satellite serving the remote UE based on the identifiers of those satellites. If there is no target satellite identifier in the notification, the P-CSCF determines that optimized media routing is no longer possible.

NOTE:	In this Release of the specification, how the P-CSCF uses the satellite identifier to derive whether optimized media routing can continue is not specified.

If the P-CSCF determines that optimized media routing can continue at change of satellite, the P-CSCF invokes Npcf_PolicyAuthorization_Update service operation to the PCF as defined in clause 4.3.6.3 of TS 23.502 [94] and as per clause AE.5.2.1 to establish a path through the target satellite for optimized media routing.

If the P-CSCF determines that optimized media routing is no longer possible e.g. due to a change of satellite, the P-CSCF executes the ground fallback procedure as per clause AE.5.2.2 to route the media via the ground.

## ---Start of the 4th Change---

AE.4.3	At change of satellite

When P-CSCF is informed that the satellite serving UE changes as defined in clause 4.3.6.3 of TS 23.502 [94] and as per clause AE.5.2, the P-CSCF selects an IMS AGW on the target satellite as per clause AE.5.2.1 or on ground as per clause AE.5.2.2, depending on whether optimized media routing continues to be possible or not.

After configuring and reserving resources in the IMS AGW on the target satellite or on ground, the P-CSCF requests the IMS AS to initiate SIP re-INVITE to inform the remote network and then the local UE to use the new IMS AGW(s) as per clause AE.5.2.1 and AE.5.2.2.

## ---End of the Changes---