SA WG2 Meeting #167	S2-2501949

17 - 21 February, 2025, Athens, Greece	(revision of S2-2500645)

Source:	ZTE

Title:	pCR to TS 23.xyz: AIoT Reader Selection

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: The paper adds description of assistance information used by AIoT RAN.

1. Discussion

In TR 23.700-13, it is concluded the following AIoT Reader selection principals:

-	Performs initial reader selection by either:

-	The AF providing information to identify readers to include in the initial reader selection network. The information can identify multiple or an individual Reader to use for the request.

NOTE 1:	Which readers are identified by the information from the AF is up to the network deployment, configuration or implementation.

- 	If a single UE Reader ID is provided by the AF via the NEF for the operation, then that is used as the selected Reader.

- 	If reader selection information or UE Reader ID is not provided, then how the AIOTF determines which readers to use is based on implementation. The AIOTF may be e.g. preconfigured with which readers to use, or take the requested target AIoT Devices last known location into account, etc.

If no readers can be selected then the request is rejected.

The paper describes AIoT Reader selection based on the principals in TR 23.700-13.

2. Proposal

It is proposed to capture the following changes into TS 23.xyz.

* * * * Start of Changes * * * *

5.3	AIoT Reader Selection

AIoT Reader selection is performed by the AIOTF when receiving AIoT service requests from the AF via the NEF.

If the AF provides information to select AIoT Reader(s), the AIOTF determines AIoT Reader(s) which can be derived from selection information provided by the AF.

NOTE 1:	Which AIoT Reader(s) are identified by the selection information from the AF is up to the network deployment, configuration or implementation.

If the AF provides target AIoT Device ID(s), the AIOTF determines AIoT Reader(s) based on the AIoT Device ID(s):

-	If the AIOTF knows the last serving AIoT Reader for the AIoT Device, the AIoTF selects this AIoT Reader; it is assumed that the AIOTF synchronizes the AIoT Device’s last serving AIoT Reader with other AIOTFs if there are multiple AIOTFs deployed in the network.

-	If the AIOTF can derive the AIoT Reader based on the target AIoT Device ID(s) and the coverage area of the AIoT Reader, the AIoTF selects this AIoT Reader.

If the AF provides target area information, the AIOTF determines the AIoT Reader(s) based on the target area information and the coverage area of the AIoT Reader(s).

NOTE 2:	If no AIoT Readers can be selected then the AIOTF rejects the service request.

* * * * End of Changes * * * *