3GPP TSG-SA2 Meeting #167	S2-2501960

# 17 - 21 February, 2025, Athens, Greece 		(revision of )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1396 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on accuracy monitoring description and procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For clause 5C.1, the analytics accuracy monitoring is achieved only by NWDAF containing AnLF, rather than AnLF or MTLF. For clause 6.2E.2, analytics consumer provides Analytics Feedback Information instead of ML Model Accuracy inforamtion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that only the NWDAF containing AnLF can perform Analytics Accuracy Monitoring. Clarify that the Analytics Feedback Information is from the analytics consumers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The corresponding descriptions are unclear and misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5C.1, 6.2E.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5C.1	General

A NWDAF may have the accuracy checking capability for Analytics and/or ML Models. The NWDAF may provide the accuracy information to consumers when requested or use it for its internal processes.

Input data is collected from Data Producer NF(s) when there is a request for inference/prediction per analytics ID in NWDAF for a specific time period in future. Ground truth data are collected from those Data Producer NF corresponding to the requested analytic ID at the time to which the prediction refers.

The ground truth data is the actual measured data observed at the time which the prediction refers to.

NOTE 1:	The ground truth data can be impacted when Analytics Feedback Information shows that an action is triggered by the analytics output in the consumer.

Analytics/ML Model Accuracy Monitoring is to be achieved by comparing the predictions using the current trained ML Model and its corresponding ground truth data i.e. the corresponding true observed events.

Analytics/ML Model Accuracy Information is to represent general performance measurements for analytics and ML Model respectively, which are composed of the number of correct predictions out of all predictions and the corresponding number of samples.

NOTE 2:	In this Release, only Analytics/ML Model Accuracy Information is supported for monitoring the performance of Analytics/ML Model.

NOTE 3:	How an MTLF/AnLF determines whether a prediction is correct is up to implementation.

The NWDAF (containing AnLF) with accuracy checking capability decides to initiate Analytics Accuracy Monitoring based on:

-	A request from an analytics accuracy consumer. The analytics accuracy consumer may be an NWDAF containing AnLF, NWDAF containing MTLF or an analytics consumer NF.

-	Analytics Feedback Information which may be provided by an Analytics Consumer NF.

The AnLF with analytics accuracy checking capability as defined in clause 6.2D is able to provide or notify the accuracy information of Analytics IDs to the analytics consumers of such service and when the analytics accuracy does not meet the analytics consumer's requirements, the analytics consumer may stop using analytics for a period of time or obtain new analytics. In addition, updated analytics for the provided Analytics IDs may be provided to analytics consumers as requested, if the updated analytics is able to be generated within the correction time period. The AnLF with analytics accuracy checking capability is as defined in clause 6.2D.1 is able to determine Analytics Accuracy Information based on e.g.:

-	Comparing predictions and its corresponding ground truth data, which are collected corresponding to the requested analytic ID at the time which the prediction refers to.

NOTE 4:	The ground truth data and the corresponding prediction is to be defined per Analytics ID.

-	Comparing changes in internal configuration for the analytics ID generation (e.g. change of data collection parameters, change in data distribution from a Data Source).

-	Previous existent records of Analytics Accuracy Information.

-	Accuracy Feedback Information provided by an NF consumer.

-	Determining analytics accuracy by comparing analytics accuracy using multiple ML Models that serve the same Analytics ID.

The MTLF with ML Model accuracy checking capability as defined in clause 6.2E is able to determine ML Model degradation based on e.g.:

-	comparing/evaluating the data: including input data, analytics output and the ground truth data either collected from various data source NFs, DCCF, AnLF, ADRF or configured by OAM; or

-	AnLF providing notifications of the Analytics Accuracy Information; or

-	AnLF providing Analytics Feedback Information of the analytics generated by the ML Model.

The NWDAF containing MTLF may reselect a new ML Model or retrain the existing ML Model and consequently notify the ML Model accuracy degradation to the ML Model consumer(s). In addition, the NWDAF containing MTLF may consider the rating of untrusted AF(s) when used as data sources.

# * * * * Next change (all new text) * * * *

6.2E.2	Procedure for MTLF-based ML Model Accuracy Monitoring

Figure 6.2E.2-1 illustrates the procedure for monitoring the accuracy of the provisioned ML Model using newly collected data. NWDAF containing AnLF may provide inference data to NWDAF containing MTLF for model accuracy monitoring and the NWDAF containing MTLF determines retraining or re-provisioning of the ML Model.

Figure 6.2E.2-1: Procedure for MTLF-based ML Model Accuracy Monitoring

1.	An analytics consumer initiates a subscription for analytics exposure services towards an NWDAF containing AnLF.

2.	The NWDAF containing AnLF requests an ML Model from the appropriate NWDAF containing MTLF, using the Nnwdaf_MLModelProvision_Subscribe service operation. The NWDAF containing AnLF may include an ML Model accuracy threshold which is used as an indicator to execute the accuracy monitoring operations as defined in clause 6.2A.2. NWDAF containing AnLF may include a DataSetTag (see clause 6.2B.1) and/or ADRF ID, which is used to store and fetch the inference data (including input data, prediction and the ground truth data at the time which the prediction refers to) from ADRF which are relevant for the accuracy monitoring and re-training/re-provisioning of ML Model.

If the NWDAF containing AnLF receives ML Model(s), the NWDAF containing AnLF sends set of tuples (unique ML Model identifier and a DataSetTag and/or ADRF ID) to the NWDAF containing MTLF by invoking Nnwdaf_MLModelProvision_Subscribe service operation for subscription modification.

3.	The NWDAF containing MTLF provides trained ML Model(s) to the NWDAF containing AnLF as specified in clause 6.2A and clause 6.2B. The NWDAF containing MTLF may include an accuracy information which is used to indicate the accuracy of ML Model during the training.

When the step 2 is for a subscription modification (i.e. including Subscription Correlation ID) and contains the set of tuples (unique ML Model identifier and a DataSetTag and/or ADRF ID), the NWDAF containing MTLF determines the relationship between the ML Model and the DataSetTag.

4.	The NWDAF containing AnLF registers the use of the ML Model with the NWDAF containing MTLF to indicate its capability of sending Analytics Feedback Information which is from the analytics consumers and/or ML Model Accuracy Information for the ML Model as described in clause 6.2E.3.

5.	Due to the registration in the previous step, the NWDAF containing MTLF may subscribe to the NWDAF containing AnLF to get Analytics Feedback Information which is from the analytics consumers and/or ML Model Accuracy Information for the provisioned ML Model by invoking Nnwdaf_MLModelMonitor_Subscribe service operation, if the service operation is supported by the NWDAF containing AnLF.

6.	The Analytics consumer may send Analytics Feedback Information in an Nnwdaf_AnalyticsSubscription_Subscribe message as described in clause 6.1.1.

7.	The NWDAF containing AnLF may send the Analytics Feedback Information which is from the analytics consumer and/or ML Model Accuracy Information received for the provisioned ML Model by invoking Nnwdaf_MLModelMonitor_Notify service operation as requested in step 5. When the NWDAF containing MTLF receives Analytics Feedback Information and/or ML Model Accuracy Information, the NWDAF containing MTLF may trigger step from 8 to 13 to enhance the ML Model accuracy.

8a-8f.	The NWDAF containing MTLF, based on the request(s) from one or more NWDAF containing AnLF or its local policy, determines whether to perform ML Model Accuracy Monitoring and re-training/re-provisioning of ML Model by collecting new data from various data sources:

-	The NWDAF containing MTLF may collect new data for ML Model Accuracy monitoring, re-training and re-provisioning from the data source NFs and DCCF by invoking Nnf_EventExposure_Subscribe and Ndccf_DataManagement_Susbscribe service operation, respectively.

-	When ADRF ID and/or DataSetTag is given by step 2, the NWDAF containing MTLF may retrieve historical data from the ADRF indicated by the NWDAF containing AnLF at step 2. by invoking Nadrf_DataManagementRetrievalRequest or Nadrf_DataManagementRetrieval_Subscribe service operation. Otherwise, the NWDAF containing MTLF may retrieves the historical data from the DCCF or the NWDAF containing AnLF by invoking Ndccf_DataManagement_Subscribe or Nnwdaf_DataManagement_Subscribe service operation, respectively.

-	If the NWDAF containing AnLF does not include a DataSetTag with ADRF ID at step 2, the NWDAF containing MLTF may request ADRF to subscribe for the collection of the analytics and data that correspond to the analytics generated by the ML Model provisioned in step 3, using the procedures defined in clause 6.2B.3.

-	The NWDAF containing MTLF may subscribe to UDM to get notification on change in the subscription data for Target of ML Model Reporting by invoking Nudm_SDM_Subscribe service operation and the UDM subscribes to the UDR to get notifications of the modification on UE subscription data by invoking Nudr_DM_Subscribe service operation.

-	The NWDAF containing MTLF may consider the data quality into the accuracy monitoring by collecting fault prediction analytics data from MDAS to determine the status of Data Source NFs, using MDA Request.

If the NWDAF containing MTLF has already collected new test data and performed ML Model Accuracy Monitoring and retraining which is triggered by other NWDAF containing AnLF(s) (for ML Model Accuracy Monitoring and retraining), the NWDAF containing MTLF, based on its internal logic, determines whether to use the data for the subscription or not.

9a-9f.	The NWDAF containing MTLF receives the requested data from various sources as requested in steps 8a-8f.

10.	Based on the collected analytics and data from steps 9a-9f, the NWDAF containing MTLF computes the accuracy using the methods described in clause 5C.1. The NWDAF containing MTLF may discard data from data sources if it detects the data quality of that source is not good. The NWDAF containing MTLF may generate prediction with the collected input data to calculate the accuracy if only input data and ground truth data are available.

NOTE:	How the NWDAF containing MTLF determines whether the data from the data source is of good quality or needs to be discarded is up to the NWDAF implementation and configuration.

11.	An accuracy report is sent to the NWDAF containing AnLF, e.g. when the reporting threshold is met by invoking Nnwdaf_MLModelProvision_Notify service operation.

12.	Based on the computed accuracy, the NWDAF containing MTLF may decide to re-train/re-provision the ML Model.

13.	When the newly generated or re-trained ML Model is ready, the NWDAF containing MTLF sends new or re-trained ML Model to the NWDAF containing AnLF by invoking Nnwdaf_MLModelProvision_Notify service operation. The NWDAF containing MTLF may send the accuracy report of the new or re-trained ML Model to the NWDAF containing AnLF.

# * * * * End of changes * * * *