3GPP TSG-WG SA2 Meeting #167	S2-2501967

Athens, GR, 17th Feb – 21st Feb, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	ENs on Ambient IoT Device Temporary ID

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: this pCR discuss ENs on Ambient IoT Device Temporary ID.

1. Introduction/Discussion

1.1 Background

Following Editor’s notes are captured in SA2 TR 23.700-13, v1.3.0:

Editor's note:	Whether the temporary ID in the AIoT NAS layer is required for the privacy protection is FFS and is pending SA WG3 decision.

Editor's note:	Whether and how the A-IoT Device Identification information will be security protected will be concluded by SA WG3.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

During the SA2#166AH e-meeting discussion, companies thought SA2 needs to address the above issues in the study phase. There were other views that SA3 is also working on the above issues so that SA2 can align with SA3 conclusions during the normative work, if any.

SA2 TR 23.700-13 has documented solutions to support “temporary AIoT Device identifier” in solution #1, #9, #13, #16, #27, #33, #34, #36, #37, to address the following SA1 requirement:

The 5G system shall be able to provide a mechanism to protect the privacy of information (e.g. location and identity) exchanged during communication between an Ambient IoT device and the 5G network or an Ambient IoT capable UE.

1.2 Work distribution on “temporary AIoT Device identifier” between SA2 and SA3

Currently both SA2 and SA3 are considering the requirement on AIoT Device privacy in the study phase, and it is necessary to decide the work distribution between SA2 and SA3 on this issue.

The SA1 requirement on privacy protection mainly falls into SA3 working scope, therefore during the previous discussion, SA2 companies believe that SA3 should lead the work but not SA2. In such case, SA2 work is usually to coordinate with SA3, if SA3 concludes the issue in the study.

SA2 TR solutions all have the same principle to use “temporary AIoT Device identifier”, while in SA3 TR 33.713, key issue#3 “privacy” includes several solution directions as follows:

- “Temporary ID” based (7 solutions)

- “Hash” based (4 solutions)

- “Encryption” based (7 solutions)

Obviously SA3 TR solutions are more diverse than those in SA2, and the only overlapping part between SA2 and SA3 is “Temporary ID” based solution. There is also an argument from some SA2 companies that if SA3 concludes “Temporary ID” based solution, SA2 needs to support temporary ID based solution from architectural perspective. In particular, SA2 work needs to be done in the study phase, as part of the overall TR conclusion. It is a bit unfortunate considering the study work should be finished as soon as possible, and usually the coordination work can be left for normative work.

Proposal 1: if SA3 concludes to support the requirement on AIoT Device privacy and solution principle is using “Temporary ID”, SA2 needs to complete and decide the architectural aspect to support “Temporary ID”.

1.3 Architectural aspect to support “temporary AIoT Device identifier”

During the previous discussion, it is clear that the Ambient IoT procedure for inventory or command service can work without using AIoT Device temporary ID. If AIoT Device temporary ID is supported, the overall design should minimize the impact on AIoT Device and impact on existing TR conclusions.

Clause 8.3.4 concludes “Principles on the procedures to support AIoT services”. In the framework of it, if temporary ID is used instead of permanent ID whenever applicable, it is necessary to consider the following issues:

- AIoT Device temporary ID allocation/reallocation;

- Use of AIoT Device temporary ID;

- Loss of synchronization of AIoT Device temporary ID between AIoT Device and AIOTF;

1.3.1 AIoT Device temporary ID allocation/reallocation

Support of allocation of AIoT Device temporary ID is optional to operator. If supported, it is assumed AIOTF is responsible for AIoT Device temporary ID allocation/re-allocation. The trigger for AIoT Device temporary ID allocation/reallocation is AIOTF implementation specific.

There is no assumption that an AIoT Device temporary ID can only be used once i.e. transmitted once on the air interface.

AIOTF provisions the AIoT Device with the allocated AIoT Device temporary ID using the command procedure (write operation with an indicator the data has a specific usage). After provisioned with AIoT Device temporary ID, the AIoT Device stores it locally in the non-volatile memory. The AIoT Device should acknowledge to the AIOTF the success of provisioning the AIoT Device temporary ID..

When receiving the allocated AIoT Device temporary ID, if there is already a temporary ID stored in the Device memory, the AIoT Device should overwrite it with the newly-provisioned one.

After successfully allocating the AIoT Device temporary ID, AIOTF may store it locally in the device context, and may use the AIoT Device temporary ID to inventory the AIoT Device one or several times.

However, the AIoT Device may move to a new area which are under the coverage of another/new AIOTF. When this happens, the new AIOTF is impossible to know the previously allocated AIoT Device temporary ID, because it cannot be transferred from the old AIOTF to the new AIOTF.

To mitigate the issue, the AIOTF needs to store the allocated AIoT Device temporary ID into the ADM, as part of the AIoT Device profile data. By this means, the new AIOTF may retrieve the AIoT Device temporary ID from the ADM and use it in the inventory procedure.

1.3.2 Use of AIoT Device temporary ID

If AIOTF has the AIoT Device temporary ID, it may send the AIoT Device temporary ID to the AIoT RAN and use it to page the AIoT Device on the air interface.

Example of use AIoT Device temporary ID is shown as below:

Figure: example of use AIoT Device temporary ID

1.3.3 Loss of synchronization of AIoT Device temporary ID between AIoT Device and AIOTF

There are several cases may lead to loss of synchronization of AIoT Device temporary ID between AIoT Device and AIOTF, at least including the follows:

- During temporary ID allocation, AIoT Device successfully stores the temporary ID into the memory but fails to acknowledge it to the AIOTF;

- AIOTF cannot locate the ADM that stores the AIoT Device data profile;

In the above two cases, the network does not have the AIoT Device temporary ID for the device. It is a more general case for the very first time the AIoT Device was inventoried / commanded.

Network should use the AIoT Device permanent ID to page the Device therefore the AIoT Device privacy cannot be protected.

1.4 Conclusions

Depending on SA3 TR conclusion on key issue 3 “privacy”, if it concludes to support the requirement on AIoT Device privacy and solution principle is using “Temporary ID”, SA2 concludes architectural aspect of support “Temporary ID” with the following principles:

- Support of allocation of AIoT Device temporary ID is optional to operator. If supported, it is assumed AIOTF is responsible for AIoT Device temporary ID allocation/re-allocation. The trigger for AIoT Device temporary ID allocation/reallocation is AIOTF implementation specific.

- There is no assumption that an AIoT Device temporary ID can only be used once i.e. transmitted once on the air interface.

- AIOTF provisions the AIoT Device with the allocated AIoT Device temporary ID using the command procedure (write operation with an indicator the data has a specific usage). After provisioned with AIoT Device temporary ID, the AIoT Device stores it locally in the non-volatile memory. The AIoT Device should acknowledge to the AIOTF the success of provisioning the AIoT Device temporary ID..

- After successful allocation of AIoT Device temporary ID, AIOTF may stores it locally in the AIoT Device context and may store it in in the ADM.

- When receiving the allocated AIoT Device temporary ID, if there is already a temporary ID stored in the Device memory, the AIoT Device should overwrite it with the newly-provisioned one.

NOTE 1: Provisioning of AIoT Device temporary ID may be unsuccessful i.e. AIoT Device does not receive the allocated AIoT Device temporary ID, due to various reason.

- If AIOTF has the AIoT Device temporary ID, it may send the AIoT Device temporary ID to the AIoT RAN and use it to page the AIoT Device on the air interface.

NOTE 2:	regardless of allocating temporary ID or not, one possibility for the network to protect device privacy is that the AIOTF always construct the MASK for inventory, e.g. including some fields (e.g. prefix) of the AIoT Device permanent ID, and provide it to AIoT RAN for paging. The MASK may be matched by one or multiple AIoT Devices, and the AIOTF should be able to filter the AIoT Device which is intended for inventory by AF request. This may require the operator to manage the AIoT Device permanent ID within the network.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * First change (all new texts) * * * *

8.2.X	Support of AIoT Device Temporary ID

Depending on SA3 conclusion on key issue 3 “Privacy by protecting AIoT device identifiers” in TR 33.713, if it concludes to support the requirement on protecting AIoT device identifier and solution principle is using “Temporary ID”, SA2 concludes architectural aspect of support “Temporary ID” with the following principles:

- Support of allocation of AIoT Device temporary ID is optional to operator. If supported, AIOTF is responsible for AIoT Device temporary ID allocation/re-allocation. The trigger of AIoT Device temporary ID allocation/reallocation is AIOTF implementation specific.

- AIOTF provisions the AIoT Device with the allocated AIoT Device temporary ID using the command procedure (write operation with an indicator the data has a specific usage). After provisioned with AIoT Device temporary ID, the AIoT Device stores it locally in the non-volatile memory. The AIoT Device should acknowledge to the AIOTF the success of provisioning the AIoT Device temporary ID..

- After successful allocation of AIoT Device temporary ID, AIOTF stores it locally in the AIoT Device context and may store it in in the ADM.

- When receiving the allocated AIoT Device temporary ID, if there is already a temporary ID stored in the Device memory, the AIoT Device should overwrite it with the newly-provisioned one.

NOTE 1: Provisioning of AIoT Device temporary ID may be unsuccessful i.e. AIoT Device does not receive the allocated AIoT Device temporary ID, due to various reason.

- If AIOTF has the AIoT Device temporary ID, it may send the AIoT Device temporary ID to the AIoT RAN and use it to page the AIoT Device on the air interface.

NOTE 2:	regardless of allocating temporary ID or not, one possibility for the network to protect device privacy is that the AIOTF always construct the MASK for inventory, e.g. including some fields (e.g. prefix) of the AIoT Device permanent ID, and provide it to AIoT RAN for paging. The MASK may be matched by one or multiple AIoT Devices, and the AIOTF should be able to filter the AIoT Device which is intended for inventory by AF request. This may require the operator to manage the AIoT Device permanent ID within the network.

# * * * * End of changes * * * *