3GPP TSG-WG SA2 Meeting #167	S2-2501968

Athens, GR, 17th Feb – 21st Feb, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	Clarifications on aggregation assistance information

Document for:	Discussion

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Overview of the different types of proposed aggregation control information and pCR for the TR for them.

1. Introduction/Discussion

1.1	Status after SA2#166ahe

During the discussion on S2-2500463 during SA2#166ahe there was discussions about whether to guide the AIoT Reader aggregation or whether to leave it to implementation. Also, part of the discussion was how to understand the options from the AIOTF to guide the AIoT Reader.

S2-2500463r11, which was ultimately noted, proposed this final text:

f. The AIOTF may provide the aggregation assistance information to the AIoT RAN to assist in the aggregation of AIoT data received from AIoT device(s). The AIOTF may determine the aggregation assistance information based on operator policy and/or request from the AF, which includes: - Aggregation per time window (e.g., aggregate and report data for a time period) - Aggregation per reader - Aggregation per Number of AIoT devices (e.g. for inventory, threshold for the maximum number of AIoT Device ID(s) required for reporting) - Any combination of above (if needed) NOTE X: The AIoT service operations being aggregated from the AIoT devices share the same Correlation identifier. NOTE X: If the AIOTF does not provide the aggregation assistance information, the aggregation in AIoT RAN is up to RAN implementation.

1.2	What the assistance information triggers

All the above options assume that the AIoT Reader sends multiple Inventory reports, as shown in Figure 6.5.2.1-1 of TR 38.769, reproduced below.

Figure 6.5.2.1-1: Message flow for A-IoT Inventory in Topology 1

The assistance information is used by, in this case, the A-IoT RAN node to determine when to trigger an Inventory Report in steps 4a/4b.

1.3	Types of Assistance Information

As can be seen from S2-2500463r11 contained 3 assistance information types:

- Aggregation per time window - Aggregation per reader - Aggregation per Number of AIoT devices

We provide an understanding of each of the assistance information types and how they help.

Aggregation per time window

This is the interval between Inventory Reports when the inventory is ongoing, it can be seen as the time between step 4a and 4b in the reproduced Figure 6.5.2.1-1 when devices are being located by the inventory process in RAN. We assume that there must be AIoT devices found, i.e., if the inventory is ongoing and no devices have been found there is no need to report them.

For example, if an inventory procedure will take 5 seconds, then the AIoTF could set this time to 1 second, so that the results come in batches every 1s for processing, smoothing the flow of results into the AIOTF.

This allows the RAN to start passing results in a timely way to the AIOTF, and allows the AIOTF to start processing inventory results, while the inventory procedure is still ongoing. As it is not related to the number of results only the time, it can be useful to handle e.g., latency within the process.

Aggregation per reader

This is related to the AIoT RAN and the readers on each AIoT RAN node, and allows batches of results to be passed to the AIOTF when a Reader has completed the request. In our view this is not to provide an order in which each reader should be used by AIoT RAN.

Figure 1.3-1 Logical view of AIOTFs and an AIoT RAN node with multiple readers

For example, in a deployment if an AIoT RAN node has multiple readers and multiple readers are used for a specific inventory request, then once a single reader has completed the request the results from that reader can be passed to the AIOTF, without waiting for all the readers used for the request to complete the operation. This can become useful when targeting a large area and the readers can perform an operation without all the readers targeted by a request having to be “idle” before starting. For example, in a large building with multiple floors and multiple rooms on each floor, instead of waiting for every reader in the building to be idle before starting a building wide inventory, the AIoT RAN may choose to sequence the readers used for the inventory room by room or floor by floor, therefore allowing efficient resource usage.

Then in this case instead of waiting for the operation be fully completed, the AIOTF request the AIoT RAN to provide results once an inventory operation has been completed per reader, saving buffering effort in AIoT RAN and allowing the AIOTF to process results while the inventory continues on other readers.

There does not seem to be a clear need for the AIOTF to provide this, as the time window, number of devices or both can achieve a smoothed and timely delivery of results.

Aggregation per Number of AIoT devices

This is a threshold for the number of devices to report. While an inventory is ongoing AIoT Devices will respond, and once a certain number of AIoT Devices have responded an inventory report is generated.

This is similar to per time window, except instead of a having a batch of results, e.g. every second, it produces batches when, for example, 20 devices have responded.

This allows the RAN to start passing results in sized batches to the AIOTF, and allows the AIOTF to start processing those batches of inventory results, while the inventory procedure is still ongoing. This can be useful to smooth the processing of results from inventory, instead of a large peak when the inventory is completed.

Finally, there was the following option:

- Any combination of above (if needed)

Any combination

In this case we assume the AIoT RAN is provided with multiple assistance information, and as soon as one of the assistance information can be met, then the inventory report is generated.

For example, in order to maintain latency a time may be given (say 1 second) but to also help smooth load a number may also be provided (say 20 devices), so when either 1 second has passed, or 20 devices are available to report in the inventory report it is generated, and time is reset again.

From this it can be seen that a combination can be useful to control load, latency and batches of reports from multiple readers while an inventory operation is ongoing.

1.4	Default Behaviour

There was the following NOTE in S2-2500463r11:

NOTE X: If the AIOTF does not provide the aggregation assistance information, the aggregation in AIoT RAN is up to RAN implementation.

It is our understanding that the note intendents to cover the cases when the AIOTF does not provide assistance information to the AIoT RAN node, therefore it can choose when to send inventory reports to the AIOTF. This is a good aspect to clarify.

1.5	Handling Multiple Requests

There was the following NOTE in S2-2500463r11:

NOTE X: The AIoT service operations being aggregated from the AIoT devices share the same Correlation identifier.

This note is saying that the assistance information only applies to a single Inventory Request, as shown in Step 1a of the preproduced Figure 6.5.2.1-1, but in written in terms of correlation identifiers, and results from different requests are not mixed/aggregated in the Inventory Reports, i.e. each inventory request is its own separate process.

This allows a single AIOTF when making multiple inventory requests to an AIoT RAN node, or different AIOTFs making requests to AIoT RAN, as shown in figure 1.3-1, to provide assistance information only per request.

This enables assistance with managing the responses for different requests, and the assistance information is dynamic, rather than set once per AIOT RAN node by the AIOTF.

1.6	Conclusion

Some of the different aggregation information types can be useful to the AIOTF to assist both AIoT RAN and itself for handling inventory responses.

If it is determined to go ahead with aggregation control information from the AIOTF, then a proposal for how that can be captured is included, based on S2-2500463r11 with some tidy-ups and removal of unneeded options.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-130.

# * * * * First change * * * *

8.1	Conclusion on Key Issue #1

8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

In this Release, normative work will take place for Topology 1 and no normative work will take place for Topology 2.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

1.	A new core network function is introduced to support Ambient IoT (e.g. AIOTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIOTF registers itself in the NRF with its NF profile, this is to enable the discovery of AIOTF instances e.g. by an NEF. The NF profile at least includes the AIOTF ID/address, NF type, information used to allow the NEF to discover AIOT instances, e.g. based on the target area information in AF request. Other details of the NF profile, if needed, will be completed in normative phase.

b.	For topology 1, the AIOTF selects AIOT RAN nodes and optionally a list of the BS readers. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

For topology 1, NGAP is terminated at an AIOT RAN node. An AIOT RAN node that supports one or more BS readers, may report its supported BS reader information e.g. BS reader IDs and their corresponding service areas. The OAM may configure the AIOTF with BS Reader information including e.g. their corresponding service area when the BS Readers are integrated in the network.

NOTE 2:	Whether RAN3 will specify BS reader information (e.g. BS reader ID, service area) reporting to AIOTF over NGAP depends on RAN3 decision and if so SA WG2 may include this feature in SA WG2 specifications.

NOTE 3:	If the request from the AIOTF to the AIOT RAN does not include a BS reader list, then AIOT RAN can use all available BS readers.

NOTE 4:	The BS reader’s service area needs to coordinate with RAN WG(s).

c.	The AIOTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIOTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

e.	The AIOTF may provide the following AIoT RAN assistance information to AIoT RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

NOTE 5:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 6:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

NOTE 7:	Further AIoT RAN assistance information can be added during the normative phase and in cooperation with other WGs if necessary.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

f.	The AIOTF may provide aggregation assistance information to AIoT RAN to assist in the aggregation of AIoT data received from AIoT device(s). The AIOTF may determine the aggregation assistance information based on operator policy and/or request from the AF, which includes:

-	Aggregation per time window (e.g., aggregate and report data for a time period),

-	Aggregation per Number of AIoT devices (e.g. for inventory, threshold for the maximum number of AIoT Device ID(s) required for reporting),

-	Any combination of above.

NOTE X: The AIoT service operations being aggregated from the AIoT devices share the same Correlation identifier.

NOTE Y:	If the AIOTF does not provide the aggregation assistance information, the aggregation in AIoT RAN is up to RAN implementation.

g.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 8:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIOTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

5.	The AIOTF may store and manage the AIoT device related information (also known as device context information) locally that includes e.g. the AIOT device permanent ID, the last known reader information of the AIoT device. The last known reader information can be used to support the AIOTF to select the serving reader to forward the message towards the specific AIoT device(s).

NOTE 9:	The storage of security related information at the AIOTF is up to the coordination with SA WG3 later.

# * * * * End of changes * * * *