3GPP TSG-WG SA2 Meeting #167	S2-2501974

Athens, GR, 17th Feb – 21st Feb, 2025	(revision of S2-2500926)

Source:	Huawei, HiSilicon, Qualcomm?

Title:	pCR for TS 23.xyz: Ambient IoT Device Identifier

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: The Ambient IoT Device Identifier is introduced to the specification.

1. Introduction/Discussion

Based on the conclusion in TR 23.700-13, the Ambient IoT Device Identifier is defined. The components in the Ambient IoT Device Identifier are renamed in order to make it clear the usage of each component of Ambient IoT Device Identifier.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xyz.

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service requirements for Ambient power-enabled IoT".

[3]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[X]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[Y]	GS1 TDS Release 2.1: "EPC Tag Data Standard".

# * * * * Next change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AIoT	Ambient IoT

AIOTF	Ambient IoT Function

EPC	Electronic Product Code

# * * * * Next change (all new text) * * * *

5.7.2	Ambient IoT Device Permanent Identifier

A globally unique Ambient IoT Device Permanent Identifier shall be stored in each Ambient IoT Device.

The Ambient IoT Device Permanent Identifier includes following components:

-	ID Type consisting of:

-	information indicating whether the network identifier is included or not in the Domain Information;

-	information indicating whether a 3rd party identifier is included or not in the Domain Information;

-	information indicating whether the ID Information contains an EPC or unstructured information.

-	 Domain Information, consisting of one or both of the following:

-	A PLMN identifier with optionally a Network Identifier (NID), or a Network Identifier (NID) as specified in TS 23.003 [X];

-	A 3rd party identifier used to identify a 3rd party.

-	ID Information of flexible length to distinguish different Ambient IoT Devices within the scope identified by the Domain Information, consisting of one of the following:

-	EPC as defined in clause 14 of [Y];

-	Unstructured information.

NOTE 1:	The format of unstructured information is beyond the scope of 3GPP.

NOTE 2:	The coding for the above information is left to stage 3.

# * * * * End of changes * * * *