3GPP TSG-WG SA2 Meeting #167	S2-2501976

Athens, Greece, 17 – 21 Feb, 2025	(revision of S2-2500928)

Source:	Huawei, HiSilicon

Title:	pCR to TS 23.xyz: Procedure for Ambient IoT Command Service

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: The Ambient IoT Inventory Service is introduced to the specification. In addition, the procedure will consider both direct architecture and indirect architecture.

1. Introduction/Discussion

The pCR aims to provide the procedure call flow and description of Ambient IoT Command Service. The procedure uses direct architecture option as the baseline with additional reference to capture the difference in indirect architecture option.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xyz.

# * * * * First change (all new texts) ***

6.2.3	Command Procedure

Figure 6.2.3-1 described the command procedure.

The command procedure uses procedures for AIOTF and AIoT Reader communication for communication between an AIOTF and AIoT RAN defined in clause 6.3.

Figure 6.2.3-1: Command Procedure

-	Steps 1-13: The Ambient IoT Inventory procedure is specified in clause 6.2.2 with following addition and modifications to the steps in Inventory Procedure:

-	In step 1: the AF sends Nnef_AIoT_Read/Write/PermanentDisable request to NEF, carrying the AF ID, Command related information.

Command related information includes the information related to the type of Command:

-	if the service operation is Read, the information related to the Read command includes address information (e.g. start address and the length of data) of the data to be read.

-	if the service operation is Write, the information related to the Write command includes the data to be written and the corresponding address information where the data to be stored (e.g. start address and the length of data).

-	if the service operation is Permanent Disable, the information related to the Permanent Disable command may include the security parameters.

NOTE 1:	The security related aspects for the commands depends on SA WG3 decision.

-	In step 2: The NEF may use the last known AIOTF information stored in the ADM if the command targets a specific device to determine which AIOTF to use for the request.

-	In step 3: NEF sends the Naiotf_AIoT_Read/Write/PermanentDisable request to the selected AIOTF.-	In step 4: The AIOTF may have a context for the targeted AIoT Device to assist with Reader Selection, e.g. containing the last known Reader or AIoT RAN.

-	In step 7: Command Indication is additionally included in the Inventory Request to indicate that the procedure is for command service operation.

-	Step 14: AIOTF generates an AIoT Device NAS message including the Command based on the command related information from the AF. The AIOTF sends Command Request to the AIOT RAN including the correlation ID, the temporary NGAP-like ID corresponding to the AIoT Device and the AIoT Device NAS message.

-	Step 15: AIOT RAN sends the AIoT NAS from the AIOTF to the AIoT Device.

-	Step 16: Based on the Command included in the AIoT NAS, the AIoT Device generates AIoT NAS including the Command related response.

-	if the command is Read command, the Command related response includes the stored data indicated by the Read command.

-	if the command is Write command, the Command related response includes the ACK indicating whether the Write command is received successfully.

-	if the command is Permanent Disable, the Command related response includes the ACK indicating whether the PermanenetDisabled command is received successfully.

The AIoT Device sends D2R transmission message including the AIoT NAS message.

-	Step 17: AIOT RAN sends Command Response to the AIOTF including the correlation ID, the temporary NGAP-like ID and the AIoT NAS message from the AIoT Device.

NOTE 2:	Steps 14 to 17 will be performed for each of AIoT Device corresponding to the same AF request.

-	Step 18: The AIOTF completes the requested AIoT Command request. The AIOTF reports the progress of the AIoT Command request to the NEF by sending the Naiot_AIoT_Notify message including a list of Ambient IoT Permanent Device ID(s) and the corresponding Command related response as specified in clause 8.2.

-	Step 19: The NEF informs the AF of the outcome of the AIoT_Command service operation request by sending the Nnef_AIoT_Notify message including the Ambient IoT Permanent Device ID(s) as specified in clause 8.3.

# * * * * End of changes * * * *