SA WG2 Meeting #167	S2-2501984

# 17 - 21 February 2025, Athens, Greece

Source:	ZTE

Title:	Proposals of VFL training trigger with possible compromise

Document for:	agreement

Agenda Item:	19.15.2

Abstract of the contribution: This discussion paper presents some possible compromise proposals for VFL training trigger.

Discussion

Key principles when designing the cases VFL training trigger:

- The AnLF should not aware the training logic of any models

- As we agreed in Rel-17 to split NWDAF to AnLF and MTLF, one for inference and one for training. We propose that AnLF should not aware the model is trained by VFL, implicitly or explicitly.

- The VFL client can not get the final aggregated inference result

- We agreed in the VFL inference procedure that only VFL server has the aggregated inference result. VFL client does not know the final inference result.

- Avoid complex triggers

- The triggers should be simple and easy for implementation.

- 1) VFL training triggered by internal logic of VFL sever or OAM

- 2) VFL training triggered by analytics request or inference request.

- Split the case discussion for trust and untrust AF

- The trust AF is inside operator domain and can be controlled by operator（OAM），so separate discussion is needed.

Proposals

- 1. OAM triggered VFL training

- 2.  VFL training triggered by configuration or internal logic

- 3.  VFL training triggered by analytics request, the NWDAF it self can be the VFL server

- 4.  When an analytics request is received, the NWDAF (acting as a VFL proxy) determines that VFL  should be used. However, since the VFL proxy itself cannot function as a VFL server, it must identify another NWDAF or a trusted AF that can act as the VFL server. This selection can be based on local configurations or via NRF.

- Once a suitable VFL server is found, the VFL proxy sends a VFL training and inference indication to it. The VFL server then conducts VFL training and inference, subsequently returning the inference results to the VFL proxy. Finally, the VFL proxy forwards the analytics results to the consumer NF.

- 5. Similar to case 4, but VFL proxy find a untrust AF in this case and the VFL training and inference indication and inference result are transmitted via NEF.

- Important points for case 4 and 5:

- It is up to implementation whether VFL proxy find and determine to use a NWDAF, trsut AF or untrust AF as VFL server for VFL training and inference.

- The VFL proxy NWDAF must not register its support for VFL client capability to the NRF. This ensures that it does not get mistakenly selected by a VFL server. The VFL client shall not know the final aggregated inference result from VFL server.

- The VFL proxy NWDAF must include both MTLF  and AnLF .This is because an AnLF alone does not have knowledge of the model training logic.

- Before the VFL proxy NWDAF registers support for an Analytics ID in the NRF, it must establish an offline agreement(e.g. SLA) with an NWDAF (includes MTLF and AnLF) or a trust AF/untrust AF to serve as the VFL server, if the only way of generating Analytics result inside NWDAF is done by VFL. This is to avoid the possibility that consumer NF select a NWDAF that can not return Analytics result. In the case of untrust AF, the NEF needs to maintain the SLA.

- Another proposal is we leave the trigger of VFL training to Rel-20 if we can not reach any consensus in Athens.