3GPP TSG-SA2 Meeting #167	S2-2501985

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3891 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transition between S&F and normal for the UE in ECM-CONNECTED |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The exception sheet in SP-241918 for 5GSAT_Ph3-ARC includes for S&F operation the following: • (2b) – Functional specification of expected UE and network behaviour when transitioning between operation in S&F mode and operation in normal mode. During SA2#166AH-e, companies discussed the UE and satellite requirements for transition between S&F mode and normal mode, and the majority agreed that if the most recent attach or TAU of a UE with S&F capability was with a satellite operating in S&F mode, the UE should perform an attach when accessing either TN or normal NTN for EPS and vice versa. However, for the UE in ECM-CONNECTED and the satellite operation mode changes, the corresponding UE/network behaviours is unclear. As the UE UE in ECM-CONNECTED cannot initiate Attach procedure, it is proposed that the MME onboard may detach the UE in ECM-CONNECTED if the satellite operation mode changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of UE and network behaviours for transition between S&F mode and normal mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE and network behaviours for transition between S&F mode and normal mode are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.13.9	Transition between S&F Mode and Normal Mode

If the most recent attach or TAU of a UE with S&F capability was with a satellite operating in S&F mode, the UE should perform an attach when accessing either TN or normal NTN (i.e. satellite not operating in S&F mode) for EPS. If the most recent attach or TAU of a UE with S&F capability was with a TN or normal NTN (i.e. satellite not operating in S&F mode) for EPS, the UE shall perform an attach when accessing a satellite operating in S&F mode.

When a satellite that is supporting regenerative payload loses its feeder link, the onboard MME may detach the UE that is in ECM-CONNECTED state. If there’s only eNB onboard the satellite, the onboard eNB should release the RRC connection with the UE. Depending on operator policy and regulatory requirements, the satellite, if S&F capable, may start support for S&F mode.

When a satellite that is supporting S&F mode obtains a feeder link, then, depending on operator policy and regulatory requirement, support for S&F mode may cease and support for normal NTN access may start. If the UE is in ECM-CONNECTED state, the onboard MME should detach the UE with the Detach type set to re-attach.

# * * * * End of changes * * * *