3GPP TSG-WG SA2 Meeting #167	S2-2501988

Athens, Greece, 17-21 February 2025	(revision of S2-25xxxxx)

Source:	Xiaomi

Title:	Discussion on UE accessing to a satellite not in the monitoring list

Document for:	Agreement

Agenda Item:	19.1.2

Work Item / Release:	5GSAT_Ph3-ARC / Rel-19

Abstract: This paper analyses

1. Discussion

SA2#166AH-e has approved on how to support S&F monitoring list in Store and Forward Satellite operation (S2-2501339), but has not reached consensus on how UE behaves when receiving a S&F monitoring list and accessing a satellite that support S&F satellite operation and is not on the monitoring list. The issue left is as follow:

For a UE which indicates support of Store and Forward Satellite operation and when an MME is operating in S&F Mode: - If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation. - If the UE is rejected with a reject cause indicating it is due to S&F operation, the UE’s EMM state shall remain unchanged. - The MME may provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting or rejecting a NAS procedure. NOTE 3: How the MME determines the S&F Wait Timer and S&F Monitoring List is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, etc. When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause. The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from. NOTE A: The S&F Wait Timer or S&F Monitoring List doesn’t affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode. NOTE B: When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. If the MME does not send a S&F Monitoring List or S&F Wait Timer in a NAS procedure, the UE deletes any stored S&F Monitoring List or S&F Wait Timer. NOTE Y: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. - The MME may indicate to the UE the Uplink S&F estimated delivery time in a NAS accept messages (i.e. Attach Accept, TAU Accept or Service Accept messages). NOTE 4: The Uplink S&F estimated Delivery time is the estimated to deliver the data to the ground from the time the data is sent by the UE. How UE uses this information is left for UE implementation. Editor’s Note: It is for further consideration whether specific UE behaviour or clarification needs to be added to prevent the UE from accessing satellites where the NW is not expected to keep a UE context. If the UE did not indicate Store and Forward capability in NAS signalling, if the network determines to reject the UE, the network shall reject the UE request with a cause non-specific to S&F Mode.

The UE and network may have the following potential behaviour on this issue:

Case A:

The monitoring list was received by UE from the initial attach rejection. Subsequently, the UE doesn’t follow the list and tries to access a satellite that supports S&F operation but out of the list, the basic attach operation defined in clause 4.13.9 (i.e. UE initiates attach and receives attach rejection and sends attach again) may be tried again by the UE and NW with no issue exists.

Observation 1: There is no issue exists when the UE receives a S&F monitoring list in attach rejection and subsequently accesses a satellite that supports S&F operation and is not in the list.

Case B:

The monitoring list was received by UE from attach accept/TAU accept, i.e. the UE has a successful attachment and in EMM-REGISTRED state. Subsequently, the UE may access to a satellite that supports S&F operation but out of the list and use CPSR to send the pending data. In this case, if the MME onboarding the satellite has the UE context, the data sent can be successful. If the MME onboarding the satellite has no UE context, the integrity check is going to fail, the MME-onboard will respond #9 to the UE. The UE will perform detachment followed by re-attach using the satellite.

There is no guarantee that the satellite out of the list has the UE context based on the current mechanism, using such kind of satellite to send/receive data may result in failure.

Observation 2: If a registered UE sends CPSR to a satellite that supports S&F operation but is not in the list, data sending may fail considering the satellite has not UE context.

Case C:

Before sending CPSR, the UE sends TAU request when accessing a satellite that supports S&F operation and is not in the monitoring list. In this case, if the MME onboarding the satellite has no UE context, the MME-onboard will respond TAU rejection with S&F cause to the UE. After receiving this, the UE stays in EMM-REGISTERED state and tries to access another available satellite. If the MME onboarding has UE context, the TAU is success, then the UE can send CPSR

Observation 3: If a registered UE sends TAU to a satellite that supports S&F operation but is not in the list, TAU may fail considering the satellite has not UE context.

Both TAU and CPSR can be success assuming the satellite has UE context, then UE can directly use CPSR to send data, TAU is not needed for the case.

Both TAU and CPSR may fail when the satellite has no UE context. The difference is TAU fail will not change UE registered state while CPSR fail will trigger UE detach first and UE needs to reattach for sending the data. Although TAU would help to keep the UE in EMM-REGISTERED state, the UE still can not send data until TAU success. Using TAU doesn’t seem to have any advantage in helping UE sending data.

Observation 4: Using TAU does not have any advantage in helping UE sending data.

Case D:

Since sending TAU before CPSR is not able to solve the problem of unsuccessful data delivery, how to resolve CPSR fail based on Case B can be considered. Potential enhancement is that the MME-ground can synchronize the UE context to all MMEs-on board once feeder link becomes available. In this case, it can ensure UE sending CPSR successful even if the registered UE doesn’t follow the monitoring list.

This may be not a perfect solution considering keeping UE context update to all satellites is not a good manner when a constellation contains many satellites. But considering NOTE 3 shows how to determine S&F monitoring list is up to MME implementation, the MME can determine a S&F monitoring list as large as possible to include all the S&F satellite that UE may access in the future, in this case, UE context synchronize to all satellite can be avoided, otherwise the UE context synchronize is needed. However, how to leverage the S&F monitoring list determination and whether synchronize UE context to all satellite is up to MME implementation, Case D is just considering synchronize UE context to all satellite as an optional supporting for MME.

Observation 5: To improve CPSR successful, MME-ground can synchronize UE context to all satellite of a constellation.

Based on above, Case D is proposed to solve the issue.

2. Proposal

Observation 1: There is no issue exists when the UE receives a S&F monitoring list in attach rejection and subsequently accesses a satellite that supports S&F operation and is not in the list.

Observation 2: If a registered UE sends CPSR to a satellite that supports S&F operation but is not in the list, data sending may fail considering the satellite has not UE context.

Observation 3: If a registered UE sends TAU to a satellite that supports S&F operation but is not in the list, TAU may fail considering the satellite has not UE context.

Observation 4: Using TAU does not have any advantage in helping UE sending data.

Observation 5: To improve CPSR successful, MME-ground can synchronize UE context to all satellite of a constellation.

Proposal: All satellites of a constellation can be synchronized UE context to minimize CPSR fail when UE sending data. It is proposed to agree S2-2501989 for TS 23.401.