3GPP TSG-SA WG2 Meeting #167	S2-2501993

Athens, Greece, 17 – 21 February 2025

Agenda item:	19.15.2

Source:	NTT DOCOMO

Title:	Discussion on SMF events of QoS Flow

WI/Release:	AIML_CN/Rel-19

Document for:	Discussion and Decision

# 1	Discussion

To provide the QoS and policy assistance analytics (defined in clause 6.23 of TS 23.288), QoS flow related events by SMF is one of the inputs that NWDAF may subscribe. The purpose of the events is to allow NWDAF to determine how long and how frequently QoS Flows (with a specific QoS parameter settings) are applied for the application traffic flows.

Three categories of the events are considered for this purpose:

- Start: When the application starts, the corresponding SDF is bounded to

  - an existing QoS flow; or

  - new QoS flow

- Change: When for any reason (e.g., new PCC rule association) the application SDF switches from the current QoS Flow to another QoS Flow, which may be

  - an existing QoS flow; or

  - new QoS flow

- End: When the QoS Flow is not used, i.e., it is released and the QFI is deallocated

Currently, as defined in clause 5.2.8.3.1 of TS 23.502, SMF supports QFI allocation event. This event covers case 1.2. To avoid backward compatibility issues, it is proposed to avoid changing the semantic of the existing “QFI allocation” event and new events are defined to support other cases.

For case 1.1, it is proposed to define a new “QoS Flow Binding” event which is sent when the application traffic is bound to an existing QoS Flow (i.e., no new QFI allocation).

Cases 2.1 and 2.2 can be supported by events of case 1.1 and 1.2 respectively since when the application traffic switches to another QoS flow, either a new QoS flow with new QFI is needed, which is case 1.1, or it is bounded to another existing QoS flow which is case 1.2.

For case 3, it is proposed to define a new “QFI deallocation” event which is sent when the QoS flow is terminated and the corresponding QFI is deallocated

# 2	Conclusions

The following changes are proposed:

Proposal#1: Keep the definition of the “QFI allocation” as it is.

Proposal#2: Define a new “QoS Flow Binding” event which is sent when traffic flow is bounded to existing QoS Flow.

Proposal#3: Define a new “QFI deallocation” event which is sent when the QoS Flow is released.