3GPP TSG-WG2 Meeting #167	S2-2502019

# Athens, Greece, February 17 – 21, 2025				                             (revision of S2-2501345)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1333 | rev | 5 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve some ENs in the vertical federated learning inference procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [OPPO, Nokia, Ericsson, Samsung], China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA2#166AHE: Trusted AF as the VFL server case is added based on the NWDAF as the server case since both the case have many overlap regarding the service operation and the VFL logic. Resolving the following EN by adding the sentenc to let server can internally store information about the features considered when deriving the analytics results together with a timestamp to identify the analytics results to enable to subsequently trace the origin of analytics results. Editor's note: It is FFS how the origin of analytics results can be traced and explained if not all clients participate and results are not satisfying. SA2#167: In VFL inference procedure, since analytics consumer is unable to be aware whether the Analytics output is generated by VFL method when it subscribes Analytics IDs, the consumer may also subscribe the Analytics accuracy information at the same time. In this case, the AnLF could provide the Analytics accuracy information based on ML model accuracy information (i.e. ML model metric) which is able to be calculated as the same logic as training process. Thus, the related accuracy monitoring procesures also need to be added in VFL inference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the trusted AF as the VFL server case. Remove the following ENs: Editor's note: The service name between the AnLF and untrusted AF as VFL server is FFS. Editor's note: It is FFS how the origin of analytics results can be traced and explained if not all clients participate and results are not satisfying. SA2#167: - Add Analytics accuracy monitoring in VFL inference. - Improve some wording on description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The VFL inference procedure is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.4.1, 6.2H.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

6.2H.2.4.1	Inference procedure for vertical federated learning when NWDAF or Trusted AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when NWDAF or Trusted AF is acting as VFL server

The inference procedure when trusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. If triggered by internal service logic of the AF acting as VFL server, the steps 0, 1,7 and 8 are skipped.

- The analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g. UE IDs and optionally Analytics Reporting Information=Analytics target period and Analytics Filter) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

If the analytics consumer NF requests Analytics Accuracy Monitoring, Analytics Accuracy Request information as defined in clause 6.1.3 can be contained in Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF can not generate the analytics output, the NWDAF containing AnLF determines the VFL Server for the requested analtyics.

If the VFL server is NWDAF, the NWDAF containing AnLF sends a request to NWDAF VFL server using Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g. UE IDs and optionally Analytics Reporting Information=Analytics target period, Analytics Filter and Analytics Accuracy Request information.

If the VFL server is trusted AF, the NWDAF containing AnLF sends a request to trusted AF VFL server using Naf_Inference_subscribe/request including Analytics ID, Target of Analytics Reporting = e.g. UE IDs and optionally Analytics Reporting Information=Analytics target period, Analytics Filter and Analytics Accuracy Request information.

Editor's note:	Parameters in the triggering of inference and Analytics is FFS.

2.	Based on the information received in the step 0 or 1, VFL server decides to initiate the VFL inference procedure with the VFL clients. VFL Server selects clients(s) using information stored in the VFL training process. The server may select some or no clients, e.g. depending on the accuracy of the VFL model, the contribution to the training result and the current status of the VFL clients.

When no VFL Clients are selected, the VFL server may generates the VFL inference results based only on its local trained ML model associated with the determined VFL correlation ID, skipping the steps 2 - 6 (and 7 if step 1 was also skipped).

NOTE 1:	If the server does not select VFL clients for all features, it can internally store information about the features considered when deriving the analytics results together with a timestamp to identify the analytics results, to enable to subsequently trace the origin of analytics results and to interpret possible feedback on analytics accuracy.

VFL server NWDAF or trusted AF determines and sends a VFL Inference request/subscription to each VFL client including the Target of VFL inference = e.g. UE IDs, VFL correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used and optionally VFL inference filter.

Editor's note:	It is FFS whether additional parameters are needed to send from the VFL server to VFL client, e.g. parameters used in training phase and parameters from Analytics request.

2a.	For each NWDAF VFL client, the VFL Server NWDAF or trusted AF sends an Nnwdaf_VFLInference_Subscribe or Nnwdaf_VFLInference_Request to the VFL client.

2b.	For each trusted AF VFL client, the VFL Server NWDAF sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the VFL client.

2c.	For each untrusted AF VFL client, the VFL Server NWDAF sends an Nnef_VFLInference_Subscribe or Nnef_VFLInference_Request to the NEF serving the AF.

2d.	For each untrusted AF VFL client, the NEF converts any internal identifiers to external identifiers and sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the untrusted AF VFL client.

NOTE 2:	For the trusted AF is acting as VFL server, the VFL client can only be the NWDAF.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

5.	VFL Clients send the intermediate local results to the VFL server.

The intermediate local results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to combine and generate the VFL inference results.

Editor's note:	It is FFS additional parameters are needed to send from the VFL client to VFL server.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

5a.	Each NWDAF VFL client sends an Nnwdaf_VFLInference_Notify or Nnwdaf_VFLInference_Request response to the VFL Server NWDAF or trusted AF.

5b.	Each trusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the VFL Server NWDAF.

5c.	Each untrusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the NEF.

5d.	For each untrusted AF VFL client, the NEF converts any external to internal identifiers and sends an Nnef_VFLInference_Notify or Nnef_VFLInference_Request response to the NWDAF VFL server.

6.	The VFL server may collect its local data and generate the intermediate local inference results. When the VFL Server selected VFL clients to participate in the VFL Inference process, it combines all the intermediate local results to generate the VFL inference results based on the VFL correlation ID. The VFL server takes into account the participation of each VFL client during the ML training process and the importance of the intermediate local results when generates the combined inference output.

The VFL server may compute the global ML model metric (e.g. ML model accuracy) based on all the intermediate local results received from VFL clients and the label.

7.	Depending on request, the NWDAF or trusted AF VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify or Naf_Inference_Response/Notify to the consumer (i.e. NWDAF containing AnLF) including the VFL inference results, optionally, ML model metric (e.g. ML model accuracy).

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

The NWDAF containing AnLF may provide Analytics Accuracy Information based on ML model metric (e.g. ML model accuracy) if the consumer NF requests Analytics Accuracy Monitoring in step 0.

* * *Next Change * * *

6.2H.2.4.2	Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

The inference procedure when untrusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. If triggered by internal service logic of the AF acting as VFL server, the steps 0, 1,7 and 8 are skipped.

0.	Same as step 0 in clause 6.2H.2.4.1.

1.	Same as step 1 in clause 6.2H.2.4.1, to NEF using Nnef_Inference_subscribe/request. NEF forwards the subscription request to AF using Naf_Inference_subscribe/request.

Editor's note:	When the AnLF determine the VFL server AF is FFS. For example, during the training phase.

2.	Same as step 2 in clause 6.2H.2.4.1, to NEF using Nnef_VFLInference_subscribe/request. An untrusted AF includes the external NWDAF ID and sends the request to the NEF.

NEF converts any received external identifiers to internal identifiers and forwards the subscription request to NWDAF using Nnwdaf_VFLInference_subscribe/request.

Editor's note:	It is FFS additional parameters are needed to send from the VFL server to VFL client.

3.	Same as step 3 in clause 6.2H.2.4.1.

4.	Same as step 4 in clause 6.2H.2.4.1.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

5.	Same as step 5 in clause 6.2H.2.4.1, to NEF using Nnwdaf_VFLInference_Notify/Request Response.

NEF converts any received internal identifiers to external identifiers and forwards the subscription notify to the untrusted AF using Nnef_VFLInference_Notify/Request Response.

Editor's note:	It is FFS additional parameters are needed to send from the VFL client to VFL server.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

6.	Same as step 6 in clause 6.2H.2.4.1.

7.	Same as step 7 in clause 6.2H.2.4.1, to NEF using Naf_Inference_Notify/Request Response.

NEF converts any received internal identifiers to external identifiers and forwards the subscription Notify to NWDAF using Nnef_Inference_Notify/Request Response.

8.	Same as step 8 in clause 6.2H.2.4.1.

* * *End of Changes * * *