3GPP TSG-WG SA2 Meeting #167	S2-2502033

# 17 - 21 February, 2025, Athens, Greece							   	 		   (revision of S2-2500677)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1362 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VFL Accuracy Monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | MTLF-based ML Model Accuracy Monitoring defined in clause 6.2E.2 is a functionality from NWDAF to ML model retraining base on accuracy determination and the introduction of VFL Training requires changes in the this functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Extensions of the procedure for MTLF-based ML Model Accuracy Monitoring in clause 6.2E.2 to enable the MTLF-base VFL training accuracy determination. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No proper support for MTLF-based accuracy monitoring when VFL is is executed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.X (new), 6.2E.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change (ALL NEW TEXT)* * * *

6.2H.X	Accuracy Monitoring for Vertical Federated Learning

The Figure 6.2E.2-1 illustrated in clause 6.2E.2 depicts the procedure for MTLF-based ML Model Accuracy Monitoring used for triggering changes in a trained ML model. The MTLF-based VFL Training Accuracy monitoring procedure with NWDAF acting as VFL server is based on the same procedure described in clause 6.2E.2.

When the MTLF-based VFL training accuracy monitoring is executed by a NWDAF containing MTLF that is also the VFL Server for a given VFL correlation ID, such NWDAF containing MTLF is capable to determine the VFL training accuracy information, which comprises the calculated value of the accuracy for a given VFL training execution identified by the VFL correlation ID, and the list of VFL Clients that participated in such VFL training. A calculated VFL training accuracy information is associated to the VFL correlation ID as well as to all VFL Models that were jointly trained.

The same steps described in clause 6.2E.2 apply for the MTLF-based VFL Training Accuracy Monitoring and the extensions to each step are described as follows.

NOTE 1:	Steps 2 – 5, and 7 in the procedure illustrated in Figure 6.2E.2-1 are not applicable because in this release of the specification, NWDAF containing AnLF and NWDAF containing MTLF are the same.

-	Step 8 in Figure 6.2E.2-1: The NWDAF containing MTLF acting as VFL Server, based on the request(s) from one or more NWDAF containing AnLF or its local policy, determines whether to perform VFL Training Accuracy Monitoring and re-training of ML Model by collecting new data from various data sources. In addition to collect new data for its own local training, the NWDAF containing MTLF acting as VFL Server determines a VFL DataSetTag associated with the VFL Correlation ID to be distributed to all VFL Clients that participated in the VFL Training in order to trigger the new data collection for verification of VFL training accuracy information. All new collected data at the VFL Server and VFL Clients is then associated with the same VFL DataSetTag.

Step 8g is introduced in the procedure to allow the NWDAF Containing MTLF acting as VFL server to invoke the Nnwdaf_ VFLInference_Subscribe service from the VFL Clients corresponding to the VFL Correlation ID associated with the analytics ID, and includes in the request the VFL Correlation ID, an indication for VFL accuracy monitoring, and the determined VFL DataSetTag. Each VFL Client, based on the received request will trigger the new data collection locally associating the collected data to the VFL Correlation ID, local ML Model unique identifier, and the received VFL DataSetTag. The VFL Clients will generate the intermediary results based on the newly collected data and notify the VFL Server.

- Step 9g (new): 	The NWDAF containing MTLF receives all the intermediary results calculated by the VFL Clients as requested in step 8g.

- Step 10 in Figure 6.2E.2-1: Based on the collected data from steps 9a-9f and the received notifications in 9g, the NWDAF computes its local inference result based on its new collected data and generates the VFL inference result combining with the intermediary results received from the VFL Clients. The NWDAF containing MTLF determines the VFL training accuracy information based on the final VFL inference result and the ground truth for the VFL correlation ID.

- Step 12 in Figure 6.2E.2-1: Based on the determined accuracy, the NWDAF containing MTLF (also the VFL Server) may decide to trigger the re-trainning for the VFL Correlation ID, using the procedure defined in 6.2H.2.3.1. The NWDAF acting as VFL Server provides to the VFL Clients an indication of retraining and the VFL DataSetTag, to allow all VFL Participants to use the new collected data in the same re-training process.

- Step 13 in Figure 6.2E.2-1: Not applicable.

# * * * * Next Change* * * *

6.2E.3.1	General

The procedures described in this clause enable the following functionality:

-	An NWDAF containing AnLF may register with an NWDAF containing MTLF when it starts using an ML Model and monitoring the accuracy of analytics generated by that ML Model for a given Analytics ID. It is assumed that the NWDAF containing AnLF obtained the ML Model in a previous interaction with the NWDAF containing MTLF, e.g. using the Nnwdaf_MLModelInfo_Request or Nnwdaf_MLModelProvision_Subscribe services. This registration enables the NWDAF containing MTLF to become aware of NWDAF containing AnLF that are using a given ML Model for certain Analytics ID and that the NWDAF containing AnLF supports the capability of monitoring the accuracy of the corresponding analytics.

-	An NWDAF containing MTLF may subscribe to an NWDAF containing AnLF where an existing Nnwdaf_MLModelMonitor service is established for receiving notifications of the accuracy of analytics generated by a given ML Model for a certain Analytics ID. NWDAF containing AnLF can generate the accuracy information in many ways: e.g. comparing predictions of ML Model and its corresponding ground truth data, comparing changes in internal configuration for the analytics ID generation, previous existent records of Analytics Accuracy Information, etc.

NOTE:	In this release of the specification, if VFL is supported by the NWDAF containing MTLF requiring AnLF-Assisted MTLF ML Model Accuracy monitoring, and this NWDAF is the VFL Server, then the NWDAF containing MTLF is the same as the NWDAF containing AnLF. Therefore, the procedures in Clause 6.2E.3.2 and 6.2E.3.3 are implementation dependent.

# * * * * End of changes * * * *