3GPP TSG-WG SA2 Meeting #167	S2-2502080

Athens, GR, 17th February – 21th February, 2025	(revision of S2-250xxxx)

Source:	NTT DOCOMO, Nokia, Deutsche Telekom, Philips International B.V., Ericsson

Title:	Discussion on globally unique Ambient IoT identifier

Document for:	Discussion

Agenda Item:	19.14.2

Work Item / Release:	Architecture support of Ambient power-enabled Internet of Things (AmbientIoT) / Rel-19

Abstract: This paper discusses the need for the permanent Ambient IoT identifier to be globally unique within the 5G system.

# 1. Introduction

During SA2#166 it was concluded in TR 23.700-13 there is enough information in the Ambient IoT permanent identifier to make it globally unique.

During the study phase it has been concluded Ambient IoT devices uses one identifier, an Ambient IoT permanent identifier, whereas UEs have SUPI, PEI, GPSI and temporary identifiers.

This discussion paper aims at describing complications that may arise if the Ambient IoT permanent identifier is not always globally unique.

# 2. Discussion

## 2.1	Ambient IoT service operation

Figure 2.1-1 Ambient IoT service operation

### 2.1.1	Complications at paging

Figure 2.1-1 illustrates an example where an operator serves two different application functions. Both application functions trigger an inventory request to the operator with a target ID X of their respective Ambient IoT device. Application Function #1 wants to reach Ambient IoT device #1 and Application Function #2 wants to reach Ambient IoT device #2.

The inventory requests will lead to Ambient IoT paging. Assuming the paging is based on permanent identifiers, which in this case are equal a result of the paging may be that both Ambient IoT device #1 and Ambient IoT device #2 responds. Now the AIOT RAN may consider the responses duplicates leading to unclear behaviour.

In another example, only Ambient IoT device #1 responds, in that case, the AIOT RAN may not be able to determine if the response belongs to paging triggered by Application Function #1 or Application Function #2.

Observation 1: Not guaranteeing globally unique identifiers may lead to unreliable service. It may become complicated for an operator to provide Ambient IoT services to multiple Application Functions and in scenarios where two target areas for Ambient IoT services are in close vicinity to each other. One such example, could be two different stores in a shopping mall.

### 2.1.2 Complications at the AIOTF

Dynamic context at the AIOTF may not be applied accurately if the Ambient IoT permanent identifier is not globally unique.

The AIOTF may receive the result from AIOT RAN and forward it to the wrong application function. For example, AIOT RAN may forward the Inventory response of Ambient IoT device #1 thinking it is Ambient IoT device #2. AIOT forwards the response to Application Function #2.

Observation 2: Not guaranteeing globally unique identifiers may lead to false positives, questioning reliability of inventory services.

### 2.1.3 Complications with charging

If the Ambient IoT device permanent identifier is not globally unique, it is unclear how charging per device can be supported. Based on TS 22.369 clause 5.2.5

“The 5G system shall be able to collect charging information in a suitable way for using Ambient IoT services on per Ambient IoT device basis or a group of Ambient IoT devices (e.g., total number of communications per charging period).”

If the Ambient IoT permanent device is not unique, charging per device and per group is unreliable as duplicates may cause charging inaccurate amounts to the wrong customer.

Observation 3: Not guaranteeing globally unique IDs may cause severe issues for an operator to charge accurately for Ambient IoT services.

# 3. Conclusion and proposal(s)

Based on the observations in clause 2, it is proposed to mandate that the Ambient IoT device permanent identifier is always globally unique in the 5GS as proposed in S2-2502082. Proceeding differently may cause delays in the Rel-19 work as it would require Liaison exchange with RAN groups to confirm paging can be done accurately and with SA5 to clarify there are no implications on charging.