3GPP TSG SA WG2 Meeting #167	S2-2502109

17 – 21 JFebruary, 2025, Athens, Greece   	 (was S2-2500409)

Source:	Ericsson

Title:	Solving KI#4 with N6 metadata using KI#2 mechanisms

Document for:	Approval

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2/ Rel-19

Abstract of the contribution: This contribution discusses whether SA2 should specify a solution for KI#4 using KI#2 mechanisms.

# Background

The purpose of this paper is to address the Editor’s Note in TS 23.501 clause 5.37.9.1:

“Editor’s Note: Other parameters related to KI#4 are FFS.

TS 23.501 Clause 5.37.9.1 describes the general aspects for how to support transferring media related information over N6. The EN leaves FFS whether KI#4 related parameters should be sent as media related information.

KI#4 has studied the Traffic detection and QoS flow mapping for multiplexed data flows. An exception request has been agreed for XRM Phase 2. Addressing this EN is covered by the exception request as follows:

“Support differentiated QoS handling of ciphered multiplexed XRM traffic other than SRTP.  Scope is DL only. It is contentious whether this feature must be done in Rel-19”.

# Discussion

Discussion follows in relation to the EN in TS 23.501 clause 5.37.9.1:

“Editor’s Note: Other parameters related to KI#4 are FFS.

This EN leaves FFS whether any parameters related to KI#4 should be added to the media related information that 5GC can receive using the mechanisms defined for KI#2.

Having KI#4 related parameters in XRM metadata over N6 was not in the conclusions of the study phase. Furthermore ,a solution based on having KI#4 related parameters in XRM Media Related Information (MRI) received over N6 (from now on  the MRI-based solution) is a partial solution which can only be used for the DL. These two aspects have not been questioned in the WG discussion and they are not going to be discussed further.

The adoption of QUIC as transport protocol is a very clear internet trend. When QUIC is used (e.g. when RTP-over-QUIC (RoQ) is used), also the RTP headers are ciphered (i.e. encrypted) and not visible. The MRI-based solution intends to provide differentiated QoS handling to ciphered XRM media flows multiplexed in a single QUIC connection.

However, this solution has an important issue. QUIC supports that multiple streams can operate with a degree of independence. It allows a stream to make progress even when other streams are blocked. However, this property alone is not sufficient to guarantee stable performance when assigning different network QoS classes to each stream. QUIC congestion control operates at the connection level rather than at the individual stream level. The QUIC Congestion control relies on packet-level metrics such as round-trip times (RTT), packet loss, and acknowledgment rates to determine optimal sending rates. Prioritizing certain packets over others, by steering media flows (same connection) to different QoS flows, can cause significant RTT variations and lead to suboptimal rate calculations. Unstable RTT measurements can result in suboptimal packet recovery performance because the associated timers are based on the estimated RTT. Moreover, Lower-priority packets may also face higher drop probabilities, and any lost packet will affect congestion control for the entire connection. This is shown in Figure 1.

Figure 1. Differentiated QoS Handling in 5GS. All media streams are transmitted in a QUIC connection (and single path)

Observation 1: if 5GS steers the data of one QUIC connection (single path) to different QoS flows, the end-point congestion control algorithm is going to misinterpret the network performance and make suboptimal transmission decisions.

To overcome this problem, the endpoint would need to implement per media flow congestion control contexts, control and loss detection, maintaining separate accounting for the media flows. Such an approach would require significant, yet currently undefined, changes to QUIC endpoints and would essentially turn an implementation into a half-baked MP-QUIC (see further below) in disguise. This approach is not according to IETF QUIC specifications.

Observation 2: the MRI-based solution requires that the QUIC transport layer congestion control implementation of end points considers per media flow congestion control contexts, which is not according to IETF QUIC specifications and undefined.

In addition, IETF has specified extensions for multipath QUIC (MP-QUIC) which could be used to send the multiple media flows multiplexed in the same end-to-end transport layer connection (i.e. one QUIC connection and single security context) on different paths using different 5-tuples for the different media (for example, these paths could share the server IP addresses but have different port numbers).

The information flow is shown in the Figure 2. Any Load balancers in the path can be configured to guarantee all the packet flows belonging to the same QUIC connection are steered to same end point.

Figure 2: MP-QUIC for stream multiplexing in the same QUIC transport connection

Using multipath QUIC, the several multiplexed encrypted media flows can be differentiated for QoS handling in uplink and in downlink using the 5-tuple. The congestion control contexts are independent for each path. And as a result, delays, loss and reordering on one QoS flow do not impact media flows on other QoS Flows. This is shown in Figure 3

Figure 3. Differentiated QoS Handling in 5GS. Different media streams are transmitted in different MP-QUIC connection paths

Observation 3: media streams with different QoS needs can receive different QoS treatment by 5GS and have independent congestion control if they are sent over different paths in same multipath QUIC connection.

At the endpoint, the QUIC transport layer with Multipath QUIC support needs application guidance to associate the different media flows to paths. One alternative is to base this association in some form of media flow relative priority. On the other hand, the application layer, and then AF, need to know the applicable 5-tuples to provide the QoS requirements (for example, the application layer could receive information of how media flows or priorities are associated to ports).

Observation 4: A solution based on MP-QUIC is simpler and it is foreseen to have less impacts on the endpoints than a MRI-based solution which requires reimplementing capabilities that are already standardised in multipath QUIC.

There is yet one more solution. The above described MP-QUIC alternative could be enhanced using MRI. In this solution, Multipath QUIC is used to send the multiple media flows multiplexed in the same end-to-end transport layer connection (i.e. one QUIC connection and single security context) on different paths. These paths can have same or different 5-tuples. But now, in addition, the application can provide Multiplexed Media Identification Information for encrypted traffic over N6 and the AF provides individual QoS requirements for each individual media flow including the Multiplexed Media Identification Information for encrypted traffic associated with the individual QoS requirements within the flow description.

By using MRI in addition to MPQUIC, MPQUIC provides independent congestion control for the paths, and the Application layer (and then AF) do not need to be aware of how the media flows are associated to 5-tuples. When the different paths have different 5-tuples, differentiated QoS handling is possible in downlink but also in uplink if combined with reflective QoS. If the paths have the same 5-tuple, differentiated QoS handling is only possible in the downlink.

Observation 5: A new solution has been identified which has less impacts on endpoints where MRI is used on top of MPQUIC. There may be other solutions.

Based on the observations above, the following Proposal is included.

# Proposal

Adding parameters related to KI#4 in the N6 metadata was not agreed in the study phase, and as shown above, the solution for differentiated QoS handling of ciphered multiplexed XRM traffic is not straight forward and requires proper analysis.

The following alternatives have been identified:

A.  Remove the EN. This solution was not well evaluated in the study phase has important issues. No further work on KI#4 related parameters for the Media related Information in Release’19. KI#4 for E2E encrypted traffic (e.g. RoQ) can be a candidate for a future release.

B.  Remove the EN adding a description of how applications can use MP-QUIC extensions to multiplex the media flows (same transport connection) using different paths (different 5-tuples), that the network can use for differentiated mapping to QoS flows. The MPQUIC based solution has no impact on 5GS.

C.  Add a description of how applications can use MP-QUIC extensions to multiplex the media flows and specify the solution that uses MRI on top of MPQUIC.

We can either accept A or B. Alternative C would require further work and it is not possible to pursue in Rel-19.

CRs are submitted for alternatives A and B, where B enhances the specifications with an informative annex that describes how applications can use the MPQUIC based solution.