3GPP TSG SA WG2 Meeting #167	S2-2502110

17 – 21 February, 2025, Athens, Greece   	 (was S2-25xxxxx)

Source:	Ericsson

Title:	Discussion on Averaging Window in AQP

Document for:	Approval

Agenda Item:	9.12.2

Work Item / Release:	XRM / Rel-18

Abstract of the contribution: This contribution discusses the aspects of inclusion of the Averaging Window in AQP.

# Discussion

Notification Control mechanism was introduced for the purpose to:

“…indicates to the NG-RAN that notifications of "GFBR can no longer (or can again) be guaranteed" are requested when the NG-RAN determines that the GFBR, the PDB or the PER of the QoS profile cannot be fulfilled (or can be fulfilled again) for a QoS Flow (during the lifetime of the QoS Flow) and that the QoS Flow should be kept while the NG-RAN is not fulfilling the requested QoS profile”

Observation: Notification Control is based on observation of the fulfilment of the GFBR, the PDB or the PER for a given QoS Flow. (In Rel-19 PDU Set variant of these parameters has been added.)

The observation of the QoS fulfillment is conducted over the Averaging Window (AW) provided to NG-RAN in the QoS profile (See TS 23.501):

“The NG-RAN determination whether the GFBR, the PDB and the PER can be fulfilled or not, is done under consideration of the QoS parameters Averaging Window and MDBV (for a GBR QoS Flow using the Delay-critical resource type), which are either provided explicitly as part of the QoS profile or implicitly via the standardized 5QI QoS characteristics defined in clause 5.7.4.”

With addition of the support for Alternative QoS Profile (AQP) concept, the NG-RAN can indicate in the Notification Control a reference of the AQP that (is close to) describes currently fulfilled QoS while the baseline of NG-RAN monitoring the QoS fulfillment remains unchanged.

Observation: Introduction of AQP does not modify the baseline of Notification control, i.e., monitoring of QoS fulfillment over AW received in NG-RAN in the QoS profile.

Adding the AW to the AQP introduces however a fundamental change to Notification Control mechanism as it introduces a new dimension to the observation process of the QoS fulfillment in NG-RAN. While in baseline the QoS fulfillment is observed over a static and single AW included in the QoS profile, adding AW in the AQP would require NG-RAN to conduct parallel/simultaneous observations as a function of different AWs which means new and demanding NG-RAN requirements.

Observation: Introduction of AW in AQP introduces new NG-RAN requirement to perform simultaneous observations using different AWs.

It shall be noted that such requirement was not part of the Rel18 XRM study conclusions and its impacts never studied.

# Conclusions

Based on the discussion we propose to remove the Averaging Window from the Alternative QoS Profile concept.  CRs to TS 23.501 in Rel-18 and mirror CR to Rel-19 are provided in S2-2502111(Rel-18, ‘F’, CR6149R0) and S2-2502112 (Rel-19, ‘A’, CR6150R0). Furthermore, we propose to un-approve CRs to TS 23.503 in 23.503 CR1280R4 (Rel-18, 'F'): Averaging Window in Alternative QoS and 23.503 CR1440R2 (Rel-19, 'A'): Averaging Window in Alternative QoS Parameter Set.