3GPP SA WG2# 167	S2-2502121

Athens, Greece, February 17 – 21, 2025	(revision of xx-yyxxxx)

{Guidance text shown in curly brackets, in italics. All guidance text is to be deleted before WID submission.

Note that this form is to be used for all types of Work Item, covering both normative work and/or studies. A Work Item covering only studies is also known as "Study Item", as stated in TR 21.900.}

# Source:	Ericsson (Moderator)

# Title:	New SID on Study on Industrial Networks enhancements

{"Revised" to be used only for WID already approved at plenary. For Revised WIDs, two versions have to be provided in a zip file: a clean one and one with revision marks showing the differences with the previously plenary-approved WID.}

# Document for:	Agreement

# Agenda Item:	30.8

Abstract: This is SI-1 as per moderator report in S2-2501576, based on initial WTs according to SA plenary outcome. The document needs to be updated based on any agreements reached during the meeting.

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Industrial Networks enhancements

{Free text. It has to be the same as in the "Title:" section above. Studies have to start by "Study on"}

Acronym:	FS_IndNwk_enh

{Propose an acronym. Final acronym to be confirmed at the plenary. The sign "-" is a level separator between (Feature)-(Building Block)-(Work Task). The sign "_" can be freely used. Studies have to start by "FS_". Each acronym level has to be simple and short, 7 characters max recommended}

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20

{ Replace XX by the intended Release, e.g. Rel-19.  Note that this field indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan}

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

{Tick one or more box(es). The full structure of all existing Work Items is shown in the 3GPP Work Plan in https://ftp.3gpp.org/Information/WORK_PLAN}

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

{"Parent" Work Item refers to the related, earlier-Stage, Work Item, e.g. the related Stage 1 Work Item shall be indicated here when a Stage 2 normative Work Item or Study Item is presented. "Parent" Work Item can also refer to the related preceding Study Item e.g. the related Study Item and the earlier-stage Work Item shall be indicated here when a normative-work Work Items is started. List here all parent Work Items of which requirements are either fully or partially covered by the proposed Item. }

{This section is mandatory to be filled out by the rapporteur. This section is to be filled with care: it indicates to the companies monitoring the parent Work Item that it will be addressed in this study/work item.}

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 820017 | Stage 2 of Vertical_LAN | Vertical_LAN |
| 910059 | Support of Enhanced Industrial IIoT | IIoT |
| 970011 | Stage 2 of Extensions to the TSC Framework to support DetNet | DetNet |

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency}

# 3	Justification

Justification for enhancing Hold and Forward buffering mechanism to support de-jittering function:

Latency characteristics of the 5G bridge deviate significantly from those of a wired bridge.

Wired bridge jitter in the range of 100s of nanoseconds.

5G bridge jitter in the range of milliseconds.

Leading to complex TSN scheduling and not scalable with increasing number of TSN streams.

With a large jitter (compared to wired bridges) there will be an uncertain queueing and transmission order

within 5GS.

Queueing issues lead to some packets missing the opportunity for transmission in the current cycle, and are

delayed till next cycle –breaking the schedule.

Additionally, for delay critical applications and in general for TSC and DetNet (when IEEE 802.1Qbv is not

required), the large 5G-introduced jitter will prevent the service to guarantee on-time delivery of periodic

traffic as per requirements of the application.

Additional comments on TSN de-jitter: some companies don’t see the need, whereas others have asked to expand the scope to include support of CQF in Annex T in IEEE 802.1Q can be used to improve the issue of large jitter in 5GS TSN bridge.

Additional comments on TSC/DetNet de-jitter: includes only focus on TSC and separate TSC and DetNet as two subtasks.

Justification in order to enable support loop detection and elimination in 5G LAN and enable support of 5GS native FRER. The WTs have been expanded to look at enhancement not restricted to 5G LAN and FRER only, but also to include IP option.

Loop in 5G-LAN: 5G-LAN defines Layer 2 Ethernet networking for 5GS, where Ethernet loop may occur in

the 5G-LAN network, which may result in broadcast storm and further network breakdown. For example,

haphazard adjustment of the device side networking may cause loop, since many end switches do not have the loop detection capability.

FRER can improve the connection availability and reliability, in the case of poor air interface quality or

temporary fault of a single UE module for multiple UE modules case, and reduce the service downtime caused

by transmission timeout. However, native support of FRER by 5GS is not supported, and the 5GS has to rely

on TSN and application level FRER which is hard to deploy.

Additional feedback is that to change the second case FRER) to the following: FRER for Ethernet and PREOF for IP can improve the connection availability and reliability, in the case

of poor air interface quality or temporary fault of a single PDU Session, and reduce the service downtime

caused by transmission timeout. However, native support of TSN FRER and DetNet PREOF by 5GS is

not currently available, and the 5GS has to rely on externally deployed TSN/DetNet and application level

FRER/PREOF which is hard to deploy. Scenarios both with single UE per device or multiple UEs per

device are possible. There are also request to remove multiple devices case and focus on single UE and multiple PDU sessions. Also questions raised what to solve and PDCP Layer solution already exists between RAN/UE. As well that previous attempts to address this has not been successful so may not be best use of time.

Additional feedback on Loop prevention: Loops should be prevented in advance, so we need a proactive solution, not a reactive one and as such adjust justification and WT accordingly. And to expand the scope significantly to include as per Siemens-WT#5 with its subtasks: Support of Multiple Spanning Tree Protocol (MSTP) for 5G System operating as logical bridge in integrated IEEE/3GPP networks.

# 4	Objective

Draft objectives taking into account different views (not consensus):

WT#1: De-jitter for TSN

WT# 1.1 Study whether and how to extend the Hold and Forward buffering mechanism with a de-jittering functionality for TSN.

WT# 1.2 Study whether the support of CQF in Annex T in IEEE 802.1Q can be used to improve the issue of large jitter in 5GS TSN bridge.

WT#2: De-jitter for TSC/DetNet

WT#2.1: Study how to extend the Hold and Forward buffering mechanism with a de-jittering functionality for TSC.

WT#2.2: Study how to extend the Hold and Forward buffering mechanism with a de-jittering functionality for DetNet.

WT#3: Loop Prevention

Alt 1: Study how to provide loop prevention functionality (i.e., MSTP/RSTP) when the 5G system is integrated in industrial Ethernet network as a bridge.

Alt 2:

- WT-3.1: How to define support of the Multiple Spanning Tree Protocol in the UPF for 5G VN scenarios to ensure loop prevention and enhancements for its activation/deactivation.

- WT-3.2: How to define support of Multiple Spanning Tree Protocol or Rapid Spanning Tree Protocol in UPF in Ethernet PDU Sessions which are not related to 5G VN.

Alt 3:

Support of Multiple Spanning Tree Protocol (MSTP) for 5G System operating as logical bridge in integrated IEEE/3GPP networks, including:

- Evaluate the topologies that create a loop

- Architectural enhancements of 5GS that supports MSTP operations (e.g. BPDU forwarding)

- How 5GS is configured for MSTP communication

MSTP (RSTP) works on physical topology that is below the active logical topology (5G-LAN), the “Support of MSTP for 5G VN group(s)” is based on aspects listed above.

NOTE: The solution should be compatible with MSTP as required by IEC/IEEE 60802 (RSTP + traffic engineering).

WT#4: Packet Replication & Elimination

Alt 1: Study how packet replication and elimination can be done within 5G system in order to establish redundant connection between UE and UPF; considering two PDU sessions for a device.

Alt 2: 3GPP entities natively support duplication of the packet data or elimination of redundant transmissions, re-using functionality of IEEE FRER or IETF DetNet where applicable. The replication procedure is controlled by the 3GPP system, and performed for two different PDU Sessions which may originate from a single device.

Contentious Issue: Whether it is for single UE, or this should be open to redundancy from Multiple UEs.

Discuss the content of the WTs and justification and further revise the scope and then determine the TU budget (initial thought, will need to be adjusted to final WT content): WT1 and 2: 2 TU study, WT#3: depending on the scope, 2 to 4 TUs, WT#4, depending on the scope, 2 – 3 TUs.

# 5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| {Possible values: "TS" or "Internal TR" or "External TR". See Note 1} | {e.g. "22.XXX" or actual number if known} | {Title of the specification (as per TR 21.801 §6.1.1), to be aligned as much as possible with the WI/SI title} | {e.g. "TSG#87"} | {e.g. "TSG#89"} | {<FamilyName>, <GivenName>, <Company>, <email address>. See Note 2} |

{Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| {e.g. "22.281"} | {Possible values: - either free text (e.g. “CS aspects to be removed") - or “Specification to be withdrawn”} | {e.g. "TSG#89"} | {Free text, e.g. "This TS covers Stage 2" or "This TS covers Stage 3" or "This TS covers both stages 2 and 3"} |

# 6	Work item Rapporteur(s)

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>}

{Optional: <FamilyName>, <GivenName>, <Company>, <email address>: Secondary task(s)}

{The first listed Rapporteur is the work item primary Rapporteur. The role of a Rapporteur is further described in www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec. By default, the primary Rapporteur shall ensure the production of the post-completion summary. 
Secondary Rapporteur(s) are possible for specific secondary task(s), such as: "Write the post-completion summary"; "In charge of a specific aspect of the work item (specify which)"; "Rapporteur for a secondary responsible WG (specify which)"}

# 7	Work item leadership

SA WG2

# 8	Aspects that involve other WGs

{This information is provided as best effort assumption, at the time of submission of the WID to TSG approval. It can be later changed without a need to revise the WID.

The “aspects” can be provided by topic (e.g. “security”, “multimedia”) and/or by specifying the WG(s) e.g.: "SA2, SA3, SA5, SA6. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet"}

For a Stage 2 WID requiring Stage 3 to be done by another group: on a best-effort basis, indicate which potential WG is expected to specify the Stage 3: {possible values: "Not applicable", " unknown", "CT WGs", etc}

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name