3GPP TSG-SA2 Meeting #167	S2-2502124

Athens, Greece, 17th – 21st Feb 2025	(revision of S2-250xxxx)

Source:	Ofinno

Title:	Proposal on Rel-20 Energy WTs

Document for:	Discussion

Agenda Item:	30.8

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This discussion paper aims to add some clarity to a few WTs of a new study on Energy Efficiency and Energy Saving Phase 2 and to make related proposals.

# 1. Introduction

The input of this contribution relates to the following subset of WTs being presented as part of SA2-led NWM-based moderated discussions:

WT#1: Whether and how to expose carbon emissions data (e.g., expanding KPIs to include traffic-based energy efficiency and carbon metrics).

WT#3: Whether and how to support UE-assisted network energy-saving.

WT#4: RAN reporting of the per-UE radio resource consumption information to improve accuracy of energy consumption.

The reason for the focus on these WTs is motivated by increasing energy related regulations and concerns being made worldwide. However, a number of concerns were raised on the current formulations of these WTs and respective scopes. Hence, rewording is required in order to easily turn these WTs into respective key issues (KIs).

This contribution aims to clarify key requirements, provide justifications as to why this subset of WTs need to be given due consideration/priority and suggest an improved formulation of each of these WTs.

# 2. Discussion

The following paragraphs provide justifications as to why the identified WTs are necessary, and provide new formulations as a way to address concerns raised in a related NWM discussion:

WT#1: Whether and how to expose carbon emissions data (e.g., expanding KPIs to include traffic-based energy efficiency and carbon metrics)

Justification: The GSMA has estimated that the telco industry consumes staggering 2% to 3% of worldwide energy consumption – hence, there is a need for a mobile network operator (MNO) to be sensitive about this, to be open and transparent in terms of how much renewal energy is used in comparison with non-renewable energy. In order, for example, for authorised consumers (e.g., government bodies or environmental agencies) to assess what type of measures an MNO takes to cut down its carbon footprint, there is a need to expose its carbon emissions data (e.g., per service area and/or services it provides) on an hourly, daily, monthly or yearly basis. The granularity of exposed data can be discussed at a normative phase.

Current concern: the current wording is not clear. Meanings of carbon emissions and energy efficiency can be different. Also, there is a need to check whether SA5 can collect and provide carbon emission data. Also, accurately determining carbon emission data per UE may at least rely on WT4 and hence, it may not be possible now.

Re-formulation: Whether and how to expose carbon emissions data (e.g., expanding KPIs to include traffic-based energy efficiency and carbon metrics) per service region (e.g., per cell, per TA,…) and or services (e.g., slices) it provides.

WT#3: Whether and how to support UE-assisted network energy-saving.

Justification: there are approximately 4.88 billion smartphone users worldwide and saving energy in each UE is also equally important. Although how fast a UE loses/drains out its (battery) energy depends on several factors such as a number of applications it runs, display technology being used etc., maintaining active connection to a network may contribute to a significant portion of its energy consumption. However, all the other factors are closely related to a network connectivity – for example, if there is no data connection, the probability for a screen use may be low. Hence, a network has to make measures to help a UE save energy especially when a UE drains out its energy at a fast rate.

Current concern: the current wording is not clear. There is an ambiguity in terms of how battery status of a UE can help a network save energy. Instead, what needs to be explored is how a network can help a UE save its own as well as network energy and/or prevent a UE from reaching its critical energy state (e.g., battery is fully drained out such that a UE cannot make even an emergency 911 call).

Formulation: Whether and how to support UE-assisted network system energy-saving.

WT#4: RAN reporting of the per-UE radio resource consumption information to improve accuracy of energy consumption.

Justification: In principle, it is beneficial for a mobile network operator (MNO) to determine how much energy the MNO has to consume in its network to serve each device. With this data, it is possible for an MNO to decide how energy efficiently it can serve its UEs.

Current concern: it is RAN dependent.

Re-formulation: current wording is fine.

# 3. Conclusion and proposal(s)

Based on the above consideration, the following proposal is made.

Proposal 1: Rel-20 FS_EnergySys_Ph2 should consider re-formulating the following WTs as follows.

WT#1: Whether and how to expose carbon emissions data (e.g., expanding KPIs to include traffic-based energy efficiency and carbon metrics) per service region (e.g., per cell, per TA,…) and or services (e.g., slices) it provides.

WT#3: Whether and how to support UE-assisted network system energy-saving.