3GPP TSG-SA WG2#167	S2-2502145

Athens, 17-21 February 2025

Source:	Vodafone

Title:	Paging and UE power saving features related to PSAP call back

Document for:	Discussion/Approval

Agenda Item:	19.50

Work Item / Release:	TEI19/Rel-19

Abstract of the contribution: Analyses the interaction of emergency call back and UE power saving features.

# Background

In the January 2025 SA2 emeeting (#166-adhoc-e) several documents were submitted relating to the incoming LS from RAN 2 in R2-2411041/S2-2500044=S2-2501367, “LS on emergency services and eDRX”.

Conclusions on the discussion was postponed to allow for F2F dialogue at SA2#167 in Athens.

# Introduction

S2-2500133 from Qualcomm provided some useful information on Emergency call back and “delayed” paging, much (but not 100%) of which the author supports.

The author believes there are some additional aspects that ought to be discussed and considered.

# Discussion

### 3.1	eDRX adds significant delay while PEI and WUS add very small delay

PEI and WUS are intended to wake the mobile up and get it to respond to CN (or RAN) paging before the CN (or RAN) retransmits the page. Typically, the CN and RAN re-transmit paging a short while after the DRX interval has expired, e.g. after 2 seconds when using a DRX of 1.28 seconds. As the MT call origination is not synchronised to the UE’s paging occasion, the use of PEI or WUS for a PSAP callback does not add any significant delay to call setup.

In contrast, eDRX intervals can be large, e.g. out to 40 minutes. Hence ensuring that eDRX does not impact a PSAP call back is important.

Observation 1: PEI and WUS add only small delays to RRC Connection setup.

Observation 2: eDRX can add large delays to RRC Connection setup

Proposal 1: the discussion on call back is handled separately for eDRX from that for PEI/WUS.

### 3.2	Adopt “failsafe” approach to WUS and PSAP call back

Interoperability testing for complex cases is difficult to arrange and to ensure sufficient breadth of testing. Given the low latency addition from the use of WUS, it is recommended that no special WUS handling is used when emergency connections are, or have been, in use.

Proposal 2: no modifications for WUS are specified for emergency PDN connections/bearers.

### 3.3	PSAP call back and eDRX

The discussion during January’s emeeting and the extracts from TS 23.401 and TS 23.501 in Annex A of this document make it clear that PSAP call back can happen after the release of the Emergency PDN connection and when eDRX is active.

While eDRX can involve substantial delays, in order for normal mobile terminating IMS calls to succeed, the eDRX interval should not be too large. This is to cope with normal telephony timer ranges of the order of 30 seconds. Hence, when a (non-emergency) IMS PDN connection has been established, the MME/AMF ought to restrict the eDRX value to probably 10.24 seconds, and the RAN does not configure any larger timer for RRC Inactive (extract from TS 23.501 v19.2.1 section 5.31.7.2.1 below).

For RAT types that support extended DRX for CM-CONNECTED with RRC_INACTIVE state, the AMF passes the UE's accepted idle mode eDRX values to NG-RAN. If the UE supports eDRX in RRC_INACTIVE, based on its UE radio capabilities, NG-RAN configures the UE with an eDRX cycle in RRC_INACTIVE as specified in TS 38.300 [27] up to the value for the UE's idle mode eDRX cycle as provided by the AMF in "RRC Inactive Assistance Information" as defined in clause 5.3.3.2.5.

Observation 3: for UEs with a normal IMS PDN connection, the AMF/MME should limit eDRX to a value that enables successful MT call establishment and hence does not impact PSAP call back.

Proposal 3: existing eDRX specifications and eDRX implementations should be OK for PSAP callback.

### 3.4 	Power Save Mode and MICO mode

In order to avoid car batteries going flat when cars are left unattended for very long periods, power saving features (e.g. Power Save Mode/MICO) may be utilized by some vehicles.

As at least PSM/MICO is a UE-initiated feature that is difficult for the network to reject completely, network-side behaviour might not cover all situations.

Looking at TS 23.501, the interactions of MICO and PSAP call back seem to have been addressed, but there seems to be no corresponding text in TS 24.301 for PSM.

TS 23.501 v19.2.1 clause 5.4.1.3, MICO mode:

A UE initiating emergency service shall not indicate MICO preference during Registration procedure. When the MICO mode is already activated in the UE, the UE and AMF shall locally disable MICO mode after PDU Session Establishment procedure for Emergency Services is completed successfully. The UE and the AMF shall not enable MICO mode until the AMF accepts the use of MICO mode in the next registration procedure. To enable an emergency call back, the UE should wait for a UE implementation-specific duration of time before requesting the use of MICO mode after the release of the emergency PDU session.

TS 24.301 v18.7.0 section 5.3.11, Power Save Mode

If the UE is attached for emergency bearer services or has a PDN connection for emergency bearer services, the UE shall not activate PSM.

Proposal 4: update the PSM specification in TSs 23.401 and 24.301 to align with the MICO mode functionality. (i.e. add text to say “To enable an emergency call back, the UE should wait for a UE implementation-specific duration of time before requesting the use of Power Save Mode after the release of the emergency PDU session.”)

Proposal 5: discuss and agree CR 3898 to TS 23.401 in S2-2502147

## Summary

It is proposed that the preceding topics are discussed, the proposals agreed, and the results communicated to the other relevant groups.

Observation 1: PEI and WUS add only small delays to RRC Connection setup.

Observation 2: eDRX can add large delays to RRC Connection setup

Proposal 1: the discussion on call back is handled separately for eDRX from that for PEI/WUS.

Proposal 2: no modifications for WUS are specified for emergency PDN connections/bearers.

Observation 3: for UEs with a normal IMS PDN connection, the AMF/MME should limit eDRX to a value that enables successful MT call establishment and hence does not impact PSAP call back.

Proposal 3: existing eDRX specifications and eDRX implementations should be OK for PSAP callback.

Proposal 4: update the PSM specification in TSs 23.401 and 24.301 to align with the MICO mode functionality. (i.e. add text to say “To enable an emergency call back, the UE should wait for a UE implementation-specific duration of time before requesting the use of Power Save Mode after the release of the emergency PDU session.”)

Proposal 5: discuss and agree CR 3898 to TS 23.401 in S2-2502147

## ******************************************************************

## Annex A

### A.1	eDRX in EPC, in TS 24.301 v18.7.0, clause

/…/

If the UE received the extended DRX parameters IE during the last attach or tracking area updating procedure, upon successful completion of the PDN disconnect procedure of the PDN connection for emergency bearer services or EPS bearer context deactivation procedure of the EPS bearer context for emergency, the UE shall resume eDRX.

If the network has provided the extended DRX parameters IE during the last attach or tracking area updating procedure, upon successful completion of the PDN disconnect procedure of the PDN connection for emergency bearer services or EPS bearer context deactivation procedure of the EPS bearer context for emergency, the network shall resume eDRX.

If the UE or the network locally releases the PDN connection for emergency bearer service, the UE or the network shall not use eDRX until the UE receives eDRX parameters during a tracking area updating procedure with EPS bearer context synchronization or upon successful completion of a service request procedure.

If the UE did not receive the extended eDRX parameters IE, or if the UE has a PDN connection for emergency bearer services, the UE shall use the stored UE specific DRX parameter, if available.

If the network did not accept the request to use eDRX, or if the UE has a PDN connection for emergency bearer services, the network shall use the stored UE specific DRX parameter, if available.

/…/

### A.2	eDRX in 5GC, TS 24.501 v18.7.0 clause 5.3.16

/…/

The UE and the network may negotiate eDRX parameters during a registration procedure when the UE has an emergency PDU session.

The network accepts the request to use the eDRX by providing the Negotiated extended DRX parameters IE when accepting the registration procedure. The UE shall use eDRX only if it received the Negotiated extended DRX parameters IE during the last registration procedure and the UE does not have an emergency PDU session.

NOTE 1: If the UE wants to keep using eDRX, the UE includes the Extended DRX parameters IE in each registration procedure.

If the UE received the Negotiated extended DRX parameters IE during the last registration procedure, upon successful completion of the PDU session release procedure of the emergency PDU session, the UE shall resume eDRX.

If the network has provided the Negotiated extended DRX parameters IE during the last registration procedure, upon successful completion of the PDU session release procedure of the emergency PDU session, the network shall resume eDRX.

If the UE or the network locally releases an emergency PDU session, the UE or the network shall not use eDRX until the UE receives eDRX parameters during a registration procedure with PDU session context synchronization or upon successful completion of a service request procedure with PDU session context synchronization.

If the UE did not receive the Negotiated extended DRX parameters IE, or if the UE has an emergency PDU session, the UE shall use the stored UE specific DRX parameter, if available.

If the network did not accept the request to use eDRX, or if the UE has an emergency PDU session, the network shall use the stored UE specific DRX parameter, if available.

/../

*********** end ********