3GPP TSG-WG SA2 Meeting #167	S2-2502159

Feb. 17 - 21, 2025; Athens, Grace

Source:	China Mobile

Title:	Discussion on Addressing the EN: KI#4 parameters processing for end-to-end ciphered XRM flows

Document for:	Information

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2 / Rel-19

Abstract: this document discusses & analyses how to address the EN that is related to the processing of (S)RTP multiplexed media identification information (KI#4) associated with ciphered multiplexed media flows.

# General Discussion

There is currently an EN in the TS 23.501 clause 5.37.9.1, discussing how the KI#4 introduced parameters could be handled via the (KI#2 related) on-path N6 signalling methods, quoted as:

Editor's note:	Other parameters related to KI#4 are FFS.

The XRM KI#4 supports the multiplexing of multiple media flows on the same end-to-end transport layer connection. For this objective, it has introduced a new set of parameters for traffic identification, namely the (S)RTP Multiplexed Media Identification Information, that have been extended via the IP packet filter set as described in the clause 5.7.6.2 of TS 23.501. KI#4 provides the mechanism for both DL and UL media flow traffic identification and provisioning. The UL case depends on the indication of a UE’s capability supporting the feature.

The XRM KI#2 supports the transfer of media related information (also called MRI in this DP) over N6, where MRI is sent, with encryption and integrity protection, only in the DL direction to UPF (from external AS). Therefore,

Proposition 1.1: The delivery of encrypted and integrity-protected (S)RTP Multiplexed Media Identification Information off the N6 interface will be supported ONLY in the DL direction via the on-path N6 signalling methods as described in the clause 5.37.9 of TS 23.501.

TS 23.501 clause 5.37.9 standardizes three different on-path N6 signalling methods, i.e., Media over QUIC or MoQ, connect-UDP via Proxying-UDP-in-HTTP and UDP-options. All the three methods are supported by IETF RFCs or WG drafts, respectively. They support the delivery of encrypted and integrity-protected media related information (MRI) over the N6 interface in the DL direction. The MRI is comprised of PDU Set information, data burst size, TTNB, etc. (clause 5.37.9). What we want to emphasize is that, in addition to the delivery of ciphered MRI case, all the MRI can be transferred in plaintext mode via RTP header extensions as in TS 26.522 (i.e., one-byte or two-byte header-extension format). In term of the ciphered mode, there is no need in TS 23.501 to use explicitly any normative text to standardize the detailed mappings between MRI and protocol header coding for all the three on-path N6 signalling methods. As in the current version of the clause 5.37.9 [TS 23.501], some method(s) could use local-configured or operator-determined mappings, while other method(s) can delegate the mappings to stage-3.

In term of the KI#4 introduced (S)RTP Multiplexed Media Identification Information, they are nothing but a bunch of (S)RTP or (S)RTCP header fields, e.g., SSRC, PT, MID, RTCP PT, etc. They are not different from the MRI and should be treated in the same way when they are transferred downlink in the ciphered mode. Therefore, the (S)RTP Multiplexed Media Identification Information does not need any normative text to standardize their mappings in TS 23.501 for any of the three on-path N6 signalling methods:

Proposition 1.2: Just like what has been done for the DL transfer of encrypted & integrity-protected media related information (MRI), the (S)RTP Multiplexed Media Identification Information does NOT need normative text to standardize the mappings for any one of the three on-path N6 signalling methods as described in the clause 5.37.9 of TS 23.501.

# MoQ for KI#4

## Current MoQ Mapping Scheme

Currently, the Media over QUIC or MoQ is fairly versatile and powerful. It can use the metadata in an MoQ object header to transfer the ciphered MRI (media related information). The clause 5.37.9.2 has explicitly stated that the mappings between the MoQ Metadata and the PDU Set Information, EoDB, etc., can be based on local configuration, or operator-determined implementations.

Different mapping schemes exist. One option is to apply the bit-overloading of MoQ object metadata as shown in the following example:

the pink-highlighted octet in the one-byte RTP header extension (in the left picture) can be mapped to the MoQ object metadata via the bit-overloading of the ‘publisher priority’ field. Other fields could be mapped similarly.

There are certainly many other mapping variations. So, there is no need to standardize the detailed mappings in TS 23.501.

In term of the KI#4 (S)RTP Multiplexed Media Identification Information, the similar mapping schemes could be adopted. Therefore, there is no need to provide normative text to standardize how the mappings would be done.

Proposition 2.1: The current mapping schemes between the MoQ Metadata and the MRI, being either local-configuration based or operator-determined implementations or anything else, can be applied to the mappings of the (S)RTP Multiplexed Media Identification Information.

## Extended MoQ Mapping Scheme

Currently, the MoQ IETF draft is evolving to incorporate a generic way to extend the MoQ object header. As show in the link: https://github.com/moq-wg/moq-transport/pull/502/commits/779e3d7a6e673dfbff07700fa727db7deb1302a4, this MoQ interim-draft has provided normative texts to allow the support of multiple extension header types. As of today, the IETF MoQ WG and also the IETF WIT AD (area director) have together laid out the plan to publish the draft soon (most likely in the next few days @ https://datatracker.ietf.org/doc/draft-ietf-moq-transport/ ).

As quoted from the above-referenced link, the MoQ object header would be extended to:

- Extension count: the number of object extensions present.

- Extension headers: varied number of Extension Headers; the number is determined by ‘Extension count’.

- Extension header: a TLV data structure to identify the type/length/value of an extension. Also, the header types will be registered in the IANA table ‘MOQ Extension Headers’ as shown below.

Also, the working draft has indicated the corresponding IANA considerations:

- MOQ Extension headers - … to reserve extension types 0-127 for standards utilization where space is a premium, 128 - 16383 for standards utilization where space is less of a concern, and 16384 and above for first-come-first-served non-standardization usage.

When integrating all the IETF normative extension work together, the MoQ draft with the said object header extension will be an evolving way to handle both the current MRI mapping and the KI#4 (S)RTP multiplexed media identification information.

Proposition 2.2: The mappings between the MoQ Metadata and the MRI, along the (S)RTP Multiplexed Media Identification Information, can be addressed by the working MoQ IETF draft with the MoQ object header extension. The extension type and the format of the mappings can be further defined by 3GPP CT working group.

# UDP-option for KI#4

As in the current version of TS 23.501 clause 5.37.9.4, the on-path N6 signalling method, UDP-option, can be applied to encode the MRI (media related information) for the encrypted & integrity-protected transfer through the UDP tunnel over N6. The 3GPP only needs to coordinate a request for the IANA assignment of a new Kind from the (recommended) SAFE range [10…126] of the UDP option registry, e.g.,

Kind     Length       Meaning

------------------------------

TBD      Varied       XRM MRI

Without much variation, the same mechanism can be extended to accommodate the KI#4 (S)RTP multiplexed media identification information.

Proposition 3: The on-path N6 signalling method, i.e., the UDP-option, can be applied to accommodate both the MRI and the (S)RTP Multiplexed Media Identification Information.

# Connect-UDP for KI#4

As in the current version of TS 23.501 clause 5.37.9.3, the on-path N6 signalling method, i.e., connect-UDP as in RFC-9298 & 9297, can be applied to encode the MRI (media related information) for the encrypted & integrity-protected transfer via HTTP datagrams over N6. The normative text further clarifies the mechanism for exchanging HTTP Datagrams and the associated format is defined in RFC 9298 and RFC 9297. The RFC-9298 provides the HTTP datagram payload format as:

UDP Proxying HTTP Datagram Payload {

Context ID (i),

UDP Proxying Payload (..),

}

The same RFC-9298 also specifies the semantics of ‘Context ID’ can be used to identify the payload type of the datagram. The ‘Context ID’ allows extensions to exchange HTTP datagrams carrying different semantics in the UDP proxying payloads. The namespace of ‘Context ID’ is tied to a given HTTP request, meaning it is possible for a Context ID with the same numeric value to be simultaneously allocated in distinct requests, potentially with different semantics. When the Context ID field is set to zero, the UDP Proxying Payload field contains the unmodified payload of a UDP packet. When the Context ID field takes some other value that is pre-negotiated between the UPF and AS, the MRI can be carried over the corresponding proxying payload. Further, some other pre-negotiated ‘Context ID’ can be applied to accommodate both the MRI and the KI#4 (S)RTP multiplexed media identification information.

Proposition 4: The on-path N6 signalling method, i.e., the connect-UDP as in RFC-9298 & 9297, can be applied to accommodate both the MRI and the (S)RTP Multiplexed Media Identification Information.

# Conclusion

Based on all the Propositions in previous sections, we can draw the following conclusion:

Conclusion: All the three on-path N6 signalling methods, i.e., Media over QUIC or MoQ, connect-UDP via RFC-9298 & 9297 and UDP-options, can be applied to accommodate the encrypted and integrity-protected transfer of both the existing MRI and the KI#4 (S)RTP Multiplexed Media Identification Information.