3GPP TSG-WG SA2 #167	S2-2502162

# Feb. 17 – 21, 2025, Athens, Greece											(revision of S2-2500650r06)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1388 | rev | 9 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2-Ki4: Handling the ciphered (S)RTP Multiplexed Media Identification Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [OPPO, Xiaomi, Lenovo, Ericsson, Huawei, HiSilicon, Samsung, Nokia, NTT DOCOMO, vivo, Google], China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is an EN in 23.501 clause 5.37.9.1 addressing how the KI#4 related parameters, i.e., the (S)RTP multiplexed media identification information, could be handled with encryption & integrity-protection by the on-path N6 signalling methods, i.e., MoQ, connect-UDP and UDP-option. Based on the associated DP (S2-2502159), we have concluded that all the three on-path N6 signalling methods can handle the ciphered scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description for ciphered scenario (changes highlighted in azure). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The integration of KI#4-KI#2 is not correctly handled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.27.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The change in this CR will be the add-on to a new clause in 23.503 6.1.3.27.x that was originally proposed to be added by CR: S2-2412660. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of changes* * * *

##### 6.1.3.27.x	Traffic identification and differentiated QoS for multiplexed media flows

Traffic identification and differentiated QoS for multiplexed media flows is as defined in clause 5.37.11 of TS 23.501 [2].

When the UE indicates the support for (S)RTP Multiplexed Media Identification Information in IP Packet Filters to SMF at PDU Session establishment, the SMF provides this UE capability information to PCF.

In order to support differentiated QoS for multiplexed media flows multiplexed on the same end-to-end transport layer connection, the AF may request for differentiated QoS handling for multiplexed media flows by including (S)RTP Multiplexed Media Identification Information (see TS 23.501 [2] clause 5.7.6.2) in the Flow Description and specific QoS requirements per media flow as described in clause 6.1.3.22.

The PCF determines the PCC Rules and QoS requirements for the media flows under consideration of the UE support for  (S)RTP Multiplexed Media Identification Information in IP Packet Filters as follows:

- When the AF request includes uplink direction and the UE capability indicates support for (S)RTP Multiplexed Media Identification Information in IP Packet Filters, and/or the AF request includes downlink direction, PCF generates different PCC rules for the corresponding media flows. Each PCC Rule includes the QoS requirements that apply to the corresponding media flow identified by (S)RTP Multiplexed Media Identification Information in the IP Packet Filter set.

- When the AF request includes uplink direction and the UE has not indicated support for (S)RTP Multiplexed Media Identification Information in IP Packet Filters, the PCF may generate PCC rules based on operator configuration. The PCF may generate PCC rules for the uplink direction for the end-to-end transport layer connection and may notify the AF that the differentiated QoS handling for multiplexed media flows cannot be fulfilled. As an alternativeve, PCF may reject the AF request indicating lack of support for (S)RTP Multiplexed Media Identification.

NOTE Y:	It is assumed the PSA UPF supports the IP Packet Filter with (S)RTP Multiplexed Media Identification Information for multiplexed media.

When the multiplexed media flows with differentiated QoS handling on the same end-to-end transport layer connection require the encrypted and integrity-protected transfer of the (S)RTP Multiplexed Media Identification Information on the downlink direction over the N6 between the UPF and AS, the on-path N6 signalling methods along with the corresponding procedures as described in the clause 5.37.9 of TS 23.501 [2] can be applied. The encrypted and integrity-protected transfer of the (S)RTP Multipldexed Media Identification Information on the uplink direction is not supported.

# * * * * End of changes * * * *