SA WG2 Meeting #167	S2-2502191

# Athens, Greece, 17 – 21 February 2025	(was S2-2501040r05, S2-2502166)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3880 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Detach for S&F Mode Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARC |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Conclusions of TR 23.700-29 in clause 8.2 state the following: The following option is agreed for supporting Store and Forward operation with a full CN onboard the satellite with the following (informative) principles: - The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc. - The implementation of the proxies and the interface between them is out of 3GPP scope. - The UE attaches, transfers data (e.g. SMS, MO and MT data, etc.) and detaches from each satellite as required and as determined by the monitoring list. A detach may be performed by the onboard MME shortly before the UE will lose coverage. Alternatively, if coverage is lost before the detach has occurred, the UE can detach locally or remain attached and attempt to access a satellite later that has no NAS state for the UE. This can lead to a rejection of the access (e.g. with a cause #9) which will cause the UE to perform a new attach. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allow a UE to detach locally in Annex O.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE accessing a satellite in S&F mode with the full EPC solution may remain attached after accessing the satellite which will lead to NAS failure on the next access to a satellite in S&Fmode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex O.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Addition to S2-2501040r05 is shown in blue. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## O.3.2	Architecture and Principles of Operation

An example architecture of Model B is shown in Figure O.3.2-1. Each satellite contains the functionally of an eNB plus a full EPC that can include an MME, SGW, PGW, HSS, E-SMLC, SMSC etc. Each satellite further includes an endpoint proxy function that emulates the behaviour of a real endpoint (e.g. an AF) from the perspective of a UE. There is also store and forward functionality on the ground that can be part of, or connected to, an NTN Gateway and that contains the proxy functionality.

NOTE 1:	This architecture does not support roaming.

Figure O.3.2-1: Example Architecture of Model B

Editor's note:	Security aspects of the usage of the S&F Proxy are FFS.

For Model B, the signalling and procedures used between a UE and satellite to support UE access and UE services in S&F operation are the same as used between a UE and serving PLMN for normal satellite access except for the differences described in clause 4.13.9. A UE thus sees the onboard eNB and EPC as being equivalent to a serving PLMN for normal operation. Some PLMN services cannot be available for S&F operation due to subscription restriction for a UE or lack of support by the onboard eNB and/or EPC.

A UE accesses and attaches to a satellite for Model B as described in clause 4.13.9. The onboard EPC can obtain the UE location and verify that the UE is allowed to access the PLMN that was selected by the UE. The UE can then establish PDN connections to the onboard EPC and utilise them for the services that are supported in S&F operation.

Depending on what is supported by the onboard eNB and EPC, the UE can perform mobile originated (MO) transactions such as sending SMS, and sending data (e.g. using IP or non-IP protocols) using User Plane or Control Plane CIoT EPS optimisation. Each MO transaction is transferred by the onboard EPC to the onboard endpoint proxy which stores transaction data and signalling (e.g. SMS, data,) and associated protocol and remote endpoint data. The onboard endpoint proxy also returns responses to the UE at a transport and application level that are necessary to allow correct transport and application protocol operation, avoid timeouts and enable a user (if participating) to be aware of the one way communication status.

Shortly before satellite coverage will be lost, the onboard EPC can detach the UE.

The UE can detach locally if the UE based on implementation can infer the satellite coverage has already moved out of the area where the UE is located.

NOTE 2:	The UE can subsequently detach locally if the UE previously accessed a satellite operating in S&F mode for the same serving PLMN and was detached by the onboard MME after performing one or more NAS procedures successfully.

After the UE loses coverage and when the satellite obtains a feeder link to a ground based portion of the serving PLMN selected by the UE for the Attach, the satellite (or the onboard endpoint proxy) transfers data for the UE (e.g. the IMSI and last known UE location), and all of the stored data and signalling for the UE MO transactions, to an S&F function for the serving PLMN. The S&F function might contain a proxy that stores the data and signalling for the UE MO transactions and then forwards the data and signalling for each MO transaction to the associated remote endpoint. The proxy can also receive and store data and signalling for mobile terminated (MT) transactions that can be returned by the remote endpoints to the UE.

The S&F function and proxy can simulate continuous reachability of the UE. The MT transaction data and signalling for the UE (e.g. the IMSI) received and stored by the proxy are transferred to one or more satellites that are expected to later provide coverage to the UE.

When MT data is transferred to the satellite(s), the onboard endpoint proxy subscribes to the reachability of the UE and optional PDN connectivity status related information for data transmission, e.g. T8 destination address for non-IP data, UE IP address for IP data. When the UE accesses and attaches to the satellite, the endpoint proxy sends MT data to the UE based on the notification from onboard EPC, e.g. sends non-IP data to the T8 destination address or replaces the target IP address for IP data to UE's IP address and sends to PGW.

# * * * * End of changes * * * *