3GPP TSG-WG2 Meeting #167	S2-2502255

# 17th – 21st February, 2025, Athens, Greece                                      (revision of S2-2501455)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0643 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to AI/ML-based positioning procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, [vivo], Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR proposes a number of changes addressing the ENs below and completing/clarifying some aspects in the procedures for AIML-based positioning, related to the exception sheet approved in SP-241513. In addition, other miscellaneous changes are proposed to clarify the specification as described below. Rev2: Details from user consent check and subscription are missing from the procedure, both figure and description. Some of the wording is borrowed from NWDAF user consent specified in TS 23.288 for Rel-17. In addition, the UE location estimation is collected for training and performance monitoring, but the LCS privacy profile for the UE/user to allow such prevacy-sensitive information to be shared with the network is not checked. The EN can be addressed after receiving feedback from SA3. |  |  |  |  |  |  |  |  |
| Summary of changes: |  | Following changes are proposed for 6.22.2: - Clarifications on user consent are proposed indicating LMF behaviour when user consent is not provided or revoked Following changes are proposed for 6.22.4: - A NOTE is added indicating the input data for AIML-based positioning is not in the scope of this specification. - User consent revocation is clarified Rev2: - Changes in figure and procedure description providing details on user consent handling. - EN on use of privacy profile by LMF - Removal of generic text on user consent revocation - Editorial changes in 6.22.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification of AI/ML based positioning won’t be completed on time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 23.273 CR 0642 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 23.273 CR 0644 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 6.22.2	Data Collection at LMF for the LMF-based AI/ML positioning based on UE measurements

The LMF needs to obtain input data for LMF-based AI/ML Positioning model training, performance monitoring and may also request the UE to provide the UE location. The LMF-based AI/ML Positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

Figure 6.22.2-1: Data collection at LMF for the LMF-based AI/ML positioning based on UE measurements

1.	The LMF starts data collection for the purpose to train an AI/ML Model for UE Positioning (this may be based on an internal trigger in the LMF or a request from the NWDAF). The LMF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover AMF(s) that served for the area of interest.

2.	The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest and the UE Positioning Capability for each UE using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest" and "UE Positioning Capability".

Editor's note:	Whether and how to limit the number of UEs is FFS.

3.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest"). If "UE Positioning Capability" is also requested in step 1, AMF includes UE Positing Capability, UE User Plane Positioning Capabilities if available for each SUPI in the response message sent to LMF.

4.	The LMF checks whether each SUPI provided user consent for data collection with UDM using Nudm_SDM_Get including data type "User consent". If user consent is not granted, the data collection for this SUPI stops here and no other steps for this SUPI are performed.

5.	If the user consent is granted, the LMF subscribes to UDM to notifications of changes on subscription data type "User consent" for this SUPI using Nudm_SDM_Subscribe. 

The LMF further determines the UEs from the list of SUPIs that received from AMF in step 2 for data collection based on UE Positioning Capability, UE User Plane Positioning Capabilities, the PRU information available in the LMF and operator's policy. The User consent check is not needed for data collection from PRU for performance monitoring.

Editor's note:	Further details on user consent for data collection for a specific purpose, e.g., for model training and/or model performance monitoring for LMF-based AI/ML Model positioning will be aligned with SA WG3.

6.	For each UE where the user provides consent to data collection and the UE can support data collection, the LMF initiates a request for input data. The UE may reject the data collection request from the LMF (e.g. considering UE status, user's input). If the UE accepts data collection request, the UE may cancel the data collection later as defined in clause 6.3.4.

7. The UDM may notify the LMF on changes of user consent at any time after step 5, then the LMF checks if the user consent is not granted. If user consent was revoked for a UE for which data have been collected, the LMF performs step 8. The LMF may unsubscribe to be notified of user consent updates from UDM for each SUPI for which data consent has been revoked.

8.	The LMF stops any collection of data from the UE.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the UE is FFS. The procedure will be coordinated with RAN WGs.

Editor's note:	The input data used for AI/ML based positioning will be decided by RAN WGs.

* * * End of Changes * * *