SA WG2 Meeting #167	S2-2502265

# 17 - 21 February, 2025, Athens, Greece													1415 was 0232r05

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1327 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of VFL model training and inference with client intermediate results sharing between VFL clients |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Lenovo, OPPO, ZTE [Nokia, Apple,] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Rev2: Remove the scenarios when NWDAF or trusted AF is VFL server. Clarify unspecified information related into VFL Interoperability Information to support the negotiation between VFL server and VFL clients to support intermediate results sharing between VFL clients. Rev1: Based on discussion in S2-2501347, when VFL using split models, whether the client intermediate results can be aggregated is based on gradient. As SA2 has agreed, the VFL models are pre-configured and the supporting gradient information of VFL client are indicated (implicitly and explicitly) by VFL interoperability indicator(s) and VFL interoperability information used during discovery and preparation stage. Therefore, the existing discovery and preparation framework is sufficient enough to support the VFL server to choose VFL client aggregator and indirect VFL clients. No VFL model is to be shared in 3GPP standardised approach. There is an EN in 0232r05: Editor’s Note: It is FFS how the client aggregator is selected and may not be selected/configured by VFL server, and also if this capability needs to be registered into NRF Different from VFL server, the VFL client aggregator does not have VFL labels. It only aggregates the client intermediate results and then compute its own intermediate result. Therefore, the VFL client aggregator and indirect VFL clients still align the definition of VFL client. No additional capability is required. The simplest way is to allow the VFL client aggregator and indirect VFL clients can be selected by VFL server based on the interoperability information received during discovery and/or preparation, as the VFL clients do not share interoperability information in current framework. Step 2 of clause 6.2H.2.4.1 documented a scenario that no VFL client is selected for VFL inference. However, VFL requires the coordination between VFL clients and VFL server for VFL training and inference. The above scenario is not support in VFL; and therefore, should be removed. Rev0: The conclusions for VFL training documented in clause 8.2 of TR 23.700-84, including: P#2.4.5: VFL clients may also provide intermediate results (e.g. gradient information, loss information) to other VFL clients as instructed by the VFL server. As it has been agreed in expection sheet (in SP-241513/ S2-2413037), the following issue should be addressed: Vertical Federated Learning (VFL): - Whether and how to exchange intermediate training results between VFL clients during VFL training and inference As discussed in DP S2-2500233, VFL algorithms using split neutral network (NN) is widely used which can reduce the energy consumption and improve the overall VFL efficiency. When using the split neutral network (NN) network, the VFL clients may share client intermediate results with a selected VFL client during the model training and inference for intermediate results aggregation to distribute the computation load of VFL server. Furthermore, when the VFL server is untrusted AF, exposing the client intermediate results of every single VFL client may increase the risks of 5GC privacy issues, as no privacy preserving methods are specified. Therefore, support client intermediate results sharing between VFL clients is will beneficial for R19 AIML_CN VFL operation. To simplify 3GPP work, in this release, VFL model training and inference with VFL client information sharing will be only support among NWDAF VFL clients; the (untrusted) AF VFL clients are not considered. This CR specifies the procedures for VFL model training with client intermediate results sharing between VFL clients. The ENs on VFL client information sharing during VFL model and inference are removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce new steps to support VFL model training and inference with VFL client information sharing on top of the agreed procedures. - Remove ENs relevant to VFL client information sharing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions for FS_AIML_CN KI#2 or VFL cannot be fully supported by 5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.3.2, 6.2H.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

##### 6.2H.2.3.2	Training Procedure for Vertical Federated Learning untrusted AF is acting as VFL server

Figure 6.2H.2.3.2-1: Training procedure for Vertical Federated Learning when untrusted AF is acting as VFL server

Editor's note:	The ENs listed in clause 6.2H.2.3.1 are also applied to this clause.

1.	Same as step 1 in Figure 6.2H.2.3.1-1.

Steps 2-7 are repeated until the training termination condition is reached.

2.	To start VFL training, the VFL server do same as in step 2 in Figure 6.2H.2.3.1-1, using Nnef_VFLTraining_Subcribe.

2c.	If a NWDAF VFL client is selected to aggregate the intermediate training results of other VFL client clients , this NWDAF VFL client may send Nnwdaf_VFLTraining_Subscribe to other one or more indirect NWDAF VFL client as configured by the VFL server from which it desires to receive the client intermediate training results.

NOTE: to support the intermediate results sharing between VFL clients in step 2c, the VFL clients might be selected based on the VFL Interoperability Indicators and/or the parameters in VFL Interoperability Information (e.g. gradient dimension of local model, split point of the preconfigured initial model, etc.).

Steps 3-7 are repeated until the training termination condition is reached.

3.	[Optional] Same as step 3 in Figure 6.2H.2.3.1-1.

4.	Same as step 4 in Figure 6.2H.2.3.1-1.

5.	Same as step 5 in Figure 6.2H.2.3.1-1.

5a.	A NWDAF VFL client sends a Nnwdaf_VFLTraining_Notify.

5b.	For an untrusted AF acting as VFL server, the NEF converts any internal identifiers to external identifiers, provides the external NWDAF ID and sends a Nnef_VFLTrainingNotify to the VFL server.

5c-5d The NWDAF VFL clients share the client intermediate training results with the NWDAF VFL client from which it received subscription request in step 2c. This NWDAF VFL client aggregates the received client intermediate training results from the NWDAF VFL clients, performs local computation using the aggregated intermediate inference results, then sends one Nnef_VFLTraining_Notify to the NEF by including its client intermediate training result.

6.	[Optional] Same as step 6 in Figure 6.2H.2.3.1-1.

7.	Same as step 7 in Figure 6.2H.2.3.1-1.

8.	Same as step 8 of Figure 6.2H.2.3.1-1. However, sub steps in that figure are not applicable.

8a.	For each NWDAF VFL client, the untrusted AF as VFL server sends a Nnef_VFLTraining Unsubscribe to the NEF handling that AF. The untrusted AF identifies the VFL client using the external NWDAF ID assigned in the discovery procedure (see clause 6.2H.2.1.1).

8b.	The NEF sends an Nnwdaf_VFLTraining_Unsubscribe to the NWDAF VFL client indicated by the received external NWDAF ID.

8c.	An NWDAF VFL client may send Nnwdaf_VFLTraining_Unsubscribe to other one or more NWDAF VFL client in step 2c.

9.	Same as step 9 of Figure 6.2H.2.3.1-1.

* * *Next Change * * *

##### 6.2H.2.4.2	Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

The inference procedure when untrusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. If triggerd by internal service logic of the AF acting as VFL server, the steps 0, 1,7 and 8 are skipped. The inference procedure triggerd by internal service logic of the AF acting as VFL server is out of 3GPP scope.

0.	Same as step 0 in clause 6.2H.2.4.1.

1.	Same as step 1 in clause 6.2H.2.4.1, to NEF using Nnef_Inference_subscribe. NEF forwards the subscription request to AF using Naf_Inference_subscribe.

Editor's note:	When the AnLF determine the VFL server AF is FFS. For example, during the training phase.

Editor's note:	The service name between the AnLF and untrusted AF as VFL server is FFS.

2.	Same as step 2 in clause 6.2H.2.4.1, to NEF using Nnef_VFLInference_subscribe. An untrusted AF includes the external NWDAF ID and sends the request to the NEF.

NEF converts any received external identifiers to internal identifiers and forwards the subscription request to NWDAF using Nnwdaf_VFLInference_subscribe.

2c	An NWDAF VFL client may send Nnwdaf_VFLInference_Subscribe or Nnwdaf_VFLInference_Request to the VFL client to other one or more NWDAF VFL client as configured  from which it desires to receive the client intermediate inference results.

NOTE: to support the intermediate results sharing between VFL clients in step 2c, the VFL clients might be selected based on the VFL Interoperability Indicators and/or the parameters in VFL Interoperability Information (e.g. gradient dimension of local model, split point of the preconfigured initial model, etc.).

Editor's note:	It is FFS additional parameters are needed to send from the VFL server to VFL client.

3.	Same as step 3 in clause 6.2H.2.4.1.

4.	Same as step 4 in clause 6.2H.2.4.1.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

5.	Same as step 5 in clause 6.2H.2.4.1, to NEF using Nnwdaf_VFLInference_Notify.

NEF converts any received internal identifiers to external identifiers and forwards the subscription notify to the untrusted AF using Nnef_VFLInference_Notify.

5c-5d. The NWDAF VFL clients share the client intermediate inference results with the NWDAF VFL client from which it received request in step 2c. This NWDAF VFL client aggregates the received client intermediate inference results, performs local computation using the aggregated intermediate inference results, then sends one notify message to the NEF by including its client intermediate inference result.

Editor's note:	It is FFS additional parameters are needed to send from the VFL client to VFL server.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

6.	Same as step 6 in clause 6.2H.2.4.1.

7.	Same as step 7 in clause 6.2H.2.4.1, to NEF using Naf_Inference_Notify.

NEF converts any received internal identifiers to external identifiers and forwards the subscription Notify to NWDAF using Nnef_Inference_Notify.

8.	Same as step 8 in clause 6.2H.2.4.1.

* * *End of Changes * * *