3GPP TSG-WG SA2 Meeting #167	S2-2502341

Athens, Greece, 17 – 21 Feb, 2025	(revision of S2-2500927, 1975)

Source:	Huawei, HiSilicon

Title:	pCR to TS 23.xyz: Procedure for Ambient IoT Inventory Service

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: The Ambient IoT Inventory Service is introduced to the specification. In addition, the procedure will consider both direct architecture and indirect architecture.

# 1. Introduction/Discussion

The pCR aims to provide the procedure call flow and description of Ambient IoT Inventory Service. The procedure uses direct architecture option as the baseline with additional reference to capture the difference in indirect architecture option.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xyz.

# * * * * First change (all new texts)***

## 6.1	General

Editor’s Note: The contents of this general clause is FFS and is expected to describe there are service procedures which are reader type/routing agnostic and there are procedures which relate to the transports to the readers.

## 6.2	Ambient IoT Service Procedures

### 6.2.1	General

Clause 6.2.2 provides the procedure for Ambient IoT Inventory. Clause 6.2.3 provides the procedure for Ambient IoT Command.

Editor’s Note: Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor’s Note: Alignment is required for how to document and describe Direct Path and Indirect Path options in the procedures.

### 6.2.2	Inventory Procedure

Figure 6.2.2-1 described the inventory procedure.

Figure 6.2.2-1: Inventory Procedure

1.	The AF sends AIoT Inventory service operation request to the NEF, carrying the AF ID, information used to identify readers and optionally AIoT Device Identification information to identify devices.. If AIoT Device Identification information is not provided then the operation is towards all devices.

2.	The NEF selects an AIOTF to handle the request. .

3.	NEF sends the AIoT Inventory service operation request to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request.

If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a correlation ID corresponding to this AF service operation request, as well as the AIoT Device Identification information to be included in the paging message sent by an AIoT RAN. The AIoT Device Identification information enables the identification of a single AIoT Device, multiple AIoT Devices or all AIoT Devices.

AIOTF performs Reader Selection based on the information provided by the AF, see clause 5.3.

The AIOTF may also use the last serving Reader in the AIOTF to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including the correlation ID, the AIoT Device Identification information to be included in the paging message and assistance information to the selected AIoT Readers.

8.	The AIoT RAN sends an Inventory Response to the AIOTF with the correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

9.	Upon reception of the Inventory Request message, the AIoT Reader(s) will execute the inventory operation.

If an AIoT device matches the AIoT Device Identification information in the paging message, the AIoT Device

responds to the inventory procedure and sends an AIOT NAS message that includes its AIoT Device ID.

NOTE 2:	Whether and how the Device ID is concealed or encrypted  will be determined and aligned with SA WG3.

10. AIoT RAN sends an Inventory Report message to AIOTF including the correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s).

11.	The AIOTF performs checks the AIoT Device ID based on the Ambient Device profile data received from the ADM.

12.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the AIoT_Notify message including a list of AIoT Device ID(s).

13.	The NEF informs the AF of the outcome of the AIoT_Inventory request by sending the AIoT_Notify message including the AIoT Device ID(s).

# * * * * End of changes * * * *