SA WG2 Meeting #167	S2-2502344

17 - 21 February, 2025, Athens, Greece	(revision of S2-2501944)

Source:	ZTE, vivo

Title:	pCR to TS 23.xyz: NEF Services

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: The paper adds NEF service definition.

1. Discussion

The paper defines the NEF services which include Inventory, Command (Read, Write, Disable), and Notify service operations; these services are consumed by AF.

2. Proposal

It is proposed to capture the following changes into TS 23.xyz.

* * * * Start of Changes * * * *

7.4	NEF services

In addition to those defined in TS 23.501 [3] clause 7.2.8 and TS 23.502 [4] clause 5.2.6, the table 7.4-1 illustrates additional NEF services to support Ambient IoT.

Table 7.4-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_AIoT | Inventory | Request/Response | AF |
|  | Command | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |

7.4.1	Nnef_AIoT_Inventory service operation

Service operation name: Nnef_ AIoT_Inventory

Description: The consumer requests to perform an inventory device operation for an AIoT device or multiple AIoT devices.

Input, Required: AF ID, Target area information.

Input, Optional: AIoT Device ID(s) or AIoT Device ID filter information, Approximate number of AIoT Devices.

Output, Required: Correlation ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

7.4.2	Nnef_AIoT_Command service operation

Service operation name: Nnef_ AIoT_Command

Description: The consumer requests to perform a command operation for an AIoT device or multiple AIoT devices.

Input, Required: AF ID, Target area information, AIoT Device ID(s) or AIoT Device ID filter information, Command type (Read, Write, or Permanent Disable), Application layer information to be configured to the target AIoT device(s) in case of Write command type.

Input, Optional: Approximate number of AIoT devices, Approximate message size from the AIoT Device for Read command type,

Output, Required: Correlation ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

7.4.3	Nnef_AIoT_Notify service operation

Service operation name: Nnef_ AIoT_ Notify

Description: The consumer receives notification of the status or results of the requested service operation. If the consumer invokes the Nnef_AIoT_Inventory, or Nnef_AIoT_Command service operation, the consumer implicitly subscribes to the results of the requested service operation.

Input, Required: Correlation ID, List of AIoT Device ID or Failure Cause in case of Failure, Information obtained from each target Ambient IoT Device.

Input, Optional: None.

Output, Required: Result indication.

Output, Optional: None.

NOTE:	The AIoT service operation definition for NEF should align with AIOTF services.

* * * * End of Changes * * * *