SA WG2 Meeting #S2-167	S2-2502355

17 - 21 February, 2025, Athens, Greece	(revision of S2-25002042)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6028 | rev | 2 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding per UE per Application level Energy consumption exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Mobile, NTT DOCOMO, Vivo, NEC, Huawei,? HiSilicon? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is an EN in the clause 5.51.2.4 on per application energy consumption information exposure. Editor's note: Whether and how the per application energy consumption information can be exposed are FFS. In the existing third bulletin, it is the QoS flow level exposure. For 5GC NF, it could be Qos flow level. But it is not the case for AF AF is not aware of Qos flow level. AF can only provide application level information. So this CR proposes to add per UE per application level granularity exposure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Application lelvel Energy consumption calculation and exposure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EN on per application energy consumption information exposure is not resolved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.51.2	Energy Consumption Information collection, calculation and exposure

#### 5.51.2.1	General

The Energy Information Function (EIF) is defined to, subject to operator's policy:

-	collect the UE related Energy Consumption information,

-	calculate the Energy Consumption information at UE, PDU Session and per UE per Application granularity, and

-	expose the Energy Consumption information to the authorized consumer NF(s) (AF/NEF or 5GC NF).

Node level energy consumption is reported PLMN-wide at a configurable period T.

#### 5.51.2.2	Energy Consumption information collection

The Energy Information Function (EIF) collects the UE related Energy Consumption information include Node-level energy consumption information, Node-level data volume from OAM and data volume of the required granularities (i.e. PDU Session and/or per UE per Application) from UPF (via SMF).

The serving SMFs are retrieved from the UDM of the UE based on the provided input parameters including the UE ID and (S-NSSAI, DNN).

EIF provides UE ID, DNN/S-NSSAI, application information (e.g. Application Identifier, or Service Data Flow description(s)) to retrieve the information from SMF. And the information collected from SMF by EIF, is shown in Table 5.51.2.2-1.

The data volume reporting period from the SMFs is the period T with PLMN-wide configuration (see clause 5.51.2.1).

Table 5.51.2.2-1: Information from SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE IP address | UE IP address. |
| UE ID | SUPI. |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| Packet Filters | Packet Filters as in 5.7.6 for IP or Ethernet traffic. |
| Application Identifier | Identification for the traffic of the service data flow. |
| List of Data Volume information | The data volume and the associated UPF(s) and gNB(s) serving the UE within the time period. |
| > UL/DL Data Volume of UPF | The UL/DL Data Volume of a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a Service Data flow (UE ID, S-NSSAI, DNN, Packet Filters/Application Identifier). |
| > (I-)UPF ID(s) | Identifier of any (I-)UPF(s) associated to a reported data volume used by a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a Service Data flow (UE ID, S-NSSAI, DNN, Packet Filters/Application Identifier). |
| > gNB serving the UE | The gNB which serving the UE. |
| Time period | Time period of the collected information (see clause 5.51.2.1). |

Table 5.51.2.2-2 provides the list of information received from OAM. EIF requests the OAM information with providing the serving gNB ID(s) and (I-)UPF ID(s), which provided by SMFs. The reporting period from the OAM is the period T which is the same as data volume reporting period from SMFs.

Table 5.51.2.2-2: Information from OAM for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| gNB energy consumption | The Energy consumed by a gNB over the configured time period T based on clause 6.7.3.4.2 of TS 28.554 [209]. |
| gNB data volume | The UL/DL data volume handled by a gNB over the configured time period T based on clause 6.7.1.1 of TS 28.554 [209]. |
| UPF energy consumption | The Energy consumed by a UPF over the configure time period T based on clause 6.7.3.1 of TS 28.554 [209]. |
| UPF data volume | Data volume consumed at a UPF. |

When the gNB and/or the (I-)UPF(s) which serving the UE are changed, the serving gNB ID and UPF ID will be updated to the EIF through SMF(s).

NOTE 1:	The Node-level Energy Consumption information received from OAM could be used by EIF for all the UEs serving by the NF Node.

NOTE 2:	In this Release, only the energy related information of user plane communication is supported and that of control plane signalling is not supported.

5.51.2.3	Energy Consumption information calculation

EIF calculates the Energy Consumption information of the required granularities (UE, PDU Session and/or  per UE per Application), based on input parameters from Tables 5.51.2.2-1 and 5.51.2.2-2 and gets the results. Some example formulas to support the above calculation are described in Annex T.

#### 5.51.2.4	Energy Consumption information exposure

AF/NEF or 5GC NF subscribes the EIF for Energy consumption information of required granularities (UE, PDU session and/or per UE per Application).

-	For UE level energy exposure, the consumer NF provides UE ID (SUPI/GPSI).

-	For PDU session level exposure, the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI.

- For per UE per application-level exposure (i.e. Service Data Flow level), the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI and and application information, e.g. Application Identifier, or Service Data Flow description(s).

-	The consumer NF may also subscribe the above information exposure with providing reporting period, reporting frequency.

Editor's note:	Whether there are other required parameters for energy consumption information exposure are FFS.

Editor's note:	Whether and how the renewable energy is introduced is FFS and depending on SA WG5.

* * * End of Changes * * * *