3GPP TSG-WG SA2 Meeting #167	                                                       	         S2-2502356

# Athens, Greece, 17 – 21 February 2025		(revision of S2-2501211)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5289 | rev | 3 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification regarding energy collection and exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Huawei, HiSilicon, ETRI], China Mobile, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In SA2#166 meeting, the basic procedure for energy information collection, calculation and exposure was specified in 23.502, and this is a good starting point for support the energy features in Rel-19. This document tries to further update the procedures with clarifying on the following aspects: 1. Add the details of the subscription; 2. Add the details of the requests in the procedure; 3. Modify the interaction between SMF and UPF, i.e., only the PSA UPF. 4. Other editorial aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the details of the subscription; Add the details of the requests in the procedure; Modify the interaction between SMF and UPF, i.e., only the PSA UPF. Other editorial aspects. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The energy collection, calculation and exposure procedures are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.29, 5.2.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

4.29	Energy consumption information collection and exposure

4.29.0	General

This clause describes how EIF collects the energy consumption information and exposes the energy consumption information to the Consumer NF.

4.29.1	Energy Consumption information exposure

The procedure is used by the AF to subscribe to notifications and to explicitly cancel a previous subscription for Energy Consumption information exposure.

Figure 4.29.1-1: Energy consumption information exposure

1a.	The untrusted AF (un)subscribes  energy consumption event by invoking the Nnef_EventExposure_(Un)Subscribe service operation. If the energy consumption information (un)subscription request is authorized by the NEF, the NEF proceeds with the steps below.

1b.	Based on the request from the untrusted AF, the NEF (un)subscribes energy consumption event by invoking the Neif_EventExposure_(Un)Subscribe/ service operation.

1c.	The trusted AF (un)subscribes to energy consumption event directly to EIF by invoking Neif_EventExposure_(Un)Subscribe service operation.

2a.	The EIF acknowledges the (un)subscription  for the energy consumption event .

2b.	The NEF acknowledges the (un)subscription  for the energy consumption event  from untrusted AF.

2c.	The EIF acknowledges the (un)subscription for energy consumption event from trusted AF.

3a.	The EIF notifies the NEF with the energy consumption information by invoking Neif_EventExposure_Notify service operation.

3b.	If the NEF receives the notification from the EIF, the NEF notifies the untrusted AF with the energy consumption information by invoking Nnef_EventExposure_Notify service operation.

6c.	The EIF notifies the trusted AF with the energy consumption information by invoking Neif_EventExposure_Notify service operation.

4.29.2	Energy Consumption information collection

Figure 4.29.2-1 shows the procedure for EIF to collect the Energy consumption information.

Figure 4.29.2-1: Energy Consumption information collection

1.	The Consumer NF (i.e. trusted AF or 5GC NF) subscribes  to the energy consumption event  with the EIF by using Neif_EventExposure_Subscribe service for the required granularities (UE, PDU Session and/or QoS flow) as described in clause 5.51.2.3 of TS 23.501 [2].

If the Consumer is untrusted AF, the subscription is sent via NEF. If the event subscription is authorized by the NEF, then the NEF sends the event subscription to the EIF via Neif_EventExposure_Subscribe.

2.	The EIF invokes Nudm_UECM_Get service operation to retrieve the SMF ID for the required granularity.

3.	The EIF invokes the Nsmf_EventExposure_Subscribe request to the SMF to request the data volume of the required granularity and gNB ID as defined in clause 5.51.2 of TS 23.501 [2], .

4.	The SMF informs the UPF with requesting the PSA UPF to report the data volume of the required granularity.

5. The SMF provides the information for energy consumption calculation (defined 5.51.2 of TS 23.501 [2])  to EIF by invoking Nsmf_EventExposure_Notify service operation for the required granularity.

NOTE:	The SMF can derive gNB ID(s)s according to the AMF provided ULI as described in clause 4.3.2.

6. If there is no related Node-level energy consumption information and Node-level data volume, the EIF obtains the Node-level energy consumption information and Node-level data volume from OAM with providing gNB  ID(s) and UPF ID(s).

7.	The EIF performs the energy consumption information calculation as described in clause 5.51.2.3 of TS 23.501 [2] and then reports the energy consumption information of the required granularity to the Consumer NF via Neif_EventExposure_Notify.

# * * * * Second Change* * * *

5.2.8.1	General

The following table shows the SMF Services and SMF Service Operations.

Table 5.2.8.1-1: NF services provided by the SMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nsmf_PDUSession | Create | Request/Response | V-SMF/I-SMF |
|  | Update | Request/Response | V-SMF/I-SMF, H-SMF |
|  | Release | Request/Response | V-SMF/I-SMF |
|  | CreateSMContext | Request/Response | AMF |
|  | UpdateSMContext | Request/Response | AMF |
|  | ReleaseSMContext | Request/Response | AMF |
|  | SMContextStatusNotify | Subscribe/Notify | AMF |
|  | StatusNotify | Subscribe/Notify | V-SMF/I-SMF |
|  | ContextRequest | Request/Response | AMF, V-SMF/I-SMF, SMF |
|  | ContextPush | Request/Response | SMF |
|  | SendMOData | Request/Response | AMF |
|  | TransferMOData | Request/Response | V-SMF/I-SMF |
|  | TransferMTData | Request/Response | SMF, H-SMF |
| Nsmf_EventExposure | Subscribe | Subscribe/Notify | NEF, AMF, NWDAF, SMF, EIF |
|  | Unsubscribe |  | NEF, AMF, NWDAF, SMF, EIF |
|  | Notify |  | NEF, AMF, NWDAF, EIF |
|  | AppRelocationInfo |  | AF |
| Nsmf_NIDD | Delivery | Request/Response | NEF |
| Nsmf_TrafficCorrelation | Notify | Subscribe/Notify | NEF |

# * * * * Third Change (newly added in rev x)* * * *

# * * * * End of changes * * * *