3GPP TSG-SA WG2 Meeting #167	S2-2502447

# 17-21 February 2025, Athens Greece	(revision of S2-2502269)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1596 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Network Capability Exposure Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to S2-2501099 endorsed at SA2#166 Ad-hoc-e meeting, update Annex AG.2.2 with detailed call flows of establishing IMS session with standalone bootstrap data channel(s), and standalone A2P application data channel(s) and remove the Editor’s Notes in Annex AG.2.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update Annex AG.2.2.x with detailed call flows of establishing IMS session with standalone A2P application data channel(s). - Remove Editor’s Notes in Annex AG.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Solution is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AG.2.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This should be implement after the last change of CR 1581 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * *  First Change * * * *

### AG.2.2.x	Establishing an IMS session with a standalone A2P application data channel

Figure AG.2.2.2-1 depicts the call flow of establishing an IMS session with a standalone A2P application data channel between the DC AS and UE A.

Figure AG.2.2.2-1: Establishment procedure of an IMS session with a standalone A2P application data channel

0.	The DC AS decides to establish an IMS session with a standalone A2P application data channel with UE A.

1.	The DC AS sends a Nnef_ImsSessionManagement_Create request to the NEF requiring to create an IMS session with a standalone A2P application DC with UE A.

The DC AS sends the media operation set including the parameters for the media type set as A2P ADC and the UE A as the target of the ADC to NEF. The DC AS may include a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Create request to be notified for the progress of the session creation.

2.	If the NEF authorizes the request, the NEF queries the HSS with Nhss_ImsUECM_AsInfoGet service operation to retrieve the IMS AS instance serving UE A.

3.	The NEF sends a Nimsas_ImsSessionManagement_Create request to the IMS AS-1 requiring to create an IMS session with a standalone A2P application DC. The media operation set includes the parameters for the media type set as A2P ADC and the UE A as the target of the A2P ADC. The NEF may include a Notification Target Address and Correlation ID to the Nimsas_ImsSessionManagement_Create request to be notified of the session create progress.

4.	The IMS AS-1 validates the user subscription data and checks the IMS DC capability of UE A to determine whether the data channel request should be notified to DCSF-1. If UE A does not have subscription of IMS data channel or does not have the IMS DC capability, then the request shall be rejected with appropriate cause and the subsequent steps are skipped. If the IMS AS-1 allows the request to proceed, it returns a successful Nimsas_ImsSessionManagement_Create response to the NEF.

5.	The NEF returns a successful Nnef_ImsSessionManagement_Create response with the session ID generated by the IMS AS-1 to the DC AS.

6.	The IMS AS-1 sends a Nimsas_SessionEventControl_Notify request to the DCSF-1 with event set to 3rdPartySessionCreate. The notification includes the necessary parameters based on the information extracted from the media operation set received in step 3 and other stored parameters in the IMS AS-1 if applicable.

7.	After receiving the session event control notification, the DCSF-1 determines the policy how to process the bootstrap data channel establishment request based on the related parameters in the notification and/or DCSF-1 service specific policy.

8.	The DCSF-1 determines that the standalone A2P application data channel media requires the DC to be anchored in MF-1 and creates the originating side MDC1 media information required by MF-1.

9.	The DCSF-1 instructs the IMS AS-1 to set up the standalone A2P application data channel by sending a Nimsas_MediaControl_MediaInstruction request.

10.	Based on the instruction from DCSF-1, the IMS AS-1 interacts with the MF-1 to allocate data channel media resource towards UE A and MDC1 media resource towards the DCSF.

11.	The IMS AS-1 returns the Nimsas_MediaControl_MediaInstruction response to the DCSF-1.

12.	The DCSF-1 returns the Nimsas_SessionEventControl_Notify response to the IMS AS-1.

13. The IMS AS-1 generates an INVITE request in which the SDP offer contains the media information of the standalone A2P application data channel required by the DC AS, binding information and the session ID. The IMS AS-1 sends the INVITE request to UE A.

If UE A does not store the DC application specified in binding information locally, then the request shall be rejected with appropriate cause and the subsequent steps are skipped.

14. DC media negotiation may be completed through 18X/PRACK/200 OK(PRACK)/200 OK(INVITE)/ACK procedure.

15. The IMS AS-1 requests MF-1 to update DC AS side media resource information.

16. UE A runs the DC application specified in binding information and establish the standalone A2P application data channel with the DC AS.

17.	If the DC AS included a Notification Target Address in the Nnef_ImsSessionManagement_Create request in step 1, the IMS AS-1 notifies the NEF for the progress of the session creation with the Nimsas_ImsSessionManagement_Notify request.

18.	The NEF notifies the DC AS for the progress of the session creation with the Nnef_ImsSessionManagement_Notify request.

19. The DC AS returns a Nnef_ImsSessionManagement_Notify response to the NEF.

20. The NEF returns a Nimsas_ImsSessionMangement_Notify response to the IMS AS-1.

# * * * *  End of Changes * * * *