3GPP TSG-SA2 Meeting #167	S2-2502477

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5524 | rev | 13 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Title: | Support Alternative QoS profile with PDU Set QoS and Notification Control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [CATT,] Nokia [, Tencent, vivo, Samsung, Xiaomi] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To implement the conclusion in clauses 8.1.2 and 8.1.3 of TR23.700-70 to support PDU Set QoS Notification without AQP, PDU Set QoS control with Alternative PDU Set QoS profile and Notification Control with Alternative PDU Set QoS profile. Existing QoS notification mechanism with(out) AQP relies on NG-RAN providing a simple notification “GFBR can no longer (or can again) be guaranteed”. By overriding this notification when PDU set QoS parameters are used by NG-RAN, the AF cannot know which QoS parameters have been used. This scenario will be very common as the AF does not control the selected NG-RAN and cannot be aware of its capabilities regarding PDU set QoS (Rel-19 or of a prior release). This CR updates TS23.501 to support the above according to XRM_Ph2 conclusions and as per the observations of the discussion paper S2-2502150. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the Notification Control on PDU Set QoS and Alternative PDU Set QoS profile(s) Support NG-RAN indication of PDU set QoS support. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Alternative PDU Set QoS profile based PDU Set QoS Control and Notification control cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Addition of the explicit indication by NG-RAN on PDU set QoS use as per discussion paper S2-2502150 |  |  |  |  |  |  |  |  |

## FIRST CHANGE

#### 5.7.2.4	Notification control

##### 5.7.2.4.1	General

The QoS Parameter Notification control indicates to the NG-RAN that notifications of "GFBR can no longer (or can again) be guaranteed" are requested when the NG-RAN determines that the GFBR, the PDB or the PER of the QoS profile cannot be fulfilled (or can be fulfilled again) for a QoS Flow (during the lifetime of the QoS Flow) and that the QoS Flow should be kept while the NG-RAN is not fulfilling the requested QoS profile. Notification control may be used for a GBR QoS Flow if the application traffic is able to adapt to the change in the QoS (e.g. if the AF is capable to trigger rate adaptation).

The NG-RAN determination whether the GFBR, the PDB and the PER can be fulfilled or not, is done under consideration of the QoS parameters Averaging Window and MDBV (for a GBR QoS Flow using the Delay-critical resource type), which are either provided explicitly as part of the QoS profile or implicitly via the standardized 5QI QoS characteristics defined in clause 5.7.4.

The SMF shall only enable Notification control when the QoS Notification Control parameter is set in the PCC rule (received from the PCF) that is bound to the QoS Flow. The Notification control parameter is signalled to the NG-RAN as part of the QoS profile.

If the NG-RAN has received the PDU Set QoS parameters PSDB and PSER for UL and/or DL (as described in clause 5.7.7) and PDU Set QoS handling is applied in the respective direction(s), the NG-RAN uses the PSDB and PSER instead of the PDB and PER in the respective direction(s) for the actions described in clauses 5.7.2.4, 5.7.2.4.1, 5.7.2.4.1a, 5.7.2.4.1b, 5.7.2.4.2 and 5.7.2.4.3. If the PDU Set QoS handling, PSER and PSDB are applied in one direction only, the NG-RAN still uses the PDB and PER in another direction together for the actions described in clauses 5.7.2.4, 5.7.2.4.1, 5.7.2.4.1a, 5.7.2.4.1b, 5.7.2.4.2 and 5.7.2.4.3. Notifications sent by the NG-RAN indicate whether PDU Set QoS parameters (PSDB and PSER) are applied by the NG-RAN.

## END of CHANGES