3GPP TSG-WG SA2 Meeting #167	S2-2502535

# Athens, Greece, February 17-21, 2025										   (revision of S2-2502298)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5376 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MPS Resume Cause alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs, CISA ECD, AT&T, Verizon, T-Mobile USA, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI19 |  |  |  |  |  | Date: |  |  | 2025-02-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is an alignment with RAN stage 3 by adding clarification in TS 23.502 that an MPS subscribed UE also obtains priority handling when the RRC connection needs to be resumed in the Connection Resume procedure specified in TS 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | First and only change: Indicate in the Network Triggered Connection Resume in RRC_INACTIVE procedure that for MPS, based on receipt of an ARP or service priority (see clause 5.4.3.3 of TS 23.501), also called RAN Paging Priority in TS 38.413 with values indicative of MPS priority handling, NG-RAN shall handle the request with MPS priority. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Continued misalignment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.2.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 6163 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First and only change * * * *

#### 4.8.2.2a	Network Triggered Connection Resume in RRC_INACTIVE procedure

The Network Triggered Connection Resume in RRC_INACTIVE procedure is used when the NG-RAN needs to send data (e.g. the N1 NAS PDU and/or downlink user plane PDU) to a UE in the RRC_INACTIVE state. During the procedure, the NG-RAN sends a RAN Paging to the UE in order to trigger the Connection Resume in RRC_INACTIVE procedure in clause 4.8.2.2.

Figure 4.8.2.2a-1: Network Triggered Connection Resume for UE in RRC_INACTIVE

1.	The NG-RAN receives downlink data (e.g. the N1 NAS PDU and/or the user plane PDU) for a UE in RRC_INACTIVE State, the NG-RAN buffers the downlink data and triggers RAN Paging message.

If the NG RAN supports the Paging Cause Indication for Voice Service feature and if the UE context in NG-RAN indicates that the UE supports the Paging Cause Indication for Voice Service feature, the NG RAN shall provide the Voice Service Indication in the RAN Paging message for the UE when it detects that the downlink data which triggers the RAN Paging message is related to voice service, as specified in clause 5.38.3 of TS 23.501 [2].

For MPS, based on receipt of an ARP (see clause 5.4.3.3 of TS 23.501 [2]), or based on receipt of service priority (also called RAN Paging Priority in TS 38.413 [10]) with values indicative of MPS priority handling, the NG-RAN shall handle the request with MPS priority.

2.	The NG-RAN sends the RAN Paging message to the UE.

3.	If the UE is in RRC_INACTIVE State, based on the RAN paging and the UE decides to accept the paging, the UE initiates the UE Triggered Connection Resume in RRC_INACTIVE procedure (see clause 4.8.2.2).

# * * * * End of changes * * * *