3GPP TSG-WG SA2 Meeting #167	S2-2502552

Athens, GR, 17th Feb – 21st Feb, 2025	(revision of S2-2501978, 2345)

Source:	Huawei, HiSilicon

Title:	pCR to TS 23.xyz: Updates to Architecture clause

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: Update of clause 4 to include overall architectures, add ADM, and provide descriptions of the NFs along with terminology alignment.

# 1. Introduction/Discussion

An update of clause 4 architecture model and concepts to:

-	Align terminology (e.g., A-IOT RAN to AIoT RAN, Ambient IoT Device to AIoT Device etc.)

-	Add contents to clause 4.2.1, to provide an overview of which architectures are included (and allowing expansion for future architectures).

-	Add complete service-based and reference architectures for AIoT RAN Readers topology, including ADM etc.

-	Identify which parts of the overall topology 1 architecture apply to direct and indirect path.

-	Add ADM based reference points and service-based interfaces.

-	Descriptions for the NFs in the system, based on the discussion in SA2#166ahe and introduce the ADM storage architecture.

-	Align the protocol stacks to use similar terms, such as Nb/N33 stack, and label reference points with consistent naming to the architecture clause.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xyz-101.

# * * * * First change * * * *

# 1	Scope

The present document specifies architectural enhancements to the 5G system to support Ambient power-enabled Internet of Things, complying to the requirements in TS 22.369 [1] applicable to the AIoT Device types, traffic types, use cases and connectivity topologies defined in TS 38.300 [5].

# * * * * Second change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AIoT Device: An Ambient IoT device is an IoT device powered by energy harvesting, with limited energy storage capability.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ADM	AIoT Data Management

AIoT	Ambient IoT

AIOTF	Ambient IoT Function

# 4	Architecture model and concepts

## 4.1	General concept

Ambient IoT is a service that can be provided by the 5GS system to support Ambient power-enabled IoT devices that are powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor) and the energy is provided through the harvesting of radio waves, light, motion, heat, or any other suitable power source.

The 5GS System architecture for Ambient IoT include the following functions and procedures for:

-	AIoT Device identification;

-	AIoT Device inventory;

-	Providing to, and obtaining from, an AIoT Device application data;

-	Disabling AIoT Devices.

## 4.2	Architecture

### 4.2.1	General

The 5GS System architecture for Ambient IoT includes core network functions, AIoT Readers and AIoT Devices. The AIoT architecture supports different AIoT Reader architectures, which allow different deployment options with different AIoT Readers to be used by the AIOTF, for the following cases:

-	AIoT RAN (which supports AIoT Reader), which includes either a direct connectivity path between AIoT RAN and the AIOTF or an indirect connectivity path between AIoT RAN and the AIOTF via an AMF.

The architecture for Network Exposure Function, using reference point representation, defined in TS 23.501 [x] clause 4.2.3 is applicable for Ambient IoT, with a southbound interface from the NEF to AIOTF.

### 4.2.2	Architecture for AIoT RAN connectivity

#### 4.2.2.1	General

Editor’s Note: The term AIoT RAN needs to be aligned with RAN WGs throughout the TS.

Editor’s Note: Whether the connection to the AIOTF should be Nx or N2 is to be determined.

5GS system architecture for Ambient IoT supports one of the following connectivity options to access an AIoT RAN:

-	Direct Connectivity: AIOTF communicates with AIoT RAN directly .

-	Indirect Connectivity via AMF: AIoT RAN and the AIOTF communicate indirectly via an AMF.

Figure 4.2.2.1-1 depicts the complete non-roaming architecture showing the overall 5GS architecture for support of AIoT, including both the Indirect Connectivity and Direct Connectivity .

Figure 4.2.2.1-1: Non-roaming AIoT System Architecture

NOTE x:	For the sake of clarity and to depict the complete reference point architecture, the AMF, Na and N2 as depicted using dashed lines, as all deployments might not use them.

Figure 4.2.2.1-2 depicts the complete non-roaming AIoT system architecture, using the reference point representation.

Figure 4.2.2.1-2: Non-roaming AIoT System Architecture (AIoT RAN Readers) in reference point representation

NOTE 1:	For the sake of clarity of the point-to-point diagrams, the AF and NRF have not been depicted. However, all depicted Network Functions can interact with the UDR, NEF and NRF as necessary.

NOTE 2:	For clarity, the UDR and its connections with other NFs, e.g. ADM, are not depicted in the point-to-point and service-based architecture diagrams. For more information on the ADM data storage architectures refer to clause 4.5.8.

NOTE x:	For the sake of clarity and to depict the complete reference point architecture, the AMF, Na, N2 and Nx as depicted using dashed lines, as all deployments might not use them.

The architectures in the following clauses showing parts of the overall AIoT architecture specific to each connectivity option:

-	Direct Connectivity: the AIOTF uses Nx to access AIoT RAN, and is described in clause 4.2.2.2.

-	Indirect Connectivity via AMF: the AIOTF uses an AMF which uses N2 to access AIoT RAN, and is described in clause 4.2.2.3.

NOTE x: A deployment that only uses, e.g., the Direct Connectivity only does not need to deploy the NFs, reference points and service-based interfaces associated with the Indirect Connectivity, and vice-versa.

#### 4.2.2.2	Direct connectivity between AIOTF and AIOT RAN

In the Direct Connectivity architecture, the AIOTF uses Nx to communicate directly with AIoT RAN.

Figure 4.2.2.2-1 depicts the AIoT architecture, using the service-based interfaces, showing only the parts of the AIoT architecture for an AIOTF connecting to AIoT RAN directly. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-1 remain unchanged.

Figure 4.2.2.2-1: AIoT RAN – AIOT Direct Path Architecture

Figure 4.2.2.2-2 depicts the AIoT architecture, using the reference point representation, showing only the parts of the AIoT architecture for an AIOTF directly connecting to AIoT RAN. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-2 remain unchanged.

Figure 4.2.2.2-2: AIoT RAN – AIOT Direct connectivity Architecture in reference point representation

#### 4.2.2.3	Indirect connectivity between AIOTF and AIOT RAN via an AMF

Figure 4.2.2.3-1 depicts the AIoT architecture, using the service-based interfaces showing only the parts of the AIoT architecture for an AIOTF connects indirectly to AIoT RAN via an AMF. The remaining parts of the AIoT Architecture shown in Figure 4.2.2.1-1 remain unchanged.

Figure 4.2.2.3-1: AIoT RAN – AIOT Indirect connectivity Architecture

Figure 4.2.2.3-2 depicts the AIoT architecture, using the reference point representation showing only the parts of the AIoT architecture for an AIOTF connects to AIoT RAN via an AMF. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-2 remain unchanged.

Figure 4.2.2.3-2: AIoT RAN – AIOT Indirect connectivity Architecture in reference point representation

## 4.3	Reference points

Editor’s Note: The reference point names (e.g. Ny etc.) need to be finalised.

The AIoT Architecture contains the following reference points:

Ny:	Reference point between the AIoT Device and the AIOTF.

Nx:	Reference point between the A-IoT RAN and the AIOTF.

The following reference points show the interactions that exist between the NF services in the NFs. These reference points are realized by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure.

Na:	Reference point between the AIOTF and the AMF.

Nb:	Reference point between the AIOTF and the NEF.

Nc:	Reference point between the AIOTF and the NRF.

Nd:	Reference point between the AIOTF and the ADM.

Ne:	Reference point between the ADM and the UDR.

Nf:	Reference point between the ADM and the NEF.

In addition to the relevant reference points defined in TS 23.501 [3], in the case of Ambient IoT, these reference points are as follows:

N2:	Reference point between the AIoT RAN and the AMF.

N33:	Reference point between NEF and AF.

## 4.4	Service-based interfaces

Naiotf:	Service-based interface exhibited by the AIOTF.

Nadm:	Service-based interface exhibited by the ADM.

In addition to the relevant services defined in TS 23.501 [3] for the following service-based interfaces, in the case of Ambient IoT, the services can be provided by corresponding NF are as follows:

Namf:	Service-based interface exhibited by AMF.

Nnef:	Service-based interface exhibited by NEF.

Naf:	Service-based interface exhibited by AF.

Nnrf:	Service-based interface exhibited by NRF.

# * * * * End of changes * * * *