SA WG2 Meeting #167	S2-2502562

17 - 21 February, 2025, Athens, Greece	(revision of S2-2502347 of S2-2501849)

Source:	LG Electronics

Title:	Resolving EN about assistance information (Clause 5.4)

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: The paper proposes to resolve EN in clause 5.4.

1. Discussion

Regarding the following EN in clause 5.4 of new TS 23.xyz entitled "Architecture support for Ambient power-enabled Internet of Things; Stage 2.", it is considered that 'approximate number of AIoT Devices' and 'approximate D2R message size' are useful for AIoT RAN in performing service operations, e.g. radio resource allocation.

Editor's note:	How the AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) without input from the AF is FFS.

Therefore, it is proposed that the AIOTF determines these information based on configuration if not provided by the AF.

2. Proposal

It is proposed to capture the following changes into new TS 23.xyz entitled "Architecture support for Ambient power-enabled Internet of Things; Stage 2."

* * * * Start of Changes * * * *

5.4	Assistance information provided to AIoT RAN node

The AIOTF provides the assistance information to the AIoT RAN together with the service operation requests. The Assistance information is used by the AIoT RAN for performing service operations, e.g. radio resource allocation.

The assistance information includes:

a)	AIoT service type (e.g. Inventory, Command);

b)	Optionally, approximate number of AIoT devices;

c)	Optionally, approximate D2R message size.

Editor's note:	Other assistance information may be added later if necessary.

For bullets b) and c) in the above assistance information provided by the AIOTF are based on the information received from the AF, or (if not provided by the AF) local configuration based on SLA between the AIoT service provider represented by an AF and the operator.

* * * * End of Changes * * * *