3GPP TSG-WG SA2 Meeting #167	S2-2502575

# Athens, Greece, 17th – 21st Feb, 2025		(revision of S2-2502569)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6010 | rev | 5 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the EventExposure service operation of SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, ETRI, Vivo, CMCC, ZTE, Samsung, NEC, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The basic conception for energy collection, calculation and exposure was introduced by SA2 in 166 meeting. This change request proposes to further update the document regarding the following aspects: Correction on the table for Information from SMF for user-plane energy consumption calculation. The UL/DL Data Volume of UPF is for the requested granularities, instead of UPF node level Data Volume, and it is the gNB-ID serving the UE that is reported by the SMF. Add a new table for user-plane energy consumption calculation information retrieval. Other editorial issue (re-arrange the structure for SMF collection and OAM collection). |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. correction on the table for SMF provided information; 2. other clarifications. 3. Add a new table for user-plane energy consumption calculation information retrieval. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.1, 5.51.2.2, 5.51.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.51.2	Energy Consumption Information collection, calculation and exposure

5.51.2.1	General

The Energy Information Function (EIF) is defined to collect the UE related Energy Consumption information, calculate the Energy Consumption information at UE, PDU Session and/or QoS flow granularity and expose the Energy Consumption information to the authorized consumer NF(s) (AF/NEF or 5GC NF) subject to operator's policy.

Procedures for energy consumption information collection and exposure are defined in Clause 4.29 of TS 23.502 [3].

# * * * * Second change * * * *

5.51.2.2	Energy Consumption information collection

5.51.2.2.1	General

The Energy Information Function (EIF) collects the UE related Energy Consumption information include Node-level energy consumption information, Node-level data volume from OAM and data volume of the required granularities (i.e., PDU Session and/or QoS flow) from UPF (via SMF).

Node level energy consumption is collected PLMN-wide at a configurable starting time with interval T such that the Energy Consumption information reported to the EIF is time-aligned.

NOTE:	If the reported Energy Consumption information is not time-aligned, it cannot be aggregated.

5.51.2.2.2	Energy Consumption information collection from SMF

The serving SMFs are retrieved by the EIF from the UDM of the UE based on the provided input parameters including the UE ID and (S-NSSAI, DNN).

The EIF invokes Nsmf_EventExposure_Subscribe as defined in TS 23.502 [3] with the required granularities (UE ID, DNN/S-NSSAI, application information (e.g., Application Identifier, or Packet Filters)) to retrieve the information from SMF, which is shown in Table 5.51.2.2.2-1. The SMF receives the data volume of the required granularities from the PSA UPF. The SMF then sends the collected data volume, along with the serving gNB ID(s) and (I-)UPF ID(s) to EIF for energy consumption calculation. And the information collected from SMF by EIF, is shown in Table 5.51.2.2.2-2.

Table 5.51.2.2.2-1: Information to SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE ID | SUPI. |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| IP 5-Tuple | IP-5-tuple. |

NOTE 1:	The user-plane energy consumption information reporting interval from the SMFs is the PLMN-wide configurable starting time and interval T.

Table 5.51.2.2.2-2: Information from SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE IP address | UE IP address. |
| UE ID | SUPI |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session |
| IP 5-Tuple | IP-5-tuple |
| List of Data Volume information | The data volume and the associated UPF(s) and gNB(s) serving the UE within the time interval. |
| > UL/DL Data Volume | The UL/DL Data Volume of a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a QoS flow (UE ID, S-NSSAI, DNN, IP 5-Tuple). |
| > (I-)UPF ID(s) | Identifier of any (I-)UPF(s) associated to a reported data volume used by a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a QoS flow (UE ID, S-NSSAI, DNN, IP 5-Tuple). |
| > gNB ID | Identifier of the gNB serving the UE. |
| Reference to Time Interval | Indicate the time interval of the collected information (e.g., time stamps). |

NOTE 2:	Each entry of List of Data Volume information represents the serving gNB and UPF(s) corresponding to the same Data Volume regarding the required granularities. A new entry is added when the gNB or UPF(s) is changed.

5.51.2.2.3	Energy Consumption information collection from OAM

Table 5.51.2.2.3-1 provides the list of information received from OAM. EIF requests the energy consumption and data volume information per NF level from OAM by providing the serving gNB ID(s) and (I-)UPF ID(s) received from SMFs.

The node-level reporting interval from the OAM is the PLMN wide configurable starting time and interval T which is the same as data volume reporting interval from SMFs.

Table 5.51.2.2.3-1: Information from OAM for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| gNB energy consumption | The Energy consumed by a gNB over the configured time interval T based on clause 6.7.3.4.2 of TS 28.554 [209]. |
| gNB data volume | The UL/DL data volume handled by a gNB over the configured time interval T based on clause 6.7.1.1 of TS 28.554 [209]. |
| UPF energy consumption | The Energy consumed by a UPF over the configure time interval T based on clause 6.7.3.1 of TS 28.554 [209]. |
| UPF data volume | Data volume consumed at a UPF. |

When the serving gNB and/or the (I-)UPF(s) of the UE are changed, the serving gNB ID and UPF ID will be updated to the EIF through SMF(s).

NOTE 1: The Node-level Energy Consumption information received from OAM could be used by EIF for all the UEs serving by the NF Node.

NOTE 2: In this Release, only the energy related information of user plane communication is supported, and that of control plane signalling is not supported.

# * * * * Third change * * * *

#### 5.51.2.3	Energy Consumption information calculation

EIF calculates the Energy Consumption information of the required granularities (UE, PDU Session and/or QoS flow), based on input parameters from Tables 5.51.2.2.2-2 and 5.51.2.2.3-1 and gets the results. Some example formulas to support the above calculation are described in Annex T.

# * * * * End of changes * * * *