3GPP TSG-WG SA2 Meeting #167	S2-2502656

# 17 – 21, February, 2025, Athens, Greece                                  (revision of S2-2502653)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5305 | rev | 4 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for AF requested slice selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Ericsson, NEC, ZTE, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_SliceSel |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Also the exact parameters for service operations are defined by replacing the “AF requested slice replacement” to “Replaced S-NSSAI and Alternative S-NSSAI” and when AF does not have the information about Replaced S-NSSAI and Alternative S-NSSAI then it need to provide “Replaced-AF-Service-Identifier and Alternative-AF-Service-Identifier” In R02: In all the service operation, it is missing whether the AF requests the slice replacement or terminates the slice replacement. Hence it is prosed to update the slice replacement as Enable/Disable |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify in the service operations from “AF requested slice replacement” to “Replaced S-NSSAI and Alternative S-NSSAI” and when AF does not have the information about Replaced S-NSSAI and Alternative S-NSSAI then it need to provide “Replaced AF-Service-Identifier and Alternative-AF-Service-Identifier” - Slice Replacmenet as Enable or Disable provided by AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The parameter is unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.9.2, 5.2.5.8.2, 5.2.5.8.3, 5.2.6.22.2, 5.2.6.22.3, 5.2.6.23.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

##### 4.15.6.9.2	Processing AF requests to influence access and mobility management policies targeting an individual UE

This procedure is used for individual UEs when the request shall be applied independently of conditions related to the application traffic. Depending on the AF deployment (see clause 6.2.10 of TS 23.501 [2]), the AF may interact with NFs of the Core Network either directly or via the NEF. The procedure for the direct case is described in Figure 4.15.6.9.2-1, while the procedure for the NEF-mediated case is described in Figure 4.15.6.9.2-2.

Figure 4.15.6.9.2-1: Handling an AF request targeting an individual UE without using NEF

This procedure concerns only non-roaming scenarios, i.e. to cases where the involved entities serving the UE (AF, PCF, BSF, AMF) belong to the home PLMN.

1.	An AM Policy Association is established for a UE as described in clause 4.16.1.

2a.	The AF searches the PCF for the UE using Nbsf_Management_Subscribe with SUPI or GPSI as input, indicating that it is searching for the PCF that handles the AM Policy Association of the UE.

2b.	The BSF provides to the AF the identity of the PCF for the UE for the requested SUPI or GPSI via an Nbsf_Management_Notify operation. If a matching entry already exists in the BSF when step 2a is performed, this shall be immediately reported to the AF.

2c.	The AF sends to the PCF for the UE its request for influencing the access and mobility management policy of the UE (identified by SUPI or GPSI) using Npcf_AMPolicyAuthorization (optionally providing a timer on how long this policy shall last, in which case the system behaviour upon expiration of this timer is as specified in TS 23.503 [20]). This may include information for AF-requested network slice replacement i.e. an Alternative S-NSSAI for a Replaced S-NSSAI (see clauses 5.15.5.2.2a of TS 23.501 [2]), As part of the Npcf_AMPolicyAuthorization request, the AF may subscribe (within the Create and Update operations) or unsubscribe (within the Delete operation) to relevant events specified in clause 6.1.3.18 of TS 23.503 [20], e.g. events related to change of service area coverage.

3.	The PCF takes a policy decision and then an AM Policy Association Modification procedure initiated by the PCF for the UE may be performed as described in clause 4.16.2.2. If the AF has subscribed to access and mobility management related events (i.e. request for service area coverage outcome) in step 2, the PCF reports the event (i.e. outcome of the request for service area coverage) to the AF.

Figure 4.15.6.9.2-2: Handling an AF request targeting an individual UE using NEF

This procedure concerns only non-roaming scenarios, i.e. to cases where the involved entities serving the UE (AF, NEF, PCF, BSF, AMF) belong to the home PLMN, or the AF belongs to a third party with which the home PLMN has an agreement.

1.	An AM Policy Association is established for a UE as described in clause 4.16.1.

2a.	The AF sends to NEF its request for influencing the access and mobility management policy of the UE (identified by GPSI) using Nnef_AMPolicyAuthorization (optionally providing a timer on how long this policy shall last, in which case the system behaviour upon expiration of this timer is as specified in TS 23.503 [20]). As part of the Nnef_AMPolicyAuthorization request, the AF may request to subscribe (within the Create and Update operations) or unsubscribe (within the Delete operation) for relevant events specified in clause 6.1.3.18 of TS 23.503 [20], e.g. events for request for service area coverage outcome. The NEF stores the request and sends a response to the AF. In this message, the AF may request the network slice replacement by providing a slice replacement requirement defined by a pair (Replaced AF-Service-Identifier, Alternative AF-Service-Identifier) to the NEF and the NEF may map the AF-Service-Identifiers to Replaced S-NSSAI and Alternative S-NSSAI (see clauses 5.15.5.2.2a of TS 23.501 [2]).

2b.	The NEF searches the PCF for the UE using Nbsf_Management_Subscribe with SUPI as input parameter, indicating that it is searching for the PCF that handles the AM Policy Association of the UE.

2c.	The BSF provides to the NEF the identity of the PCF for the UE for the requested SUPI via an Nbsf_Management_Notify operation. If a matching entry already exists in the BSF when step 2b is performed, this shall be immediately reported to the NEF.

2d.	The NEF sends to PCF for the UE the request for influencing the access and mobility management policy of the UE (identified by SUPI) using Npcf_AMPolicyAuthorization (having potentially translated GPSI to SUPI via UDM). This may include information for AF-requested network slice replacement i.e. an Alternative S-NSSAI for a Replaced S-NSSAI (see clauses 5.15.5.2.2a of TS 23.501 [2]),As part of the Npcf_AMPolicyAuthorization request, the NEF may subscribe or unsubscribe (according to what the AF requested in step 2a) for relevant events specified in clause 6.1.3.18 of TS 23.503 [20], e.g. events for change of service area coverage.

2e.	The NEF informs the AF about events to which the AF has potentially subscribed (i.e. events for change of service area coverage) using Nnef_AMPolicyAuthorization_Notify.

3.	The PCF takes a policy decision and then an AM Policy Association Modification procedure initiated by the PCF for the UE may be performed as described in clause 4.16.2.2. If the AF has subscribed to access and mobility management related events i.e. request for service area coverage outcome in step 2, then the PCF reports the event (i.e. outcome of the request for service area coverage) to the AF as described in clause 4.16.2.2.

* * * Next Change * * * *

##### 5.2.5.8.2	Npcf_AMPolicyAuthorization_Create service operation

Service operation name: Npcf_AMPolicyAuthorization_Create

Description: Authorizes the request and optionally determines and installs AM influence data according to the information provided by the NF Consumer.

Inputs, Required: SUPI.

Inputs, Optional: GPSI, Throughput requirements, service coverage requirements, AF requested network slice replacement(i.e. pair (Replaced S-NSSAI, Alternative S-NSSAI)), policy duration, subscribed events(s), 5G access stratum time distribution indication (enable, disable), Uu time synchronization error budget, clock quality detail level, clock quality acceptance criteria.

The subscribed event includes Event ID as specified in Npcf_AMPolicyAuthorization_Notify service operation, Event Reporting Information defined in Table 4.15.1-1 (only the Event Reporting mode and the immediate reporting flag when applicable), Notification Target Address.

Outputs, Required: Success or Failure.

Outputs, Optional: Identification of the created application context, the inputs that can be accepted by the PCF.

* * * Next Change * * * *

##### 5.2.5.8.3	Npcf_AMPolicyAuthorization_Update service operation

Service operation name: Npcf_AMPolicyAuthorization_Update

Description: Provides updated information to the PCF.

Inputs, Required: Identification of the application context.

Inputs, Optional: Throughput requirements, service coverage requirements, AF requested network slice replacement(i.e. pair (Replaced S-NSSAI, Alternative S-NSSAI)), AF requested network slice replacement termination indication, policy duration, 5G access stratum time distribution indication (enable, disable), Uu time synchronization error budget, clock quality detail level, clock quality acceptance criteria.

Outputs, Required: Success or Failure.

Outputs, Optional: The inputs that can be accepted by the PCF.

* * * Next Change * * * *

##### 5.2.6.22.2	Nnef_AMPolicyAuthorization_Create service operation

Service operation name: Nnef_AMPolicyAuthorization_Create

Description: Authorizes the request and triggers an Npcf_AMPolicyAuthorization_Create, potentially translating GPSI to SUPI.

Inputs, Required: GPSI.

Inputs, Optional: Throughput requirements, service coverage requirements, AF requested slice replacement requirement (i.e. pair (Replaced S-NSSAI, Alternative S-NSSAI), or pair (Replaced-AF-Service-Identifier, Alternative-AF-Service-Identifer) in the case when AF does not have information about the Replaced S-NSSAI and Alternative S-NSSAI), policy duration, subscribed event(s).

The subscribed event includes Event ID as specified in Nnef_AMPolicyAuthorization_Notify service operation, Event Reporting Information defined in Table 4.15.1-1 (only the Event Reporting mode and the immediate reporting flag when applicable), Notification Target Address.

Outputs, Required: Success or Failure.

Outputs, Optional: Identification of the created application context, the inputs that can be accepted by the PCF.

* * * Next Change * * * *

##### 5.2.6.22.3	Nnef_AMPolicyAuthorization_Update service operation

Service operation name: Nnef_AMPolicyAuthorization_Update

Description: Provides updated information about an application context to the NEF and triggers an Npcf_AMPolicyAuthorization_Update.

Inputs, Required: Identification of the application context.

Inputs, Optional: Throughput requirements, service coverage requirements, AF requested slice replacement requirement (i.e. pair (Replaced S-NSSAI, Alternative S-NSSAI), or pair (Replaced-AF-Service-Identifier, Alternative-AF-Service-Identifer) in the case when AF does not have information about the Replaced S-NSSAI and Alternative S-NSSAI), AF requested network slice replacement termination indication, policy duration.

Outputs, Required: Success or Failure.

Outputs, Optional: The inputs that can be accepted by the PCF.

* * * Next Change * * * *

##### 5.2.6.23.2	Nnef_AMInfluence_Create operation

Service operation name: Nnef_AMInfluence_Create

Description: Authorize the request and store the AM influence data in the UDR, potentially translating GSPI to SUPI and External Group Identifier to Internal Group Identifier.

Inputs, Required: AF Transaction Id.

The AF Transaction Id refers to the request.

Inputs, Optional: List of (DNN, S-NSSAI)(s), Target (GPSI, or External Group Identifier(s), or any UE, or any inbound roaming UEs identified by their PLMN ID(s)), throughput requirements, service coverage requirements, AF requested network slice replacement, policy duration, List of External Application Identifiers, subscribed event(s).

The subscribed event(s) includes Event ID(s) as specified in Nnef_AMInfluence_Notify service operation, Event Reporting Information defined in Table 4.15.1-1 (only the Event Reporting mode and the immediate reporting flag when applicable), Notification Target Address.

When a list of External Application Identifiers is provided, the throughput requirement, service coverage requirements, AF requested network slice replacement (i.e. pair (Replaced S-NSSAI, Alternative S-NSSAI), or pair (Replaced-AF-Service-Identifier, Alternative-AF-Service-Identifer) in the case when AF does not have information about the Replaced S-NSSAI and Alternative S-NSSAI), policy duration and subscribed event(s) are assigned the same value for all External Application Identifiers.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

NOTE:	"any UE" refers to the UEs within the PLMN of the NEF.

* * * End of Changes * * * *