3GPP TSG-WG SA2 Meeting #167	S2-2502786

# 17 - 21 February, 2025, Athens, Greece		(revision of S2-2502353)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6124 | rev | 2 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the UPF Event Exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - As specified in clause 5.2.1.3.3 and 5.2.1.3.4 of TS 29.564, the consumer will subscribe to UPF directly (instead of indirectly via SMF) for "any UE" or a specific PDU session when the subscription does not include any of the following parameters: AoI, BSSID/SSID and DNAI. However, in the current description in clause 5.18.2.17, “may” is used, which is not aligned with CT4 specification. Hence it is suggested to change this description to align with CT4 specification and also a NOTE is added to clarify this. - For the QoS Monitoring, it can be activated due to PCC rule or due to SMF event exposure. For the activation due to PCC rule, the report should be sent to AF immediately as it may be used for application layer adjustment. Hence it is clarified that the delayed reporting is only sent to UPF if it is received from the subscribed UPF event. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change “may” to “should”. - Clarify that reporting suggestion information is only applicable for SMF event exposure activation case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Inconsistent specification. - The AF adjustment of application based on QoS Monitoring reports is impossible when reporting suggestion information is set. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.17, 5.8.2.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.8.2.17	Data exposure via Service Based interface

The UPF may expose information by means of UPF Event Exposure service as described in TS 23.502 [3] clause 5.2.26.2, via a service-based interface directly. The NF consumers, which may receive UPF event notifications, are AF/NEF, TSNAF/TSCTSF, SMF and NWDAF/DCCF/MFAF.

When the UPF supports the data exposure via the service based interface, it may register its NF profile to the NRF including the UPF Event Exposure services and the related Event ID(s).

For data collection from UPF (see clause 4.15.4.5 of TS 23.502 [3]), NF consumers can do the subscription to the UPF either directly or indirectly via SMF. As described in clause 4.15.4.5.1 of TS 23.502 [3], for User Data Usage events (i.e. User Data Usage Measures and User Data Usage Trends), an NF consumer should subscribe to the UPF Event Exposure service directly if target of data collection is "any UE" or if target of data collection is a UE IP address and if the subscription is not including any of the following parameters: AoI, BSSID/SSID and DNAI. Otherwise, the NF consumer shall subscribe indirectly via SMF.

NOTE 1:  	Direct subscription to UPF is more efficient, therefore it is recommended that NF consumer subscribes directly to UPF whenever possible.

NOTE 2:	The UE IP address is used to discover the PSA UPF serving the particular UE IP address. The NF consumer can only use the UE IP address to discover PSA UPF(s) that serve the corresponding IPv4 address/IPv6 prefix range and register the corresponding information in the NRF.

Upon release of an N4 Session the UPF implicitly terminates subscriptions for a UE related to the N4 Session. When the N4 Session to the UPF is released, the UPF may send a notification with a subscription termination indication and an optional cause code to the UPF event consumer. the UPF may report to the consumer any data that has been collected but not sent to the consumer yet when the N4 session to the UPF is released (see clause 4.15.4.5 of TS 23.502 [3]).

To alleviate the load of UPF due to frequent event notification for data collection related events, the event subscription may include Reporting suggestion information. The Reporting suggestion information includes Report urgency and Reporting window information. Reporting urgency information represents whether this event report can be delay tolerant, i.e. the event report can be delayed. If the Reporting urgency information indicates "delay tolerant", the Reporting window is also provided, which defines the last valid reporting time and UPF shall report the event before that time. If the Reporting suggestion information allows this, the UPF can concatenate several event reports (of the same event) to the same notification endpoint into one notification message.

The UPF may also expose UE information by means of the Nupf_GetUEPrivateIPaddrAndIdentifiers service as described in clause 5.2.26.3 of TS 23.502 [3]. An UPF which is deployed with NAPT (Network Address Port Translation) functionality may support to provide the 5GC UE IP address to NEF based on NEF request containing public IP address and port number using the Nupf_GetUEPrivateIPaddrAndIdentifiers service as described in clause 4.15.10 of TS 23.502 [3] for AF specific UE ID retrieval.

The UPF event exposure service may allow NF consumer to obtain one NATed UE public IP address and Port number for a particular PDU Session, based on the UE private IP address allocated by 5GC. An UPF which is deployed with NAT functionality may support to provide the UE public IP address and port number to consumer based on Nupf_EventExposure_Subscribe request as defined in clause 5.2.26.2.3 of TS 23.502 [3]. For information flow for getting UE public IP address and port number, see clause 6.2.8.2.4 of TS 23.288 [5].

# * * * * Next change * * * *

5.8.2.18	QoS Flow related QoS monitoring and reporting

The SMF may configure the UPF to perform QoS monitoring for a QoS Flow and to report the monitoring results with the help of the following parameters provided in the Session Reporting Rule (SRR) described in clause 5.8.5.11:

-	QoS monitoring parameter(s) indicating the subject of the QoS monitoring as defined in clause 5.45;

-	Reporting period indicating the time interval in which a new measurement result and a potential report has to be available. Generally, if no measurement result is available to the UPF within the Reporting period, the UPF shall report a measurement failure; however, for some QoS monitoring parameters (e.g. congestion information, PDV and data rate), the measurement failure report is not applicable.

-	Reporting frequency indicating the type of the reporting as "periodic" or "event triggered":

-	If the Reporting frequency indicates "periodic", the UPF shall send a report each time the reporting period is over.

-	If the Reporting frequency indicates "event triggered", a Reporting threshold for each parameter in the QoS monitoring parameter(s) and a Minimum waiting time are provided as well. The UPF shall send a report when the measurement result matches or exceeds the indicated Reporting threshold. Subsequent reports should not be sent by the UPF during the Minimum waiting time. The UPF shall continue to report a measurement result that matches or exceeds the indicated Reporting Threshold when the Minimum waiting time is over.

NOTE:	As an implementation option, the UPF can be configured to send subsequent report(s) during the Minimum waiting time, e.g. if the UPF determines that this report is considerably different from the previous report.

-	(Optional) Target of the reporting and Indication of direct event notification indicating that the UPF shall send the reports to a different NF than the SMF (e.g. to the NEF/AF or the NWDAF/DCCF/MFAF). The NF is identified by a Notification Target Address and a Notification Correlation ID. The SMF can also indicate that the UPF shall send the reports to both, the NF indicated by the Target of reporting and to the SMF. If so, the UPF shall send the reports to the SMF as well. If the Indication of direct event notification is not provided, the UPF shall send the reports to the SMF.

-	(Optional) Reporting suggestion information as defined in clause 5.8.2.17 applicable to Target of the reporting to reduce the UPF performance impacts. The SMF includes Reporting Suggestion information if received in a subscription to UPF QoS Monitoring event sent by the consumer via SMF for UPF Data Collection (see clause 4.15.4.5 of TS23.502[3]).

-	(Optional) Indication of QoS Flow associated with the default QoS Rule (see clause 4.15.4.5.1 of TS 23.502 [3]). The UPF shall forward this indication, that the QoS monitoring report is for the QoS Flow associated with the default QoS Rule, in the Nupf_EventExposure_Notify service operation when sending reports.

The UPF shall send the QoS Monitoring Report as follows:

-	when the UPF sends reports to the SMF, the UPF shall use QoS Monitoring Reports as described in clause 5.8.5.12; and/or

-	When the UPF sends reports to a different NF than the SMF (e.g. the NEF/AF or the NWDAF/DCCF/MFAF), the UPF shall use the Nupf_EventExposure_Notify service operation described in clause 5.2.26.2.2 of TS 23.502 [3].

# * * * * End of changes * * * *