SA WG2 Meeting ##168	S2-2502807

07 - 11 April, 2025, Goteborg, Sweden

3GPP TSG-CT WG1 Meeting #153	C1-250885

# Athens, Greece, 17-21 February 2025

# Title:	LS on multiplexing capability negotiation

# Response to:	-

# Release:	Rel-19

# Work Item:	NG_RTC_Ph2

Source:	CT1

To:	SA2

Cc:

Contact Person:

Name:	Xu Chen

Tel. Number:

E-mail Address:	chenxu@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

CT1 is now working on support of IMS DC multiplexing feature in Rel-19 NG_RTC_Ph2 and notices the following requirements of P-CSCF and S-CSCF are specified in TS 23.228 AC.7.10.2.1:

AC.7.10.2.1	Capability negotiation between UE and IMS network

The UE and IMS network may support DC multiplexing capability negotiation as follows:

........

-	If the P-CSCF supports data channel multiplexing, when the P-CSCF receives the REGISTER request from the UE, the P-CSCF shall include a Feature-Caps header field indicating its capability of supporting data channel multiplexing in the initial and any subsequent REGISTER request.

NOTE:	The P-CSCF and IMS-AGW need to understand the multiplexed SDP to avoid gating the multiplexed media streams.

-	If the home IMS network supports this feature, the S-CSCF includes a Feature-Caps header field indicating its capability of supporting data channel multiplexing in the 200 OK response to the initial and any subsequent REGISTER request.

-	If the IMS network supports this feature, the S-CSCF includes a Feature-Caps header field indicating its capability of supporting data channel multiplexing in the SIP request messages to the remote network.

CT1 would like to ask SA2 the following questions:

On the requirement of the P-CSCF:

Q1: During the capability negotiation between UE and IMS network, what is the consideration to require the P-CSCF to indicate its capability to the home IMS network of the UE?

If the P-CSCF and IMS-AGW do not support data channel multiplexing and the MF in the home IMS network of the UE supports data channel multiplexing:

Q2A: what the S-CSCF will indicate in the Feature-Caps header field included in 200 OK response to REGISTER request?

Q2B: what the P-CSCF will do if the Feature-Caps header field included in 200 OK response to REGISTER request indicates support of data channel multiplexing and the P-CSCF and IMS-AGW do not support data channel multiplexing?

On the requirement of the S-CSCF:

Q3: What is the consideration to require the S-CSCF to indicate its capability to the remote IMS network？

Q4: If the P-CSCF and IMS-AGW do not support data channel multiplexing and the MF in the home IMS network of the UE supports data channel multiplexing what the S-CSCF will indicate in the Feature-Caps header field of the INVITE request to the remote network?

Q5: Can the multiplexed data channel media descriptions within an SDP offer included in the SIP request messages implicitly point out this capability of the IMS network?

2. Actions:

To SA2

ACTION: 	CT1 kindly asks SA2 to provide answers to the above questions.

3. Date of Next CT1 Meetings:

CT1#154                           7th April- 11th April 2025...............................China

CT1#155                           19th May- 23rd  May 2025.............................Bratislava, Slovakia

CT1#156                           25th August- 29th August 2025.....................Goteborg, Sweden

CT1#157                           13th October- 17th October 2025..................Sophia Antipolis, France

CT1#158                           17th November- 21st November 2025............Dallas, US

CT1#159                           9th February- 13th February 2025..................India