SA WG2 Meeting ##168	S2-2502812

07 - 11 April, 2025, Goteborg, Sweden

3GPP TSG-CT WG4 Meeting #127	C4-250555

# Athens, GREECE; 17th – 21st February 2025

# Title:	LS on Security related protocol-specific parameters for N6 delay measurement

# Release:	Rel-19

# Work Item:	eEDGE_5GC_Ph3

Source:	CT4

To:	SA3

Cc:	CT3, SA2

Contact Person:

Name:	Bruno Landais

Tel. Number:

E-mail Address:	bruno.landais@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

CT4 is designing protocol extensions to support the Rel-19 stage 2 requirements on N6 delay measurement and reporting.

Clause 5.8.2.23 (N6 Delay Measurement and Reporting) of TS 23.501 specifies:

AF may trigger the N6 delay measurement by providing Indication of considering N6 delay measurement through AF influence as described in clause 5.6.7. The SMF requests the candidate UPF(s) (via N4 association procedures described in clause 4.4.3 of TS 23.502 [3]) to perform the N6 delay measurement between the UPF and measurement endpoint and to report the N6 delay measurement to SMF. The request includes the following information: measurement endpoint address (e.g., EAS IP address and optionally port number), measurement protocol (e.g. TWAMP (IETF RFC 5357 [211]), OWAMP (IETF RFC 4656 [212]), STAMP (IETF RFC 8762 [213]), or other protocols), protocol-specific configuration parameters. The UPF leverages the measurement protocol to measure the N6 delay. Then, the candidate UPF(s) report the N6 delay measurement and the corresponding endpoint address to the SMF (via the N4 association report procedure described in clause 4.4.3 of TS 23.502 [3]. N6 delay measurement reporting is triggered based on the reporting frequency and/or reporting threshold provided as part of the configuration.

Clause 6.2.3.4 (EAS Deployment Information Management) of TS 23.548 specifies:

Table 6.2.3.4-1 Description of EAS Deployment Information

| N6 delay measurement assistance information | Includes the following parameters per DNAI and/or per EAS to enable N6 delay monitoring: - measurement endpoint address information (consisting of IP address and optionally port number). - measurement protocol(s) supported by the application measurement endpoint, - protocol-specific configuration parameters. [optional]. See NOTE 4, NOTE 5, NOTE 6. |
| --- | --- |

CT4 assumes that the protocol-specific configuration parameters may include security information. See also the following note from SA2 TR 23.700-49:

NOTE 2:	Protocol-specific configuration parameters may include port number of the remote N6 entity for the measurement protocol, measurement role such as session-sender for STAMP, operation mode, security information, etc.

CT4 would like SA3 to provide guidance on the security related protocol-specific parameters that PFCP should support providing to the UPF over N4, for the indicated measurement protocols (i.e. TWAMP, OWAMP and STAMP).

2. Actions:

To SA3 group.

ACTION: 	CT4 kindly asks SA3 group to provide guidance on the security related protocol-specific parameters that PFCP should support providing to the UPF over N4, for the indicated measurement protocols (i.e. TWAMP, OWAMP and STAMP).

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm