SA WG2 Meeting ##168	S2-2502815

07 - 11 April, 2025, Goteborg, Sweden

3GPP TSG-CT WG4 Meeting #127	C4-250629

# Athens, GREECE; 17th – 21st February 2025

# Title:	LS on Transport Level Marking on N3/N9

# Release:	Rel-19

# Work Item:	XRM_Ph2

Source:	CT4

To:	SA2

Contact Person:

Name:	Frank Yong Yang

Tel. Number:

E-mail Address:	frank.yong.yang@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

CT4 discussed the following SA2 requirements in clause 5.8.2.7 of 3GPP TS 23.501 for the Transport Level Marking on N3/N9.

5.8.2.7    PDU Session and QoS Flow Policing

ARP is used for admission control (i.e. retention and pre-emption of the new QoS Flow). The value of ARP is not required to be provided to the UPF.

For every QoS Flow, the SMF shall determine the transport level packet marking value (e.g. the DSCP in the outer IP header) based on the 5QI, the Priority Level (if explicitly signalled) and optionally, the ARP priority level and provide the transport level packet marking value to the UPF.

For QoS Flows that are configured for PDU Set QoS handling, the SMF may additionally take into account the PDU Set Importance when determining the transport level packet marking values. In this case, the SMF provides a list of transport level packet marking values for the downlink direction to the UPF in a FAR, each of the transport level packet marking values corresponding to one or more PDU Set Importance values. When an I-SMF/I-UPF is inserted in the PDU Session, the I-SMF may instruct the I-UPF to derive the transport level packet marking of the outgoing N3 downlink packet based on the transport level packet marking of the incoming N9 downlink packet.

CT4 would like to ask if the highlighted requirement in yellow implies a new requirement for N4 interface specific for XRM_Ph2?

If so, how can the I-SMF learn from the anchor SMF to instruct the I-UPF for a QoS flow to derive the transport level packet marking of the outgoing N3 downlink packet based on the transport level packet marking of the incoming N9 downlink packet?

More generally, could SA2 clarify how transport level marking should be performed by I-UPF?

2. Actions:

To SA2 group.

ACTION: 	CT4 kindly request SA2 to answer the questions and clarify the matter in their specifications if necessary.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm