SA WG2 Meeting ##168	S2-2502832

07 - 11 April, 2025, Goteborg, Sweden

3GPP TSG-RAN WG3 Meeting #127						R3-250905

Athens, Greece, 17-21 Feb, 2025

# Title:	LS on Ambient IoT progress of RAN3

# Response to:

# Release:	Release 19

# Work Item:	Ambient_IoT_solutions-Core

Source:	RAN3

To:	SA2, SA5

Cc:

Contact Person:

Name:	Yan Wang

E-mail Address:	wangyan7 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	N/A

1. Overall Description:

RAN3 would like to inform SA2 and SA5 about the following RAN3 progress related to Architecture and OAM aspects:

- A-IoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

- Support both the direct and indirect communication options between the gNB and the AIOTF in Topology 1.

- Working assumption: Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP.

- It is FFS whether to define a new A-IoT Area, or use Tracking Area dedicated for A-IoT, to support e.g., the selection of gNBs by the AIOTF.

- AIOTF obtains the A-IoT RAN information (supported Area, served reader ID list) via OAM configuration, FFS on RAN3 signalling for the A-IoT RAN information.

- AIOTF may also be aware of the other A-IoT RAN information (location of reader) via OAM configuration, FFS on RAN3 signalling for the other A-IoT RAN information...

- In Inventory Request, gNB receives the requested Service Area Information (encoded as Area and/or reader ID list) from AIOTF.

- The gNB always sends the A-IoT device’s location at reader ID granularity to the AIOTF in Inventory Report.

- “Global gNB ID + Reader index" is used to uniquely identify the Reader globally.

2. Actions:

To SA2 and SA5

ACTION: 	RAN3 asks SA2 and SA5 to take above into account. RAN3 will further discuss the FFS aspects in the following meetings and kindly ask SA2 and SA5 to provide feedback, if any.

3. Date of Next TSG RAN WG3 Meetings:

RAN3#127-bis	2025-04-07 - 2025-04-11		Wuhan, CN

RAN3#128	2025-05-19 - 2025-05-23		Malta, MT