SA WG2 Meeting ##168	S2-2502840

07 - 11 April, 2025, Goteborg, Sweden

3GPP TSG-SA5 Meeting #159	S5-250826

Sophia-Antipolis, France, 17 - 21 February 2025

Title:	Reply LS on ML Model provisioning by OAM to LMF for UE positioning

Response to:	LS (S2-2412890) on ML Model provisioning by OAM to LMF for UE positioning from SA2

Release:	Rel-19

Work Item:	AIML_MGT_Ph2

Source:	SA5

To:	SA2

Cc:	-

Contact person:	Pengxiang Xie

xie.pengxiang@zte.com.cn

Hassan Al-Kanani

Hassan.alkanani@emea.nec.com

Yizhi Yao

Yizhi.yao@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

SA5 thanks SA2 for the LS on ML Model provisioning by OAM to LMF for UE positioning in document S2-2412890.

SA5 has concluded the following answers for the questions of the LS as below:

SA2’s Question 1: SA2 kindly asks SA5 to clarify whether and how provisioning trained ML model by the OAM to LMF for UE positioning use cases 2b and 3b documented in TR 38.843 is feasible.

SA5’s Answer: SA5 confirms that it is feasible to provision a trained ML model by the OAM to LMF for UE positioning.  This can be achieved by leveraging the ML model loading management capabilities specified in clause 7.3a.3 of TS 28.105 allowing a consumer to request and control the producer-initiated loading of a trained ML model into LMF.

SA2’s Question 2: SA2 kindly asks SA5 to clarify if any functionalities/services need to be supported by SA2 specifications to enables ML model provisioning by the OAM to LMF for UE positioning use cases.

SA5’s Answer: From SA5 perspective, LMF is already modelled within SA5's NRM (Network Resource Model) specifications with a dedicated IOC in TS 28.541. In current TS 28.105, the IOCs representing the functions/functionalities that use the AI/ML inference function include MDAFunction, AnLFFunction, DMROFunction, DLBOFunction, and DESManagementFunction but not LMFFunction. SA5 needs to update TS 28.105 and then allow the LMFFunction to be supported by the AIMLInferenceFunction, so that the existing management framework in TS 28.105 can facilitate the provisioning of ML models to LMF.

To streamline the management of ML models for UE positioning across their entire lifecycle (including training/re-training, testing, emulation, deployment, and inference), both the training data collection and performance monitoring of the ML model for UE positioning and the inference function needs to be supported in OAM. In this regard, SA5 would need SA2’s input on defining the mechanisms to collect relevant data, specifically:

- Performance measurements (as defined in TS 28.552), and/or

- UE-level measurements (referencing TS 32.422 and TS 28.558).

# 2	Actions

To SA2

ACTION: 	SA5 kindly asks SA2 to take the SA5’s reply above into consideration and welcomes further cooperation on the provision of ML Model for UE positioning.

# 3	Dates of next TSG SA WG5 meetings

SA5#160		07 April - 11 April 2025			Gothenburg, Sweden

SA5#161		19 May - 23 May 2025			Fukuoka, Japan