3GPP TSG-SA2 Meeting #168	S2-2502886

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0544 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are multiple instances of inconsistent use of the word “Multi-hop” in the specification e.g. “multihop” or “multi-hop” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align the wording to have a consistent use of the terminology |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent use of the terminology |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.1A, 4.3.3, 4.3.4, 4.3.5, 4.3.8, 4.3.12.2, 5.1.1, 5.1.1a, 5.1.4.1a, 5.1.4.2.1a, 5.1.4.2.2a, 5.1.5.1a, 5.1.5.2.1a, 5.1.5.2.2a, 5.2.6, 5.6.3.1, 5.8.3.1, 5.8.3.2, 6.3.2.5, 6.3.2.5.1, 6.3.2.5.2, 6.3.2.5.4, 6.3.2.6.3.1, 6.4.3.8, 6.4.3.8.1, 6.4.3.9.2, 6.4.3.9.3, 6.4.3.10.2, 6.5.1.4, 6.5.1.5, 6.6.2, 6.7.4.1, 6.7.4.4, 6.7.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

#### 4.2.7.1A	5G ProSe Multi-hop Layer-3 UE-to-Network Relay reference architecture

Figure 4.2.7.1A-1 shows the high level reference architecture for 5G ProSe Multi-hop Layer-3 UE-to-Network Relay. There may be one or multiple 5G ProSe Intermediate UE-to-Network Relays between the 5G Prose Remote UE and the 5G ProSe UE-to-Network Relay. In this figure, the 5G ProSe Layer-3 UE-to-Network Relay may be in the HPLMN or a VPLMN.

Figure 4.2.7.1A-1: Reference architecture for 5G ProSe Multi-hop Layer-3 UE-to-Network Relay

* * * * Next Change * * * *

### 4.3.3	PCF

In addition to the functions defined in TS 23.501 [4] and TS 23.503 [9], the PCF includes functions to provision the UE with necessary policies and parameters to use 5G ProSe services, as part of the UE ProSe Policy information as defined in TS 23.503 [9] clause 4.2.2:

-	PC5 usage reporting configuration.

-	Authorization policy and parameters for 5G ProSe Direct Discovery and Communication.

-	Authorization policy and parameters for 5G ProSe UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-2 Remote UE, as 5G ProSe Layer-3 Remote UE, as 5G ProSe Layer-2 UE-to-Network Relay, as 5G ProSe Layer-3 UE-to-Network Relay).

-	PDUID allocation with its validity timer.

-	Authorization policy and parameters for 5G ProSe UE-to-UE Relay Discovery and Communication.

-	Authorization policy and parameters for 5G ProSe Layer-3 Multi-hop UE-to-Network Relay Discovery and Communication.

-	Authorization policy and parameters for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery and Communication.

The PCF may update the 5G ProSe policy and parameters to the UE under certain conditions.

When receiving the 5G ProSe Capability in Npcf_UEPolicyControl_Create Request from the AMF or when receiving the updated subscription data from UDR, the PCF generates the PC5 QoS parameters used by NG-RAN corresponding to a UE as defined in clause 5.4.2 of TS 23.287 [2].

* * * * Next Change * * * *

### 4.3.4	AMF

In addition to the functions defined in TS 23.501 [4], the AMF performs the following functions:

-	Select a PCF supporting 5G ProSe Policy/Parameter provisioning based on indication of 5G ProSe Capability as part of the "5GMM capability" in the Registration Request.

-	Store the 5G ProSe Capability.

-	Forward the 5G ProSe Capability to PCF in Npcf_UEPolicyControl_Create Request.

-	Obtain from UDM the subscription information related to 5G ProSe and store them as part of the UE context data.

-	Obtain PC5 QoS parameters from the PCF and store them as part of the UE context data.

-	Provision the NG-RAN with indication about the UE authorization status about the following:

-	5G ProSe Direct Discovery and 5G ProSe Direct Communication (i.e. as 5G ProSe-enabled UE for ProSe Direct Discovery, as 5G ProSe-enabled UE for ProSe Direct Communication);

-	5G ProSe UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-2 Remote UE, as 5G ProSe Layer-2 UE-to-Network Relay, as 5G ProSe Layer-3 UE-to-Network Relay);

-	Multi-path communication via direct Uu path and via 5G ProSe Layer 2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE;

-	5G ProSe UE-to-UE Relay Discovery and Communication (i.e. as 5G ProSe Layer-2 End UE, as 5G ProSe Layer-2 UE-to-UE Relay);

-	5G ProSe Layer-3 Multi-hop UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay;

-	5G ProSe Layer-2 Multi-hop UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 Multi-hop UE-to-Network Relay, as 5G ProSe Layer-2 Intermediate UE-to-Network Relay, as 5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 Multi-hop UE-to-Network Relay).

-	Provision the NG-RAN with PC5 QoS parameters related to 5G ProSe Direct Communication.

-	Optionally support security procedures over Control Plane for 5G ProSe UE-to-Network relaying as defined in TS 33.503 [29].

* * * * Next Change * * * *

### 4.3.5	UDM

In addition to the functions defined in TS 23.501 [4], the UDM performs the following functions:

-	Subscription management for 5G ProSe Direct Discovery and Communication.

-	Subscription management for 5G ProSe UE-to-Network Relay Discovery and Communication.

-	Subscription management for multi-path communication via direct Uu path and via 5G ProSe Layer 2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE.

-	Subscription management for 5G ProSe UE-to-UE Relay Discovery and Communication.

-	Subscription management for 5G ProSe Layer-3 Multi-hop UE-to-Network Relay Discovery and Communication.

-	Subscription management for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery and Communication.

* * * * Next Change * * * *

### 4.3.8	ProSe Application Server

The ProSe Application Server supports the following functionalities.

For 5G ProSe Direct Discovery:

-	Maintains permission information for the restricted 5G ProSe Direct Discovery using RPAUIDs;

-	Storage of ProSe Discovery UE IDs and metadata;

-	Mapping of RPAUID and PDUID for restricted 5G ProSe Direct Discovery;

-	Provisioning parameters for Group Member Discovery to UE.

-	Interaction with 5G DDNMF for 5G ProSe Direct Discovery, including:

-	Allocation of the ProSe Restricted Code Suffix pool, if restricted Direct Discovery with application-controlled extension is used;

-	Allocation of the mask(s) for ProSe Restricted Code Suffix, if restricted Direct Discovery with application-controlled extension is used;

-	Allocation of the ProSe Application Code Suffix pool, if open Direct Discovery with application-controlled extension is used;

-	Allocation of the mask(s) for ProSe Application Code Suffix, if open Direct Discovery with application-controlled extension is used.

For 5G ProSe Direct Communication:

-	Provisioning a path preference for 5G ProSe Services to UDR;

-	Provisioning parameters for 5G ProSe Direct Communication to UE.

For 5G ProSe UE-to-Network Relay service:

-	Provisioning parameters for 5G ProSe UE-to-Network Relay Discovery and 5G ProSe UE-to-Network Relay Communication to UDR.

For 5G ProSe UE-to-UE Relay service:

-	Provisioning parameters for 5G ProSe UE-to-UE Relay Discovery and 5G ProSe UE-to-UE Relay Communication to UDR.

For 5G ProSe Layer-3 Multi-hop UE-to-Network Relay service:

-	Provisioning parameters for 5G ProSe Layer-3 Multi-hop UE-to-Network Relay Discovery and 5G ProSe Multi-hop UE-to-Network Relay Communication to UDR.

For 5G ProSe Layer-3 Multi-hop UE-to-UE Relay service:

-	Provisioning parameters for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery and 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Communication to UDR.

* * * * Next Change * * * *

#### 4.3.12.2	5G ProSe Layer-3 UE-to-UE Relay

In addition to the common 5G ProSe UE-to-UE Relay functionality defined in clause 4.3.12.1, a 5G ProSe Layer-3 UE-to-UE Relay supports the following functions:

-	5G ProSe Direct Communication via 5G ProSe Layer-3 UE-to-UE Relay as specified in clause 6.7.1, for the communication between 5G ProSe Layer-3 End UEs for relay operations;

-	QoS handling and end-to-end QoS treatment for the 5G ProSe Layer-3 End UE's traffic as defined in clause 5.6.3.1;

-	IP address management for the 5G ProSe Layer-3 End UE as defined in clause 5.5.1.4 when the 5G ProSe Layer-3 End UE uses IP traffic type;

-	5G ProSe UE-to-UE Relay reselection as specified in clause 6.7.4.3.

If 5G ProSe Multi-hop UE-to-UE Relay is supported, a 5G ProSe Layer-3 UE-to-UE Relay supports the following functions:

-	5G ProSe Multi-hop UE-to-UE Relay Discovery service;

-	5G ProSe Communication via Multi-hop UE-to-UE Relay as defined in clause 6.7.5;

-	Relay unicast traffic between the 5G ProSe End UEs via one or more Layer-3 UE-to-UE Relays, supporting IP, Ethernet or Unstructured traffic type;

-	For IP PDU type MANET based routing protocols are used to provide multi-hop forwarding functionality. The MANET routing protocols to be supported, i.e. Neighborhood Discovery Protocol (NHDP) [36] and OLSRv2 [35], are associated with the RSCs based on configuration;

-	For non-IP (Ethernet or Unstructured) PDU type with multi-hop support the Relay discovery procedures as defined in clause 6.3.2.6.3.1 (Model A) and clause 6.3.2.6.3.2 (Model B), the Layer-2 link establishment and management procedures as defined in clause 6.4.3.10.2 and the Communication procedure as defined in clause 6.7.5.2.2.

* * * * Next Change * * * *

### 5.1.1	General

In 5GS, the parameters for 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay, 5G ProSe UE-to-UE Relay, 5G ProSe Layer-3 Multi-hop UE-to-Network Relay and 5G ProSe Layer-3 Multi-hop UE-to-UE Relay services may be made available to the UE in following ways:

-	provisioned in the ME; or

-	configured in the UICC; or

-	provisioned in the ME and configured in the UICC; or

-	provided or updated by the ProSe Application Server via PCF and/or PC1 reference point; or

-	provided or updated by the PCF to the UE.

If the same parameters described in clauses 5.1.2.1, 5.1.3.1, 5.1.4.1 and 5.1.5.1 are provided by different sources, the UE shall consider them in the following priority order:

-	provided or updated by the PCF (including parameters determined by the PCF itself and parameters provided by the ProSe Application Server to the PCF as specified in clause 6.2.5);

-	provided or updated by the ProSe Application Server via PC1 reference point;

-	configured in the UICC;

-	provisioned in the ME.

The parameters provided or updated by the ProSe Application Server via PC1 reference point may need to be complemented with configuration data from other sources listed above.

NOTE:	The ProSe Application Server can provision the same ProSe parameters via 5GC as specified in clause 6.2.5 or directly to the UE via PC1 reference point and can revoke (e.g. delete) the ProSe parameters via 5GC as specified in clause 6.2.5 in order for the provisioning via PC1 reference point to take effect.

The basic principles of service authorization and provisioning for 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay, 5G ProSe UE-to-UE Relay, 5G ProSe Layer-3 Multi-hop UE-to-Network Relay and 5G ProSe Layer-3 Multi-hop UE-to-UE Relay services are as follows:

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorized to use 5G ProSe Direct Discovery.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to use 5G ProSe Direct Communication.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe UE-to-Network Relay. Authorisation for 5G ProSe Layer-2 UE-to-Network Relay and 5G ProSe Layer-3 UE-to-Network Relay are independent of each other.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to access 5GC via 5G ProSe UE-to-Network Relay (i.e. to act as 5G ProSe Remote UE). Authorisation to access via 5G ProSe Layer-2 UE-to-Network Relay and via 5G ProSe Layer-3 UE-to-Network Relay are independent of each other.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe UE-to-UE Relay. Authorisation for 5G ProSe Layer-2 UE-to-UE Relay and 5G ProSe Layer-3 UE-to-UE Relay are independent of each other.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to access 5G ProSe UE-to-UE Relay (i.e. to act as 5G ProSe End UE). Authorisation to access via 5G ProSe Layer-2 UE-to-UE Relay and via 5G ProSe Layer-3 UE-to-UE Relay are independent of each other.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe Layer-3 Multi-hop Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe Layer-3 Multi-hop Intermediate UE-to-Network Relay.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay.

-	The PCF in the HPLMN merges authorization information from home and other PLMNs and provides the UE with the final authorization information.

-	The PCF in the VPLMN or HPLMN may revoke the authorization (via H-PCF when roaming) at any time by using the UE Configuration Update procedure for transparent UE Policy delivery procedure defined in clause 4.2.4.3 of TS 23.502 [5].

-	The ProSe Policy/parameters provisioning to UE is controlled by the PCF and may be triggered by UE. The PCF provisions one or more of the following ProSe Policy/parameters:

-	ProSe Policy/parameters for 5G ProSe Direct Discovery as specified in clause 5.1.2.1;

-	ProSe Policy/parameters for 5G ProSe Direct Communications as specified in clause 5.1.3.1;

-	ProSe Policy/parameters for 5G ProSe Layer-2 and/or Layer-3 UE-to-Network Relay as specified in clause 5.1.4.1;

-	ProSe Policy/parameters for 5G ProSe Layer-2 and/or Layer-3 Remote UE as specified in clause 5.1.4.1.

-	ProSe Policy/parameters for 5G ProSe Layer-2 and/or Layer-3 UE-to-UE Relay as specified in clause 5.1.5.1.

-	ProSe Policy/parameters for 5G ProSe Layer-2 and/or Layer-3 End UE as specified in clause 5.1.5.1.

-	ProSe Policy/parameters for 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay as specified in clause 5.1.4.1a.

-	ProSe Policy/parameters for 5G ProSe Layer-3 Multi-hop Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay as specified in clause 5.1.4.1a.

-	ProSe Policy/parameters for 5G ProSe Layer-3 Multi-hop Intermediate UE-to-Network Relay as specified in clause 5.1.4.1a.

-	ProSe Policy/parameters for 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay as specified in clause 5.1.5.1a.

-	ProSe Policy/parameters for 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay as specified in clause 5.1.5.1a.

-	The PCF includes the 5G ProSe Policy/parameters in a Policy Section identified by a Policy Section Identifier (PSI) as specified in clause 6.1.2.2.2 of TS 23.503 [9].

In addition to the above, ProSe usage reporting configuration and rules for charging can be (pre)configured in the UE or provided by the PCF.

In addition to the above, the path selection policy can be (pre)configured in the UE or provided by the PCF as defined in clause 5.11. A path preference for ProSe Services can be provided by ProSe Application Server to UDR and may be used by PCF for path selection policy generation and update.

When a 5G ProSe Layer-3 Remote UE is using a 5G ProSe Layer-3 UE-to-Network Relay without involving N3IWF, the PCF based provisioning and update of 5G ProSe Policy/parameters to the 5G ProSe Layer-3 Remote UE are not supported.

* * * * Next Change * * * *

### 5.1.1a	General principles for applying policy/parameters

For services (i.e. 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay discovery, 5G ProSe UE-to-Network Relay communication, 5G ProSe UE-to-UE Relay discovery, 5G ProSe UE-to-UE Relay communication, 5G ProSe Layer-3 Multi-hop UE-to-Network Relay discovery, 5G ProSe Layer-3 Multi-hop UE-to-Network Relay communication, 5G ProSe Layer-3 Multi-hop UE-to-UE Relay discovery and 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication) over PC5 reference point, the operator may pre-configure the UEs with the required provisioning parameters for the service, without the need for the UEs to connect to the 5GC to get this initial configuration. The following apply:

-	The provisioning parameters for the service could be from different sources and their priorities are described in clause 5.1.1.

-	The ME provisioning parameters shall not be erased when a USIM is deselected or replaced.

-	The UE shall use radio resources for the service as follows:

-	While a UE has a serving cell and is camped on a cell and the UE intends to use for 5G ProSe the radio resources (i.e. carrier frequency) operated by this cell, then the UE shall use the radio resource description indicated by this cell the UE is camped on and ignore any radio resource description of the same radio resource provisioned in the ME or the UICC. If the cell does not provide radio resources for ProSe, the UE shall not perform ProSe message transmission and reception on radio resources operated by this cell. The UE is allowed to perform the service with another UE not served by the same PLMN;

-	If the UE intends to use "operator-managed" radio resources (i.e. carrier frequency) for 5G ProSe that are not operated by the UE's serving cell, as specified in the related Policy/Parameter provisioning, or if the UE is out of coverage, the UE shall search for a cell in any PLMN that is operating the provisioned radio resources (i.e. carrier frequency) as defined in TS 38.300 [12] and TS 38.304 [13]; and:

-	If the UE finds such a cell in the registered PLMN or a PLMN equivalent to the registered PLMN and authorization for the service to this PLMN is confirmed, the UE shall use the radio resource description indicated by that cell. If that cell does not provide radio resources for ProSe, the UE shall not perform ProSe message transmission and reception on those radio resources;

-	If the UE finds such a cell but not in the registered PLMN or a PLMN equivalent to the registered PLMN and that cell belongs to a PLMN authorized for the service and provides radio resources for ProSe then the UE shall perform PLMN selection triggered by the service as defined in TS 23.122 [14];

-	If the UE finds such cell but not in a PLMN authorized for the service the UE shall not use the service;

-	If the UE does not find any such cell in any PLMN, then the UE shall consider itself "not served by NG-RAN" and use radio resources provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize the service, then the UE is not authorized to transmit;

-	The UE is allowed to use "operator-managed" radio resources (i.e. carrier frequency) provisioned in the ME or the UICC for the service if the UICC indicates it is authorized;

-	If the UE intends to use "non-operator-managed" radio resources (i.e. carrier frequency) for 5G ProSe, according to TS 36.331 [15] or TS 38.331 [16] and as specified in clause 5.1.2.1, then the UE shall perform the service using resource provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize the service, then the UE is not authorized to transmit;

NOTE 1:	It is possible for operators to configure UEs (e.g. Public Safety UEs) to use only "operator-managed" radio resources (i.e. carrier frequency) for the service when the UE is "not served by NG-RAN".

-	The UE provisioning shall support setting Geographical Areas;

NOTE 2:	It is possible for a UE to use other radio resources for 5G ProSe based on the Geographical Area instead of those operated by the serving NG-RAN cell, when provisioned in the UE, even if the UE's serving cell offers normal service and the SIBs for NR Sidelink communication defined in TS 38.331 [16] indicates that the service (the service) is available. This is to cover the scenario when e.g. the radio resources used for the service are not owned by the serving network of the UE.

NOTE 3:	When cross-carrier operation is supported, according to TS 36.331 [15] or TS 38.331 [16], a UE can be instructed by its serving cell to perform the service over a different carrier frequency. The UE is still considered as "served by NG-RAN" in this case.

NOTE 4:	The scenario that a cell is detected and the cell does not provide support for the service when the UE attempts to use a carrier frequency configured for the service, is considered a configuration error. Therefore, the UE does not transmit on that frequency to avoid interference to the network.

-	The service is only specified for NR.

NOTE 5:	It is out of scope of the present specification to define how the UE can locate itself in a specific Geographical Area. When the UE is in coverage of a 3GPP RAT, it can for example, use information derived from the serving PLMN. When the UE is not in coverage of a 3GPP RAT, it can use other techniques, e.g. Global Navigation Satellite System (GNSS). User provided location is not a valid input.

* * * * Next Change * * * *

#### 5.1.4.1a	Policy/Parameter provisioning for 5G ProSe Multi-hop UE-to-Network Relay

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay when "served by NG-RAN":

-	PLMNs in which the UE is authorized to relay traffic for 5G ProSe Layer-3 Remote UEs supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay.

The authorisation for a UE to act as a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay also authorizes the use of 5G ProSe Layer-3 Multi-hop UE-to-Network Relay Discovery with Model A and Model B.

NOTE 1:	It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe Layer-3 Multi-hop UE-to-Network Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 Multi-hop UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer Indicator per RSC, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. RSC dedicated for emergency service may also be provisioned. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe Multi-hop UE-to-Network Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

NOTE 2:	The RSC used for multi-hop service is different from the RSC used for single-hop service.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement and Relay Discovery Additional Information messages and receiving Relay Discovery Solicitation messages;

-	For 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic for each ProSe Relay Service Code;

NOTE 3:	5G ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the 5G ProSe UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay.

3)	For 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay to relay Ethernet or Unstructured traffic from 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay by using IP type PDU Session:

-	Mapping of ProSe Service(s) to ProSe Application Server address information (consisting of IP address/FQDN and transport layer port number).

4)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 Multi-hop UE-to-Network Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay and thereby enabling the use of a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay:

1)	Authorisation policy for using a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay:

-	For 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay, indicates whether the UE is authorised to use a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay.

The authorisation for a UE to act as a 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay also authorizes the use of 5G ProSe Layer-3 Multi-hop UE-to-Network Relay discovery with Model A and Model B.

NOTE 4:	It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously.

2)	Policy/parameters for 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay:

-	Includes the parameters for 5G ProSe Relay Discovery and for enabling the UE to connect to the 5G ProSe UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay after discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 Multi-hop UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer indicator per RSC, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. RSC(s) dedicated for emergency service may be provisioned to enable the support of emergency services for Multi-hop UE-to-Network Relaying. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe Multi-hop UE-to-Network Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

NOTE 5:	The RSC used for multi-hop service is different from the RSC used for single-hop service.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Solicitation messages and receiving Relay Discovery Announcement and Relay Discovery Additional Information messages;

-	For 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic without using N3IWF access, or an indication of N3IWF access, for each ProSe Relay Service Code;

-	For 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay, optionally the ProSe application Traffic Descriptor(s) (as defined in TS 23.503 [9]) to be used for the relayed traffic for each ProSe Relay Service Code;

3)	Policy/parameters for N3IWF selection for 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay:

-	N3IWF identifier configuration for 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay (either FQDN or IP address) in the HPLMN.

-	5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay access node selection information - a prioritized list of PLMNs for N3IWF selection. It also indicates if selection of an N3IWF in a PLMN should be based on Tracking Area Identity FQDN or on Operator Identifier FQDN.

NOTE 6:	5G ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay.

4)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 Multi-hop UE-to-Network Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 Intermediate UE-to-Network Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 Intermediate UE-to-Network Relay:

-	For 5G ProSe Layer-3 Intermediate UE-to-Network Relay, indicates whether the UE is authorised to act as a 5G ProSe Layer-3 Intermediate UE-to-Network Relay.

The authorisation for a UE to act as a 5G ProSe Layer-3 Intermediate UE-to-Network Relay also authorizes the use of 5G ProSe Layer-3 Multi-hop UE-to-Network Relay Discovery with Model A and Model B.

NOTE 7:	It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe Layer-3 Intermediate UE-to-Network Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe Layer-3 Multi-hop UE-to-Network Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 Multi-hop UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer Indicator per RSC, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. RSC dedicated for emergency service may also be provisioned. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe Multi-hop UE-to-Network Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

NOTE 8:	The RSC used for multi-hop service is different from the RSC used for single-hop service.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement and Relay Discovery Additional Information messages and receiving Relay Discovery Solicitation messages;

-	For 5G ProSe Layer-3 Intermediate UE-to-Network Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

NOTE 9:	5G ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the 5G ProSe Layer-3 Intermediate UE-to-Network Relay.

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 Intermediate UE-to-Network Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay, in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay and thereby enabling the use of a 5G ProSe Layer-3 UE-to-Network Relay and in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 Intermediate UE-to-Network Relay:

1)	Radio parameters for 5G ProSe Layer-3 Multi-hop UE-to-Network Relay Discovery when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

-	Default PC5 DRX configuration (see TS 38.331 [16]).

2)	Radio parameters for 5G ProSe Layer-3 Multi-hop UE-to-Network Relay communication when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Communication over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 10:	The validity time of these radio parameters is the same as the validity time of the Policy/Parameter listed above for 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay or 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay or 5G ProSe Layer-3 Intermediate UE-to-Network Relay.

3)	Policy/parameters related to privacy:

-	A privacy timer value indicating the duration after which the UE shall change each source Layer-2 ID self-assigned by the UE when privacy is required.

* * * * Next Change * * * *

##### 5.1.4.2.1a	Principles for applying parameters for 5G ProSe Multi-hop UE-to-Network Relay discovery

The general principles for applying policy/parameters defined in clause 5.1.1a apply to the 5G ProSe Multi-hop UE-to-Network Relay discovery service.

* * * * Next Change * * * *

##### 5.1.4.2.2a	Principles for applying parameters for 5G ProSe Multi-hop UE-to-Network Relay communication

The general principles for applying policy/parameters defined in clause 5.1.1a apply to the 5G ProSe Multi-hop UE-to-Network Relay communication service.

* * * * Next Change * * * *

#### 5.1.5.1a	Policy/Parameter provisioning for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay:

-	when the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to relay traffic for 5G ProSe Layer-3 End UEs supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay accessing 5G ProSe Layer-3 UE-to-UE Relays supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay over PC5 reference point.

-	when the UE is not "served by NG-RAN":

-	Indicates whether the UE is authorized to relay traffic for 5G ProSe Layer-3 End UEs supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay accessing 5G ProSe Layer-3 UE-to-UE Relays supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay over PC5 reference point.

-	The authorisation for a UE to act as a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay also authorizes the use of 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery with Model A and Model B.

NOTE 1:	It is up to UE and application implementation to select a discovery model, or whether to perform both models simultaneously.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-UE Relay Layer indicator, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-UE Relay Layer indicator indicates the associated RSC is offering 5G ProSe Layer-3 UE-to-UE Relay service. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement message and sending/receiving Relay Discovery Solicitation messages;

-	For 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay and thereby enabling the use of a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay:

1)	Authorisation policy for using a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay:

-	When the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to use a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay.

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorised to use a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay.

The authorisation for a UE to act as a 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay also authorizes the use of 5G ProSe Layer-3 UE-to-UE Relay discovery with Model A and Model B.

NOTE 2:	It is up to UE and application implementation to select a discovery model, or whether to perform both models simultaneously.

2)	Policy/parameters for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery:

-	Includes the parameters for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery as a 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay and for enabling the UE to connect to the 5G ProSe Layer-3 UE-to-UE Relay after discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery parameters (Relay Service Code(s), UE-to-UE Relay Layer indicator, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-UE Relay Layer indicator indicates the associated RSC is offering 5G ProSe Layer-3 UE-to-UE Relay service. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

-	Default Destination Layer-2 ID(s) for sending/receiving Relay Discovery Solicitation messages and receiving Relay Discovery Announcement message;

-	For 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay as well as in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay and thereby enabling the use of a 5G ProSe Layer-3 UE-to-UE Relay:

1)	Radio parameters for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

-	Default PC5 DRX configuration (see TS 38.331 [16]).

NOTE 3:	Radio parameters for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery when the UE is not "served by NG-RAN" and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe Direct Discovery in clause 5.1.2.1 are expected to be aligned for direct and relayed discovery for UE to UE communication.

2)	Radio parameters for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Communication over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 4:	Radio parameters for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication when the UE is not "served by NG-RAN" and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe Direct Communication in clause 5.1.3.1 are expected to be aligned for direct and relayed UE to UE communication.

NOTE 5:	The RSC used for multi-hop service is different from the RSC used for single-hop service.

* * * * Next Change * * * *

##### 5.1.5.2.1a	Principles for applying parameters for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay discovery

The general principles for applying policy/parameters defined in clause 5.1.1a apply to the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay discovery service.

* * * * Next Change * * * *

##### 5.1.5.2.2a	Principles for applying parameters for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication

The general principles for applying policy/parameters defined in clause 5.1.1a apply to the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication service.

* * * * Next Change * * * *

### 5.2.6	5G ProSe Multi-hop UE-to-Network Relay Discovery

For 5G ProSe Multi-hop UE-to-Network Relay discovery, both Model A and Model B discovery are supported.

The procedures for 5G ProSe Multi-hop UE-to-Network Relay discovery Model A are defined in clause 6.3.2.5.2.

The procedures for 5G ProSe Multi-hop UE-to-Network Relay discovery Model B are defined in clause 6.3.2.5.3.

For Additional parameters announcement procedure, only Model A discovery is used and is defined in clause 6.3.2.5.4.

* * * * Next Change * * * *

#### 5.6.3.1	QoS handling for 5G ProSe Layer-3 UE-to-UE Relay

For a 5G ProSe Layer-3 End UE connecting with another 5G ProSe Layer-3 End UE(s) via 5G ProSe Layer-3 UE-to-UE Relay, the QoS requirement of the relay traffic between the peer 5G ProSe Layer-3 End UE(s) can be satisfied by the corresponding QoS control for the PC5 link between source 5G ProSe Layer-3 End UE and 5G ProSe Layer-3 UE-to-UE Relay (i.e. first hop PC5 QoS control) and the QoS control for the PC5 link between 5G ProSe Layer-3 UE-to-UE Relay and target 5G ProSe Layer-3 End UE (i.e. second hop PC5 QoS control). The first hop PC5 QoS and second hop PC5 QoS is controlled with PC5 QoS rules and PC5 QoS parameters (e.g. PQI, GFBR, MFBR, PC5 LINK-AMBR) as specified in clause 5.6.1.

As shown in figure 5.6.3.1-1 below, the end-to-end QoS is met only when the QoS requirements are properly translated and satisfied over the two legs respectively.

Figure 5.6.3.1-1: End-to-End QoS for 5G ProSe Layer-3 UE-to-UE Relay operation

To achieve this, the source 5G ProSe Layer-3 End UE initiates PC5 QoS Flows setup or modification during the Layer-2 link establishment or modification procedure, the source 5G ProSe Layer-3 End UE provides the QoS Info as described in clause 6.4.3.7.3 to the 5G ProSe Layer-3 UE-to-UE Relay. The received PC5 QoS parameters of the QoS Info (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.) are interpreted as the end-to-end QoS requirements by the 5G ProSe Layer-3 UE-to-UE Relay for the traffic transmission between source 5G ProSe Layer-3 End UE and target 5G ProSe Layer-3 End UE. The source 5G ProSe Layer-3 End UE derives the end-to-end QoS parameters as defined in clause 5.6.1. The 5G ProSe Layer-3 UE-to-UE Relay, based on its implementation, decides the PQI for the first hop PC5 QoS control and the PQI for the second hop PC5 QoS control, by considering the received PC5 QoS parameters from the source 5G ProSe Layer-3 End UE. The 5G ProSe Layer-3 UE-to-UE Relay provides the QoS Info (including PQI value chosen by the 5G ProSe Layer-3 UE-to-UE Relay for the second hop) to the target 5G ProSe Layer-3 End UE. After accepted QoS Info of the second hop QoS from the target 5G ProSe Layer-3 End UE is received, 5G ProSe Layer-3 UE-to-UE Relay provides the QoS Info (including PQI value chosen by the 5G ProSe Layer-3 UE-to-UE Relay for the first hop) to the source 5G ProSe Layer-3 End UE with considering the received second hop QoS. If the source 5G ProSe Layer-3 End UE performs the Layer-2 link modification procedure to add new PC5 QoS Flow(s) or modify the existing PC5 QoS Flow(s) for IP traffic or Ethernet traffic over PC5 reference point, the source 5G ProSe Layer-3 End UE may also provide the PC5 QoS Rule(s) for the PC5 QoS Flow(s) to be added or modified to the 5G ProSe Layer-3 UE-to-UE Relay. The 5G ProSe Layer-3 UE-to-UE Relay may generate the Packet Filters used over the second hop based on the received PC5 QoS Rule(s).

For Multi-hop 5G ProSe Layer-3 UE-to-UE Relay for non-IP type PDU, each 5G ProSe Layer-3 UE-to-UE Relays split the QoS parameters, according to the received QoS Info, into two parts: one part is the QoS parameters of the previous hop, the other part is the QoS parameters from the 5G ProSe Layer-3 UE-to-UE Relay to the target End UE (the rest QoS parameters). The 5G ProSe Layer-3 UE-to-UE Relay sends the rest QoS parameters to the next hop. When 5G ProSe Layer-3 UE-to-UE determines the QoS parameter, such as PDB, of the previous hop, it may consider the maximum value as the received PDB devided by the number of hops to the target End UE.

As shown in figure 5.6.3.1-2 below, the end-to-end QoS is met only when the QoS requirements are properly satisfied over the multiple legs respectively.

Figure 5.6.3.1-2: End-to-End QoS for Multi-hop 5G ProSe Layer-3 UE-to-UE Relay for non-IP type PDU operation

To achieve this, when the source End UE sets up a PC5 QoS Flow, it provides the End-to-End QoS parameters to a 5G ProSe Layer-3 UE-to-UE Relay. Each 5G ProSe Layer-3 UE-to-UE Relay splits the QoS parameters, according to the received QoS Info, into the QoS parameters of the previous hop and the QoS parameters from itself to the target End UE. The 5G ProSe Layer-3 UE-to-UE Relay sends the rest PC5 QoS parameters to the next hop. Each 5G ProSe Layer-3 UE-to-UE Relay may equally split the PDB among the hops, or up to its own implementation (based on equally splitted value as upper bound).

The target End UE and each 5G ProSe Layer-3 UE-to-UE Relay sends the accepted PC5 QoS parameters and optionally the accumulated QoS parameters to the previous hop during the Layer-2 link establishment or modification procedure. The accepted PC5 QoS parameters may be determined based on the QoS parameters of the previous hop as mentioned above, with considering the received accumulated QoS from next hop.

* * * * Next Change * * * *

#### 5.8.3.1	Common identifiers for 5G ProSe UE-to-Network Relay

The following parameters are used for the 5G ProSe UE-to-Network Relay Discovery Announcement message (Model A), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Announcer Info and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery is selected based on the configuration as described in clause 5.1.4.1.

-	Announcer Info: provides information (i.e. User Info ID) about the announcing user.

-	Relay Service Code: parameter identifying a connectivity service the 5G ProSe UE-to-Network Relay provides to a 5G ProSe Remote UE. The Relay Service Codes are configured in a 5G ProSe UE-to-Network Relay for advertisement. Additionally, the Relay Service Code may also identifies authorized users the 5G ProSe UE-to-Network Relay would offer service to and may be used to select the related security policies or information e.g. necessary for authentication and authorization between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay (e.g. a Relay Service Code for relays for police members only would be different than a Relay Service Code for relays for Fire Fighters only, even though potentially they provided connectivity to same DN e.g. to support Internet Access).

The 5G ProSe UE-to-Network Relay Discovery Announcement message (Model A) is extended and modified as follows when used for 5G ProSe Multi-hop UE-to-Network Relay discovery:

-	Hop-Count: This value reflects the number of PC5 hops for the 5G ProSe Remote UE to reach the network. It is set to 1 by the 5G ProSe UE-to-Network Relay and shall be incremented by 1 every time the 5G ProSe UE-to-Network Relay Discovery Announcement message is forwarded by a 5G ProSe Intermediate UE-to-Network Relay.

A 5G ProSe Intermediate UE-to-Network Relay shall only process the 5G ProSe UE-to-Network Relay Discovery Announcement if the Hop-Count is present and the value is less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message.

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay or the 5G ProSe Intermediate UE-to-Network Relay self-selects a Source Layer-2 ID when sending the discovery message.

-	Destination Layer-2 ID: the 5G ProSe Intermediate UE-to-Network Relay sets the Desination Layer-2 ID of the 5G ProSe UE-to-Network Relay Discovery Announcement message based on configuration described in clause 5.1.4.1.

-	Announcer Info: identify information (i.e. User Info ID) of the announcing 5G ProSe UE-to-Network Relay or the 5G ProSe Intermediate UE-to-Network Relay.

-	(optinal) Root Relay Info: this is the User Info ID of the 5G ProSe UE-to-Network Relay. A 5G ProSe Intermediate UE-to-Network Relay may stored it in the discovery entry and included it in the 5G ProSe UE-to-Network Relay Discovery Announcement message. This information can be used in the relay selection at the 5G ProSe Remote UE or other 5G ProSe Intermediate UE-to-Network Relays.

-	(optional) Accumulated QoS for PC5 link: this reflects the QoS supported over all the PC5 links to the Root Relay, i.e. the 5G ProSe UE-to-Network Relay, for this RSC.

-	(optional) Hop-Limit: this is the hop limit set by the announcing 5G ProSe UE-to-Network Relay to a value smaller than the configured maximum number of hops.

The following parameters are used for the 5G ProSe UE-to-Network Relay Discovery Solicitation message (Model B), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Discoverer Info and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe Remote-UE self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery is selected based on the configuration as described in clause 5.1.4.1.

-	Discoverer Info: provides information (i.e. User Info ID) about the discoverer user.

-	Target Info: provides information (i.e. User Info ID) about the targeted discoveree user.

-	Relay Service Code: information about connectivity that the discoverer UE is interested in. The Relay Service Codes are configured in the 5G ProSe Remote UEs interested in related connectivity services.

To support Multi-hop 5G ProSe UE-to-Network Relay discovery the following parameters are added:

-	(Optional) Hop count: indicates the number of hops that the message is already relayed. It is increased by 1 per hop.

-	(Optional) Hop-Limit: a unmodified value that indicates the hop limit of the message. It is set, by the 5G ProSe Remote UE, to a value smaller than the (pre)configured maximum number of hops.

-	Path information: an (ordered) list of User Info ID(s) of Intermediate UE-to-Network Relay(s) that indicates the transmitted path of the message.

-	(optional) Accumulated QoS for PC5 link: this reflects the QoS supported over all the PC5 links starting from the PC5 link between the Remote UE and a Intermediate UE-to-Network Relay in the path information for this RSC.

The following parameters are used in the 5G ProSe UE-to-Network Relay Discovery Response message (Model B), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Discoveree Info and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery.

-	Destination Layer-2 ID: set to the Source Layer-2 ID of the received 5G ProSe UE-to-Network Relay Discovery Solicitation message.

-	Relay Service Code: identifies the connectivity service the 5G ProSe UE-to-Network Relay provides to 5G ProSe Remote UEs that matches the Relay Service Code from the corresponding Discovery Solicitation message.

-	Discoveree Info: provides information (i.e. User Info ID) about the discoveree.

To support Multi-hop 5G ProSe UE-to-Network Relay discovery the following parameters are added:

-	(Optional) Hop count: indicates the number of hops between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay on the path selected by the 5G ProSe UE-to-Network Relay.

-	Path information: an (ordered) list of User Info ID(s) of the discoverer user and the Intermediate UE-to-Network Relay(s) on the path selected by the 5G ProSe UE-to-Network Relay.

-	(optional) Accumulated QoS for PC5 link: this reflects the QoS supported over all the PC5 links along the path from Remote UE to the 5G ProSe UE-to-Network Relay for this RSC.

The following parameters may be used in the Relay Discovery Additional Information message (using Model A) based on the procedure defined in clause 6.5.1.3 for 5G ProSe UE-to-Network Relay and clause 6.3.2.5.4 for 5G ProSe Multi-hop UE-to-Network Relay where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and the other parameters are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay or the 5G ProSe Intermediate UE-to-Network Relay self-selects a Source Layer-2 ID to send the Relay Discovery Additional Information message.

-	Destination Layer-2 ID: the Destination Layer-2 ID to send the Relay Discovery Additional Information message is selected based on the configuration as described in clause 5.1.4.1.

-	Relay Service Code: the Relay Service Code associated with the message. The Relay Service Code is used to identify the security parameters needed by the receiving UE to process the discovery message.

-	Announcer Info: provides information about the announcing user (i.e. User Info ID of the 5G ProSe UE-to-Network Relay).

-	Hop-Count: This value reflects the number of PC5 hops for the 5G ProSe Remote UE to reach the network. It is set to 1 by the 5G ProSe UE-to-Network Relay and shall be incremented by 1 every time the Relay Discovery Additional Information message is forwarded by a 5G ProSe Intermediate UE-to-Network Relay.

A 5G ProSe Intermediate UE-to-Network Relay shall only process the Relay Discovery Additional Information message if the Hop-Count is present and the value is less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message.

-	Announcer Info of 5G ProSe Intermediate UE-to-Network Relay: identify information (i.e. User Info ID) of the 5G ProSe Intermediate UE-to-Network Relay connected to 5G ProSe UE-to-Network Relay. This parameter is only applicable to the additional parameter announcement procedure over 5G ProSe Multi-hop UE-to-Network Relay specified in clause 6.3.2.5.4.

-	(optional) Hop-Limit: this is the hop limit set by the announcing 5G ProSe UE-to-Network Relay to a value smaller than the configured maximum number of hops.

-	Additional parameters: the additional parameters for 5G ProSe Layer-3 UE-to-Network Relay (when applicable) are defined in clause 5.8.3.2.

NOTE 1:	The UE implementation needs to ensure that when the UE self-selects Source Layer-2 IDs, the self-selected Source Layer-2 IDs are different between 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery) in clause 6.3.2 and 5G ProSe Direct Communication (including 5G ProSe UE-to-Network Relay Communication) in clause 6.4 and are different from any other provisioned Destination Layer-2 IDs as described in clause 5.1 and any other self-selected Source Layer-2 IDs used in a simultaneous 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery) with a different discovery model.

NOTE 2:	If a 5G ProSe UE-to-Network Relay and 5G ProSe Remote UE from different PLMNs discover each other, it means that the Relay Service Code is associated with the same connectivity service and the same Relay Service Code is provisioned based on Service Level Agreement among PLMNs.

NOTE 3:	The Hop count, Hop-Limit and Path information are used for Multi-hop 5G ProSe UE-to-Network Relay Discovery with Model B. The Multi-hop UE-to-Network Relay Discovery message has no impact on 5G ProSe UE-to-Network Relay or 5G ProSe Remote UE that only supports the single-hop UE-to-Network Relay. Single-hop and Multi-hop UE-to-Network Relays discoveries can be distinguished based on RSC.

* * * * Next Change * * * *

#### 5.8.3.2	Identifiers for 5G ProSe Layer-3 UE-to-Network Relay

For 5G ProSe Layer-3 UE-to-Network relay, a Relay Service Code in the Announcement Message is associated with a set of PDU session parameters (e.g. PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference). The Relay Service Code may also represent if the relay UE can provide secure N3IWF connection.

For 5G ProSe Layer-3 Remote UE discovering 5G ProSe Layer-3 UE-to-Network relay, the Relay Service Code in the Solicitation Message represents the PDU session parameters that a PDU session of the relay should be able to support. The Relay Service Code may also represent if the remote UE requires secure N3IWF connection.

The following additional parameters may be used in the Relay Discovery Additional Information message (using Model A) for 5G ProSe Layer-3 UE-to-Network Relay:

-	NCGI: indicates the NCGI of the serving cell of the 5G ProSe Layer-3 UE-to-Network Relay. This parameter may be requested by application running on 5G ProSe Layer-3 Remote UE.

-	TAI: indicates the Tracking Area Identity of the serving cell of the 5G ProSe Layer-3 UE-to-Network Relay. This parameter may be used by 5G ProSe Layer-3 Remote UE to select a N3IWF.

-	NCGI of 5G ProSe Intermediate UE-to-Network Relay: indicates the NCGI of the serving cell of the 5G ProSe Intermediate UE-to-Network Relay connected to the 5G ProSe UE-to-Network Relay. This parameter may be requested by application running on 5G ProSe Layer-3 Remote UE and is only applicable to the additional parameter announcement procedure over 5G ProSe Multi-hop UE-to-Network Relay specified in clause 6.3.2.5.4.

* * * * Next Change * * * *

#### 6.3.2.5	5G ProSe Multi-hop UE-to-Network Relay Discovery

##### 6.3.2.5.1	General

5G ProSe Multi-hop UE-to-Network Relay Discovery is applicable to both 5G ProSe Multi-hop Layer-3 and Layer-2 UE-to-Network relay discovery for public safety use and commercial services. To perform 5G ProSe Multi-hop UE-to-Network Relay Discovery, the 5G ProSe Remote UE, the 5G ProSe Intermedidate UE-to-Network Relay and the 5G ProSe UE-to-Network Relay are (pre-)configured or provisioned with the related information as described in clause 5.1.4

The RSC is used in the 5G ProSe Multi-hop UE-to-Network Relay discovery, to indicate the connectivity service the 5G ProSe Intermedidate UE-to-Network Relay and the 5G ProSe UE-to-Network Relay provide to the 5G ProSe Remote UE. The maximum number of hops is provisioned by the network to the 5G ProSe Remote UE, 5G ProSe Intermedidate UE-to-Network Relay and the 5G ProSe UE-to-Network Relay per RSC.

A 5G ProSe UE-to-Network Relay supporting multiple RSCs can advertise the RSCs using multiple discovery messages, with one RSC per discovery message.

Additional information not directly used for discovery can also be advertised using the PC5-D protocol stack in single or separate discovery messages of type "Relay Discovery Additional Information".

* * * * Next Change * * * *

##### 6.3.2.5.2	Procedure for 5G ProSe Multi-hop UE-to-Network Relay Discovery with Model A

Figure 6.3.2.5.2-1 presents the procedures for 5G ProSe Multi-hop UE-to-Network Relay Discovery using Model A discovery.

Figure 6.3.2.5.2-1: 5G ProSe Multi-hop UE-to-Network Relay discovery procedure with Model A

0.	The 5G ProSe Remote UE, the 5G ProSe Intermedidate UE-to-Network Relay and the 5G ProSe UE-to-Network Relay are (pre-)configured or provisioned with the related information to support Multi-hop UE-to-Network Relay discovery, as specified in clause 5.1.4. If the 5G ProSe Remote UE, 5G ProSe Intermedidate UE-to-Network Relay and 5G ProSe UE-to-Network Relay belong to different PLMNs, their HPLMNs ensure that consistent configuraitons are provided.

1.	If a 5G ProSe UE-to-Network Relay is successfully registered and authorized by the 5GS and has the configurations to operate Multi-hop UE-to-Network Relay in the current region, it can start the discovery procedure. Otherwise, it shall not perfrom the 5G ProSe Multi-hop UE-to-Network Relay discovery, i.e. not sending any announcement message.

2.	The 5G ProSe UE-to-Network Relay sends a 5G ProSe UE-to-Network Relay Discovery Announcement message. To support Multi-hop UE-to-Network Relay discovery, the 5G ProSe UE-to-Network Relay sets the Hop-Count to 1, sets the Announcer Info to its own User Info ID and sets the RSC according to the configuration as in clause 5.1.4.

3.	The 5G ProSe Intermedidate UE-to-Network Relay creates a corresponding discovery entry based on the received 5G ProSe UE-to-Network Relay Discovery Announcement message, if the discovery message meets the AS layer criteria and the Hop-Count is less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message.

The discovery entry includes the following information:	RSC, (optional) Root Relay Info, Announcer Info, Hop-Count, (optional) Accumulated QoS for PC5 link, (optional) NCGI, (optional) RRC Container, (optional) Hop-Limit.

The Announcer Info is set to the Announcer Info of the received message. The Root Relay Info is set to the Root Relay Info of the reived message. The Hop-count is set to the value of the received message. The Accumulated QoS for PC5 link, if present in the received discovery message, will be updated based on the QoS of the PC5 link between the 5G ProSe Intermedidate UE-to-Network Relay and the sender of the message (e.g. sum of delay of each hop).

If an discovery entry already exist, i.e. with the same RSC and (optional) Root Relay Info and if the Hop-Count of the received message is less than the Hop-Count stored in the discovery entry, the 5G ProSe Intermedidate UE-to-Network Relay updates the discovery entry accordingly. The Announcer Info and Hop-Count are set to the Annoncer Info and Hop-Count of the newly received discovery message.

NOTE:	Based on implementation, a 5G ProSe Intermedidate UE-to-Network Relay may decide to update the discovery entry based on the Accumulated QoS for PC5 link instead of the Hop-Count, or a combination of the two information elements.

4a.	5G ProSe Intermedidate UE-to-Network Relay sends a 5G ProSe UE-to-Network Relay Discovery Announcement message, using the information from the stored discovery entry. The Hop-Count value will be incremented by 1 and the Announcer Info is set to the User Info of the 5G ProSe Intermedidate UE-to-Network Relay.

When received by the 5G ProSe Remote UE, the information in the 5G ProSe UE-to-Network Relay Discovery Announcement message will be used for relay (re)selection, if the Hop-Count is less or equal to the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message. The 5G ProSe Remote UE can also take the Accumulated QoS for PC5 link into account. For relay reselection, the Root Relay Info may be used to prioritize a relay offering the connection to the same 5G ProSe UE-to-Network Relay.

4b.	5G ProSe Intermedidate UE-to-Network Relay sends a 5G ProSe UE-to-Network Relay Discovery Announcement message, using the information from the stored discovery entry. This message may reach another 5G ProSe Intermedidate UE-to-Network Relay.

5.	The 5G ProSe Intermedidate UE-to-Network Relay drops the received message if the Hop-Count is not less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message.

6.	The 5G ProSe Intermedidate UE-to-Network Relay sets a locally configured timer for each of the discovery entry based on implementation. If the 5G ProSe Intermedidate UE-to-Network Relay does not have an active PC5 link with the relay identfied by the Announcer Info and it does not receive any new announcement message from that relay when the timer expires, the 5G ProSe Intermedidate UE-to-Network Relay removes the corresponding discovery entry and stops sending related 5G ProSe UE-to-Network Relay Discovery Announcement message.

The 5G ProSe Intermediate UE-to-Network Relay shall establish a Layer-2 link with the parent Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay identified by the Announcer Info in the discovery entry before sending the 5G ProSe UE-to-Network Relay Discovery Announcement message.

* * * * Next Change * * * *

##### 6.3.2.5.4	Additional parameters announcement procedure over 5G ProSe Multi-hop UE-to-Network Relay

Additional parameters announcement procedure outlined in figure 6.3.2.5.4-1 is used by a 5G ProSe Remote UE to request a 5G ProSe UE-to-Network Relay to announce additional parameters (using Model A). The Additional parameters announcement procedure specified in clause 6.5.1.3 is used with the following differences described in the procedure below.

Figure 6.3.2.5.4-1: Procedure for additional parameters announcement for Multi-hop UE-to-Network relaying

1.	5G ProSe Remote UE has discovered a 5G ProSe UE-to-Network Relay for connectivity to the network as described in clause 6.3.2.5.2 and requires additional parameters.

2.	The 5G ProSe Remote UE sends to the 5G ProSe Intermediate UE-to-Network Relay an Additional Parameters Announcement Request including the following additional information to obtain additional parameters.

-	Relay Service Code: information about the connectivity service provided by the 5G ProSe UE-to-Network Relay.

-	Root Relay Info: identify information (i.e. User Info ID) of the 5G ProSe UE-to-Network Relay.

3-4.	The Additional Parameters Announcement Request is sent up to the 5G ProSe UE-to-Network Relay.

5.	The 5G ProSe UE-to-Network Relay acknowledges receipt of the request in step 4 with an Additional Parameters Announcement Response (Additional_Parameters_Announcement_Request_Refresh Timer).

6-7.	The Additional Parameters Announcement Response is sent to the 5G ProSe Remote UE, relayed via the 5G ProSe Intermediate UE-to-Network Relay.

8.	The 5G ProSe UE-to-Network Relay reuses the Relay Discovery Additional Information message as defined in clause 5.8.3 with the following additional information:

-	Hop-Count: the value is set to 1.

The Destination Layer-2 ID to send the Relay Discovery Additional Information message is selected based on the configuration as described in clause 5.1.4.1.

9.	The 5G ProSe Intermediate UE-to-Network Relay forwards the Relay Discovery Additional Information message. The 5G ProSe Intermediate UE-to-Network Relay updates the Hop-Count as below and may include additional information, Announcer Info and NCGI of 5G ProSe Intermediate UE-to-Network Relay if its NCGI is available:

-	Hop-Count: the value of the received Relay Discovery Additonal Information message is incremented by 1.

-	Announcer Info of 5G ProSe Intermediate UE-to-Network Relay: identity information (i.e. User Info ID) of the 5G ProSe Intermediate UE-to-Network Relay transmitting this message.

-	NCGI of 5G ProSe Intermediate UE-to-Network Relay: indicates the NCGI of the serving cell of the 5G ProSe Intermediate UE-to-Network Relay.

10.	The 5G ProSe Intermediate UE-to-Network Relay forwards the Relay Discovery Additional Information message, with the Hop-Count incremented. The 5G ProSe Intermediate UE-to-Network Relay shall not forward the Relay Discovery Additional Information message if the Hop-Count is bigger than the configured maximum number of hops.

11.	The 5G ProSe UE-to-Network Relay detects new or updated additional parameters.

12-14.	Same as steps 8-10.

If the 5G ProSe Intermediate UE-to-Network Relay connected to 5G ProSe UE-to-Network Relay detects new or updated additional parameters (e.g. due to change of its serving cell, moving into network coverage), it may send a Relay Discovery Additional Information message including the new or updated additional parameters. In this case, the 5G ProSe Intermediate UE-to-Network Relay acts as the 5G ProSe UE-to-Network Relay in figure 6.3.2.5.4-1, i.e. to send Relay Discovery Additional Information message. The 5G ProSe Intermediate UE-to-Network Relay also keeps a record of the additional parameters from the 5G ProSe UE-to-Network Relay.

NOTE:	The NCGI of the 5G ProSe Intermediate UE-to-Network Relay connected to 5G ProSe UE-to-Network Relay can be used as supplementary by the 5G ProSe Remote UE and not provided to the network side from the 5G ProSe Remote UE.

* * * * Next Change * * * *

###### 6.3.2.6.3.1	Procedure for Multi-hop 5G ProSe UE-to-UE Relay Discovery of non-IP PDU type with Model A

Depicted in Figure 6.3.2.6.3.1-1 is the procedure for Multi-hop 5G ProSe UE-to-UE Relay Discovery of non-IP PDU type with Model A.

Figure 6.3.2.6.3.1-1: 5G ProSe Multi-hop UE-to-UE Relay Discovery of non-IP PDU type with Model A

1.	The 5G ProSe UE-to-UE Relay has discovered End UEs in proximity and obtains the Direct discovery set from End UEs in proximity per RSC. (e.g. via a previous 5G ProSe UE-to-UE Relay Discovery or via secure PC5 connection between 5G ProSe U2U Relay and 5G ProSe End UE (refer to TS 33.503 [29])). 5G ProSe UE-to-UE Relay may also get the Hop-Limit from End UEs.

2.	The 5G ProSe UE-to-UE Relay sends a UE-to-UE Relay Discovery Announcement message. The UE-to-UE Relay Discovery Announcement message contains the User Info ID of itself, a list of protected user info of End UEs, a list of paths indicated by an (ordered) list of User Info ID of Relays (initially it only contains the User Info ID of UE-to-UE Relay1 in the figure), hop count (which is set to 1 initially) corresponding to each protected user info in the message and optionally Hop-Limit corresponding to each protected user info in the message. Each path corresponds to a user info in the Direct Discovery set.

The UE-to-UE Relay Discovery Announcement message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

3.	After receiving UE-to-UE Relay Discovery Announcement message(s) from another 5G ProSe UE-to-UE relay, a 5G ProSe UE-to-UE Relay checks the information associated to the received protected User Info(s). For each protected user info in the received message, if the corresponding path does not include the User Info ID of the 5G ProSe UE-to-UE Relay and the hop count is less than the corresponding Hop-Limit, the relay will add the protected user info into its own list of protected user info to be sent (if it is not added before) and add its own User Info ID into the path(s) of the corresponding protected user info(s) and increase the hop count by 1. If the relay already has the protected user info in its own list of protected user info to be sent with the hop count less than or equal to the received one, then it is up to relay's implementation to determine if add the protected user info again.

NOTE:	Based on implementation, Relay UEs can determine if two protected user infos are identical, e.g. by comparing the protected user info bit by bit if it is encrypted.

A 5G ProSe UE-to-UE Relay may associate the User Info ID and Layer-2 ID of neighbour UE-to-UE Relay(s) according to the Announcement message. The association may be used in the subsequent Link Management procedures.

4.	A 5G ProSe End UE or a 5G ProSe UE-to-UE Relay monitors Announcement message from a 5G ProSe UE-to-UE Relay. The 5G ProSe End UEs and 5G ProSe UE-to-UE Relays determine the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

* * * * Next Change * * * *

#### 6.4.3.8	Layer-2 link management over PC5 reference point for 5G ProSe Multi-hop UE-to-Network Relay (based on Model A discovery)

##### 6.4.3.8.1	Layer-2 Link establishment and management

Each of the 5G ProSe Intermediate UE-to-Network Relay shall establish a Layer-2 Link with its parent Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay, before it can serve the 5G ProSe Remote UE, or a child Intermediate UE-to-Network Relay. The 5G ProSe Intermediate UE-to-Network Relay shall establish the Layer-2 Link with a parent relay before sending the 5G ProSe UE-to-Network Relay Discovery Announcement message.

Layer-2 link management procedures as defined in clause 6.4.3.6 for 5G ProSe UE-to-Network Relay operation are used. When establishing the Layer-2 link with its parent relay (either a 5G ProSe UE-to-Network Relay or another 5G ProSe Intermediate UE-to-Network Relay), the 5G ProSe Intermediate UE-to-Network Relay takes the role of the 5G ProSe Remote UE and its parent relay takes the role of 5G ProSe UE-to-Network Relay. The Layer-2 link is established per RSC.

The 5G ProSe Remote UE uses the procedure defined in clause 6.4.5.6 to establish a Layer-2 Link with the selected 5G ProSe Intermediate UE-to-Network Relay. The Intermediate UE-to-Network Relay takes the role of of 5G ProSe UE-to-Network Relay in the procedure.

NOTE:	In the case of Layer-3 UE-to-Network Relay operation, only the root 5G ProSe UE-to-Network Relay needs to establish the PDU session with the network.

When an Intermediate UE-to-Network Relay has a child Intermediate UE-to-Network Relay or a 5G ProSe Remote UE connected, it updates the Layer-2 link with its parent relay, using the procedure defined in clause 6.4.3.4, with the following enhancements:

-	The link modification procedure is enhanced to inform the parent relay, e.g. another 5G ProSe Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay, of the User Info of the child relay(s) or the Remote UE(s). The Link Modification Request message contains a list of 5G ProSe Remote UE (Remote User ID, Remote UE Info, hop-count) and the corresponding PC5 QoS Flow associated with each 5G ProSe Remote UE. The Remote User ID and Remote UE Info are as defined in clause 6.5.1.1. The hop-count corresponds to the PC5 hops of the Remote UE from the network.

The Remote UE triggers the link modification procedure to its parent relay after obtaining the IP address/prefix from the 5G ProSe UE-to-Network Relay.

An Intermediate UE-to-Network Relay triggers a link modification procedure towards its parent relay if a new Remote User ID is received from its child relay during the link modification procedure.

The 5G ProSe UE-to-Network Relay's Remote UE reporting procedure includes all the Remote UE information reported by its child relays.

* * * * Next Change * * * *

##### 6.4.3.9.2	IP address/prefix management for 5G ProSe Layer-3 Multi-hop UE-to-Network Relay

When IP based PDU session types are used by the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay acts as an IP Router.

The IP address allocation is performed by the 5G ProSe UE-to-Network Relay, as defined in clause 6.5.1.

DHCP based IP address configuration and IPv6 routing advertisement is performed as described in clause 6.4.3.8.2.

After IP address assignment, the Remote UE may initiate a Link Modification Procedure which includes the Remote UE ID and the Remote UE information. The 5G ProSe Intermediate UE-to-Network Relays builds their local IP routing table based on the Remote UE info and the received multi-hop path info for UE-to-Network Relay during link establishment procedure for unicast PC5 connection (step 4 in clause 6.5.1.4 or step 3 in clause 6.5.1.5).

* * * * Next Change * * * *

##### 6.4.3.9.3	Relay reselection

After being connected to the 5G ProSe UE-to-Network Relay, the 5G ProSe Remote UE and 5G ProSe Intermediate UE-to-Network Relay keep performing the measurement of the signal strength of PC5 unicast link with its parent Relay (Intermediate Relay or U2N relay) for relay reselection.

When a 5G ProSe Intermediate UE-to-Network Relay detects the link failure or link quality degradation, the 5G ProSe Intermediate UE-to-Network Relay sends to the 5G ProSe Remote UE a Relay re-selection indication with the cause (e.g. the link failure or link quality degradation of the PC5 link) so that the 5G ProSe Remote UE may initiate multi-hop UE-to-Network Relay reselection procedure.

The 5G ProSe Remote UE may perform the 5G ProSe UE-to-Network Relay discovery procedures defined in clause 6.3.2.3 to discover available 5G ProSe UE-to-Network Relays or 5G ProSe multi-hop UE-to-Network Relay Discovery procedures defined in clause 6.3.2.5.3 to discover available multi-hop paths to the 5G ProSe UE-to-Network Relays for 5G ProSe (multi-hop) UE-to-Network Relay reselection and select a new path for 5G ProSe (multi-hop) UE-to-Network relay. Additionally, the 5G ProSe Remote UE may initiate link establishment procedure for unicast PC5 connection by sending a DCR or a LMR with the newly selected path info to the next hop 5G ProSe Intermediate UE-to-Network Relay at the newly selected path.

After the 5G ProSe multi-hop UE-to-Network relay reselection procedure, if the same 5G ProSe UE-to-Network Relay is used for connection with a 5G ProSe Remote UE, the existing IP address of the 5G ProSe Remote UE is reused. The IP address of the Remote UE may be included by the 5G ProSe UE-to-Network Relay and Intermediate UE-to-Network Relays in the DCA/LMA message.

After a preconfigured duration, the Relay that detected the need for reselection releases the link with the previous and next relays if applicable, unless it is reselected by the Remote UE. This will trigger them to perform the same procedure until the complete path has been released. Then, the 5G ProSe UE-to-Network Relay removes its association with the 5G ProSe Remote UE including the allocated IP address.

* * * * Next Change * * * *

##### 6.4.3.10.2	Layer-2 link establishement and management over PC5 reference point for non-IP PDU type

For the 5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-UE Relay as described in clause 6.7.1a, the description in clause 6.4.3.7.1 applies with following differences and clarifications:

-	The Direct Communication Request message over the PC5 reference point between source End UE and the first hop UE-to-UE Relay additionally includes:

-	Path Information: an (ordered) list of User Info ID of 5G ProSe UE-to-UE Relays selected by the source 5G ProSe End UE based on the path information provided to the source 5G ProSe End UE during 5G ProSe UE-to-UE Relay Discovery procedure.

-	QoS Info: indicates the End to End QoS Info from the source End UE to the target End UE.

The Direct Communication Request message over the PC5 reference point between two UE-to-UE Relays includes:

-	User Info of source 5G ProSe End UE.

-	User Info of target 5G ProSe End UE.

-	Path information.

-	ProSe Service Info: the information about the ProSe identifier(s).

-	RSC: the connectivity service provided by the 5G ProSe UE-to-UE Relay as requested by the source 5G ProSe End UE.

-	Security Information: the information for the establishment of security for the PC5 link establishment between two UE-to-UE Relays or between target End UE and UE-to-UE Relay (see TS 33.503 [29]).

-	QoS Info: indicates the rest QoS Info from the UE-to-UE Relay to the target End UE.

The Direct Communication Request message over the PC5 reference point between between the target End UE and UE-to-UE Relay additionally includes:

-	Path information.

-	QoS Info: indicates the rest QoS Info from the UE-to-UE Relay to the target End UE.

The Direct Communication Accept message over the PC5 reference point between target End UE and UE-to-UE Relay additionally includes:

-	QoS Info which is the accepted QoS Info of the hop between target End UE and the last UE-to-UE Relay.

The Direct Communication Accept message over the PC5 reference point between two UE-to-UE Relays includes:

-	User Info of target 5G ProSe End UE.

-	Path information.

-	QoS Info which is the accepted QoS Info of the hop between two UE-to-UE Relays and the accumulated QoS Info of hops from the receiver UE-to-UE Relay to the target End UE.

The Direct Communication Accept message over the PC5 reference point between the source End UE and UE-to-UE Relay additionally includes:

-	Path information.

-	QoS Info which is the accepted QoS Info of the hop between two UE-to-UE Relays and the accumulated QoS Info of hops from the source End UE to the target End UE.

The Link Modification Request message over the PC5 reference point between source End UE and the first hop UE-to-UE Relay additionally includes:

-	Path Information: an (ordered) list of User Info ID of 5G ProSe UE-to-UE Relays provided to the source 5G ProSe End UE during 5G ProSe UE-to-UE Relay Discovery procedure.

-	QoS Info: indicates the End-to-End QoS Info from the source End UE to the target End UE.

The Link Modification Request message over the PC5 reference point between two UE-to-UE Relays includes:

-	User Info of source 5G ProSe End UE.

-	Path Information.

-	User Info of target 5G ProSe End UE.

-	QoS Info: indicates the rest QoS Info from the UE-to-UE Relay to the target End UE.

The Link Modification Request message over the PC5 reference point between the target End UE and UE-to-UE Relay additionally includes:

-	Path Information.

-	QoS Info: indicates the rest QoS Info from the UE-to-UE Relay to the target End UE.

The Link Modification Accept message over the PC5 reference point between the target End UE and UE-to-UE Relay additionally includes:

-	Path information.

-	QoS Info which is the accepted QoS Info of the hop between target End UE and UE-to-UE Relay.

The Link Modification Accept message over the PC5 reference point between two UE-to-UE Relays includes:

-	Path information.

-	QoS Info which is the accepted QoS Info of the hop between two UE-to-UE Relays and the accumulated QoS Info of hops from the UE-to-UE Relay to the target End UE.

The Link Modification Accept message over the PC5 reference point between source End UE and the first UE-to-UE Relay additionally includes:

-	Path information.

-	QoS Info which is the accepted QoS Info of the hop between source End UE and the first UE-to-UE Relay and the accumulated QoS Info of hops from the UE-to-UE Relay to the target End UE.

NOTE:	The path information in DCR/LMR/DCA/LMA message is used for Multi-hop 5G ProSe UE-to-UE Relay Discovery only. A 5G ProSe UE-to-UE Relay Discovery that only supports single-hop UE-to-UE Relay will ignore the messages that contains path information.

In the Security Procedure of the PC5 reference point between source End UE and UE-to-UE Relay, the source 5G ProSe Layer-3 End UE provides the QoS Info of the end-to-end QoS to the 5G ProSe Layer-3 UE-to-UE Relay. If the PC5 link is used for transferring Ethernet traffic, the source 5G ProSe Layer-3 End UE provides its Ethernet MAC address. The 5G ProSe Layer-3 UE-to-UE Relay may provide the Ethernet MAC address of the target 5G ProSe Layer-3 End UE in the DCA/LMA message.

In the Security Procedure of the PC5 reference point between two UE-to-UE Relays, a 5G ProSe Layer-3 UE-to-UE Relay provides the QoS Info from itself to the target 5G ProSe Layer-3 UE-to-UE Relay. If the PC5 link is used for transferring Ethernet traffic, the 5G ProSe Layer-3 UE-to-UE Relay provides the Ethernet MAC address of the source 5G ProSe Layer-3 End UE. The 5G ProSe Layer-3 UE-to-UE Relay may provide the Ethernet MAC address of the target 5G ProSe Layer-3 End UE in the DCA/LMA message

In the Security Procedure of the PC5 reference point between target End UE and a UE-to-UE Relay, the 5G ProSe Layer-3 UE-to-UE Relay provides the QoS Info of the last hop QoS to the target 5G ProSe End UE. If the PC5 link is used for transferring Ethernet traffic, the 5G ProSe Layer-3 UE-to-UE Relay provides the Ethernet MAC address of the source 5G ProSe Layer-3 End UE. The target 5G End UE may provide its Ethernet MAC address in the DCA/LMA message

The 5G ProSe Layer-3 UE-to-UE Relay decides the QoS Info of the next hop to source End UE with considering the accumulated QoS Info of the hop(s) to the target End UE which is received in the Direct Communication Accept message.

For adding new end-to-end QoS flow or modifying existing end-to-end QoS flow, the Link Modification Accept message over each PC5 reference point additionally includes QoS Info of that hop. If the PC5 link is used for transferring Ethernet traffic, the target 5G ProSe Layer-3 End UE may provide its Ethernet MAC address. For a path between a specific pair of End UEs, if at least one of the PC5 link is released, the 5G ProSe Layer-3 UE-to-UE Relay(s) in the path may release other PC5 link(s) in the path.

* * * * Next Change * * * *

#### 6.5.1.4	5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay without N3IWF (based on Model B Discovery)

Figure 6.5.1.4-1: Connection establishment via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay without N3IWF support

Connection establishment via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay without N3IWF support as described in clause 6.5.1.1 is used for Multi-hop UE-to-Network Relay after Model B Discovery with the following differences and clarifications:

1.	Service authorization and provisioning are performed for the 5G ProSe Layer-3 UE-to-Network Relay, Intermediate Relay(s) and 5G ProSe Layer-3 Remote UE.

2.	The 5G ProSe Layer-3 UE-to-Network Relay may establish a PDU Session for relaying.

3.	The 5G ProSe Layer-3 Remote UE performs Model B discovery of a 5G ProSe Layer-3 UE-to-Network Relay. The Remote UE obtains the path information to the UE-to-Network Relay(s) from the discovery procedure.

4.	The 5G ProSe Layer-3 Remote UE selects a path and establishes a connection for unicast mode communication as described in clause 6.4.3.9.

5.	For IP PDU Session Type and IP traffic over PC5 reference point, the UE-to-Network Relay assigns IPv6 prefix or IPv4 address to Intermediate Relay(s) and the Remote UE as defined in clause 6.4.3.9.2.

6.	The 5G ProSe Layer-3 Remote UE may provide PC5 QoS Info and PC5 QoS rule(s) to the 5G ProSe Layer-3 UE-to-Network Relay using Layer-2 link modification procedure via Intermediate Relay(s). Intermediate Relays may split the PC5 QoS hop-by-hop as described in clause 5.6.2.

Editor's note:	It is FFS how to handle QoS.

7.	For the Remote UE Report, the Remote User ID is an identity of the 5G ProSe Layer-3 Remote UE.

For IP PDU Session Type, the Remote UE info is IP info related to Remote UE.

-	for IPv4, the 5G ProSe Layer-3 UE-to-Network Relay shall report IP address and TCP/UDP port ranges that are assigned to 5G ProSe Layer-3 Remote UE(s) by the UE-to-Network Relay;

-	for IPv6, the 5G ProSe Layer-3 UE-to-Network Relay shall report IPv6 prefix(es) assigned to individual 5G ProSe Layer-3 Remote UE(s).

For Ethernet PDU Session Type, the Remote UE info is Remote UE MAC address which is detected by the 5G ProSe Layer-3 UE-to-Network Relay.

NOTE:	For IP type of PDU Sessions, the Intermediate Relays forward the Downlink packet to the correct PC5 links based on destination IP address of the packet. For Ethernet type of PDU Sessions, the Intermediate Relay forwards the Downlink packets according to the destination Ethernet MAC address.

* * * * Next Change * * * *

#### 6.5.1.5	5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay with N3IWF after Model B Discovery

5G ProSe Communication via 5G ProSe Layer-3 UE-to-Network Relay with N3IWF as described in clause 6.5.1.2 is used for Multi-hop UE-to-Network Relay after Model B Discovery with the following differences and clarifications:

Figure 6.5.1.5-1: 5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay with N3IWF support

1.	5G ProSe Layer-3 UE-to-Network Relay performs Registration procedures and obtains the ProSe Policy that corresponds to the operation supporting the access to N3IWF. The 5G ProSe Layer-3 Remote UE is configured with the corresponding ProSe Policy and URSP rules.

Supporting of the RSC configured for making the 5G ProSe Layer-3 Remote UE access to 5GC via N3IWF is preconfigured or provisioned to the Intermediate Relay by the ProSe Policy.

2-4. A 5G ProSe Layer-3 UE-to-Network Relay, 5G ProSe Intermediate Relay(s) and 5G ProSe Layer-3 Remote UE perform multi-hop discovery using the RSC configured for making the 5G ProSe Layer-3 Remote UE access to 5GC via N3IWF. The Remote UE obtains the path information to the UE-to-Network Relay(s) from the discovery procedure. Details of discovery procedure and path establishement procedure are described in clause 6.3.2.5.3 and clause 6.4.3.9, respectively.

5.	The 5G ProSe Layer-3 Remote UE that connects to a 5G ProSe Layer-3 UE-to-Network Relay with N3IWF support selects an N3IWF and determines the N3IWF IP address.

6.	The 5G ProSe Layer-3 Remote UE establishes a signalling IPsec tunnel using IKE procedures with an N3IWF via Intermediate Relays and UE-to-Network Relay performs NAS Registration as described in clause 4.12.2.2 of TS 23.502 [5].

7.	Based on Additional QoS Information received from the N3IWF, the 5G ProSe Layer-3 Remote UE determines whether it is necessary to request for QoS session modification for the dedicated QoS Flows toward the 5G ProSe Layer-3 UE-to-Network Relay as described in clause 5.6.2.2, taking the number of hops into account.

* * * * Next Change * * * *

### 6.6.2	Registration procedure

The Registration procedure for UE is performed as defined in TS 23.502 [5] clause 4.2.2.2 with the following additions:

-	The UE includes the 5G ProSe Capability as part of the "5GMM capability" in the Registration Request message. The AMF stores the 5G ProSe Capability for 5G ProSe operation.

-	The 5G ProSe Capability indicates whether the UE supports one or more of the following ProSe capabilities:

-	5G ProSe Direct Discovery;

-	5G ProSe Direct Communication;

-	5G ProSe Layer-2 UE-to-Network Relay;

-	5G ProSe Layer-3 UE-to-Network Relay;

-	5G ProSe Layer-2 Remote UE;

-	5G ProSe Layer-3 Remote UE;

-	5G ProSe Layer-2 UE-to-UE Relay;

-	5G ProSe Layer-3 UE-to-UE Relay;

-	5G ProSe Layer-2 End UE;

-	5G ProSe Layer-3 End UE;

-	5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay;

-	5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay;

-	5G ProSe Layer-3 Intermediate UE-to-Network Relay;

-	5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 Multi-hop UE-to-Network Relay;

-	5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 Multi-hop UE-to-Network Relay;

-	5G ProSe Layer-2 Intermediate UE-to-Network Relay;

-	5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay;

-	5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay.

NOTE 1:	Encoding of these capabilities is defined in TS 24.501 [38].

-	The AMF obtains the 5G ProSe subscription data as part of the user subscription data from UDM during UE Registration procedure using Nudm_SDM service as defined in clause 4.2.2.2.2 of TS 23.502 [5].

-	The AMF determines whether the UE is authorised to use 5G ProSe services based on UE's 5G ProSe Capability and the ProSe Service Authorisation included in the subscription data received from UDM as specified in clause 5.7. ProSe NR UE-PC5-AMBR is also provided to the AMF as part of the subscription data for 5G ProSe services. The AMF stores the authorized 5G ProSe Capability.

-	The AMF sends the authorized 5G ProSe Capability for 5G ProSe operation to PCF. Based on the received 5G ProSe Capability from the AMF, the PCF provides the PC5 QoS parameters for 5G ProSe to AMF. The AMF stores such information as part of the UE context.

-	If the UE is authorised to use 5G ProSe services, then the AMF shall include in a NGAP message sent to NG-RAN:

-	"5G ProSe authorised" information, including one or more of the following:

-	whether the UE is authorized to use 5G ProSe Direct Discovery;

-	whether the UE is authorized to use 5G ProSe Direct Communication;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-3 UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 Remote UE;

-	whether the UE is authorized to use multi-path communication via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-UE Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 End UE;

-	whether the UE is authorized to act as a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 Multi-hop UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 Intermediate UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 Multi-hop UE-to-Network Relay.

NOTE 2:	The authorization for "5G ProSe Direct discovery" and "5G ProSe Direct communication" can be used for 5G ProSe Layer-3 UE-to-UE Relay, 5G ProSe Layer-3 End UE, 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay and 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay.

-	ProSe NR UE-PC5-AMBR, used by NG-RAN for the resource management of UE's PC5 transmission for 5G ProSe services in network scheduled mode.

-	the PC5 QoS parameters for 5G ProSe used by the NG-RAN for the resource management of UE's PC5 transmission for ProSe services in network scheduled mode.

-	If the UE is authorised to use 5G ProSe services, then the AMF should not initiate the release of the signalling connection after the completion of the Registration procedure. The release of the signalling connection relies on the decision of NG-RAN, as specified in TS 23.502 [5].

* * * * Next Change * * * *

#### 6.7.4.1	General

After being connected to the 5G ProSe UE-to-UE Relay, the 5G ProSe End UEs may trigger the 5G ProSe UE-to-UE Relay reselection based on conditions (e.g. the measured signal strength of PC5 unicast link with the 5G ProSe UE-to-UE Relay) as specified in TS 38.300 [12]. For 5G ProSe Multi-hop Layer-3 UE-to-UE Relay for non-IP PDU, when a 5G ProSe UE-to-UE Relay in the path detects link failure or link quality degradation to its adjacent 5G ProSe UE-to-UE Relay, it sends Relay reselection indication and optionally a cause value to 5G ProSe End UE. After receving the reselection indication the 5G ProSe End UE may trigger the Multi-hop Layer-3 UE-to-UE Relay reselection.

For 5G ProSe UE-to-UE Relay reselection, a 5G ProSe UE-to-UE Relay may be discovered by either the discovery procedures defined in clause 6.3.2.4 or by the negotiated 5G ProSe UE-to-UE Relay reselection procedure defined in clause 6.7.4.2 or clause 6.7.4.3. For 5G ProSe Multi-hop Layer3 UE-to-UE Relay for non-IP, in addition, a 5G ProSe UE-to-UE Relay may be discovered by the discovery procedure defined in clause 6.3.2.6.3, by the negotiated 5G ProSe Multi-hop Layer3-UE-to-UE Relay reselection for non-IP type PDU defined in clause 6.7.4.4.

In the negotiated UE-to-UE Relay reselection defined in clause 6.7.4.2 or clause 6.7.4.3, one 5G ProSe End UE initiates the UE-to-UE Relay reselection procedure, the 5G ProSe End UEs can negotiate a new 5G ProSe UE-to-UE Relay using the existing connection and to establish the communication via the reselected 5G ProSe UE-to-UE Relay prior to releasing the communication via the current 5G ProSe UE-to-UE Relay.

* * * * Next Change * * * *

#### 6.7.4.4	Negotiated 5G ProSe Multi-hop Layer-3 UE-to-UE Relay Reselection for Non-IP type PDU

Depicted in Figure 6.7.4.4-1 is the procedure for the negotiated 5G ProSe Multi-hop Layer-3 UE-to-UE Relay reselection.

Figure 6.7.4.4.-1. Negotiated Multi-hop Layer-3 UE-to-UE Relay reselection procedure.

1.	5G ProSe End UEs have set up PC5 unicast links with one or multiple 5G ProSe UE-to-UE Relay, based on the procedure defined in clause 6.7.5.

2.	5G ProSe End UEs are transferring data traffic e.g. Ethernet traffic, via 5G ProSe UE-to-UE Relay(s).

3.	The initiating 5G ProSe End UE determines, e.g. based on PC5 signal strength, to perform U2U Relay reselection.

4.	The initiating 5G ProSe End UE may discover new multi-hop U2U relay path(s) to the target 5G ProSe End UE.(refer to clause 6.3.2.6.3) as candidate path(s).

5.	The initiating 5G ProSe End UE sends a Link Modification Request to the responding 5G ProSe UE-to-UE Relay, which include a Relay re-selection indication and the candidate path information(s) and the User Info ID of the responding 5G ProSe End UEs.

6.	5G ProSe UE-to-UE Relay determines the responding 5G ProSe End UE based on the received User Info ID of End UEs in step 5 and sends a Link Modification Request message to the responding End UEs (via other UE-to-UE Relay(s) if the responding End UE is connected via other UE-to-UE Relay(s)).

The Link Modification Request message includes a Relay re-selection indication, the candidate path information(s).

7.	The responding 5G ProSe End UE may select a path from the candidate path information(s) in the received link modification as the newly selected multi-hop path.

8.	The responding 5G ProSe End UE may initiate PC5 connection setup or modification procedure for communication with the initiating 5G ProSe End UE with path info for the selected multi-hop path in step 7 (refer to clause 6.7.5.2.2). For Ethernet traffic, MAC address of the initiating 5G ProSe End UE and MAC address of the responding 5G ProSe End UE are included.

9.	The responding 5G ProSe End UE sends a Link Modification Accept message via the existing Multi-hop 5G ProSe UE-to-UE Relay(s) to the initiating 5G ProSe End UE. Link Modification Accept message includes the newly selected multi-hop path info, User Info ID of the iniating End UE, User Info ID of the responding End UE and Relay re-selection indication.

10.	The responding 5G ProSe UE-to-UE Relay sends a Link Modification Accept message to the initiating 5G ProSe End UE. The Link Modification Accept message includes the newly selected multi-hop path, User Info ID of the initiating End UE, User Info ID of the responding End UE and Relay re-selection indication.

11. The initiating 5G ProSe UE-to-UE Relay sends Link Modification Ack to the responding 5G ProSe UE-to-UE Relay.

12.	The 5G ProSe UE-to-UE Relay sends Link Modification Ack to the responding 5G ProSe End UE. Link Modification Ack includes User Info ID of the iniating End UE, User Info ID of the responding End UE and Relay re-selection indication.

13. 5G ProSe End UEs transfer traffic via the newly selected multi-hop path. For Ethernet traffic, existing MAC address of 5G ProSe End UEs are reused.

* * * * Next Change * * * *

##### 6.7.5.2.2	5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-UE Relay for Non-IP Type PDU

Figure 6.7.5.2.2-1: Layer-2 link establishment via 5G ProSe Layer-3 UE-to-UE Relay for Non-IP Type PDU

1.	5G ProSe Layer-3 End UE1 and 5G ProSe Layer-3 End UE2 are authorized for Multi-hop UE-to-UE Relay service as End UE and are provisioned with parameters for discovery and connection setup with other UEs via Multi-hop UE-to-UE Relay services.

5G ProSe Layer-3 UE-to-UE Relays are authorized for Multi-hop UE-to-UE Relay service as Relay UE and are provisioned with parameters for discovery and connection setup with other UEs and relay UEs via Multi-hop UE-to-UE Relay services (as described in clause 5.1.5.1). The provisioned parameter may include parameters such as RSC (Relay service Code)(s), list of PLMN, user info of UE for application which are allowed at multi-hop relay connection.

2.	It is assumed that the source 5G ProSe Layer-3 End UE1 selects a relay path to the Target End UE2 from the discovery procedure which is captured in clause 6.3.2.4.

3.	The source 5G ProSe Layer-3 End UE1 initiates the unicast Layer-2 link establishment procedure with the 5G ProSe Layer-3 UE-to-UE Relays by sending a Direct Communication Request message to the UE-to-UE Relay. The parameters included in the Direct Communication Request message are described in clause 6.4.3.10.2.

If there is already a PC5 link with the same RSC been established between the End UE and the UE-to-UE Relay or between UE-to-UE Relays, a Link Modification Request message is sent instead of Direct Communication Request message. The parameters included in the LMR message are described in clause 6.4.3.10.2.

4.	(Optional) When the security protection is enabled, the source 5G ProSe Layer-3 End UE sends the parameters as described in clause 6.4.3.10.2 to the 5G ProSe Layer-3 UE-to-UE Relay.

If 5G ProSe Layer-3 UE-to-UE relay detects the Ethernet MAC address of source 5G ProSe Layer-3 End UE is already used by another 5G ProSe Layer-3 End UE, then the 5G ProSe Layer-3 UE-to-UE Relay rejects the direct link establishment indicating that the MAC address is not unique.

5.	A UE-to-UE Relay sends a Direct Communication Request or Link Modification Request message to the next UE-to-UE Relay according to the path information in the received Direct Communication Request or Link Modification Request message.

The Source Layer-2 ID of the Direct Communication Request message is self-assigned by the UE-to-UE Relay and the Destination Layer-2 ID is the unicast Layer-2 ID associated with the User Info ID of the next hop UE-to-UE Relay. This association may be determined by UE-to-UE relay during discovery procedure in step 2.

6.	(Optional) When the security protection is enabled, the UE-to-UE Relays may establish security association as described in clause 6.4.3.10.2.

7.	A UE-to-UE Relay may know it is the last Relay in the path (e.g. according to the User Info ID of Relays in the received Direct Communication Request/Link Modification Request message). The UE-to-UE Relay sends a Direct Communication Request/Link Modification Request message to the target 5G ProSe Layer-3 End UE.

The Destination Layer-2 ID is the unicast Layer-2 ID of target End UE or a broadcast Layer-2 ID. Unicast Layer-2 ID is used if the Layer-2 ID of the Target End UE is known to the UE-to-UE Relay.

8.	(Optional) When the security protection is enabled, the UE-to-UE Relay (here, U2U Relay 2) may establish security association with the target 5G ProSe Layer-3 End UE as described in clause 6.4.3.10.2.

9.	The target End UE sends a Direct Communication Accept message to the UE-to-UE Relay it has successfully established security with. The DCA message may include the path information.

10-11.	After receiving a DCA message, the UE-to-UE Relay (e.g. Relay2) sends a DCA message to the next UE-to-UE Relay (e.g. Relay1) or Source End UE it has successfully established security with. The UE-to-UE Relay may decide the next UE-to-UE Relay (e.g. Relay1) according to the received DCA message or from which it previously received a corresponding DCR message.

NOTE 1:	In step 6, U2U Relay2 may detect the Ethernet MAC address of source 5G ProSe Layer-3 End UE is already used by another 5G ProSe Layer-3 End UE.

NOTE 2:	In step 9 and step 10, U2U Relay1 and U2U Relay2 may detect the Ethernet MAC address of target 5G ProSe Layer-3 End UE is already used by another 5G ProSe Layer-3 End UE.

For Ethernet communication, the 5G ProSe Layer-3 UE-to-UE Relay maintains the association between PC5 links and Ethernet MAC addresses received from the 5G ProSe Layer-3 End UE.

For Unstructured traffic communication, for each pair of source and target 5G ProSe Layer-3 End UEs, the 5G ProSe Layer-3 UE-to-UE Relay maintains the 1:1 mapping between two PC5 links to the (next hop to) source 5G ProSe Layer-3 End UE and the (next hop to) target 5G ProSe Layer-3 End UE.

* * * * End of Changes * * * *