3GPP SA WG2 Meeting #168 	S2-2502888

Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

Source:	KPN N.V.

Title:	Discussion on Ambient IoT Phase 2.

Document for:	Discussion

Agenda item:	30.6

# 1. Introduction

This paper discusses

# 2. Discussion

TS 22.369 captures the use cases, scenarios, and performance requirements for ambient power-enabled Internet of Things. The functional service requirements defined in SA1 are organized into six categories:

- Communications aspects;

- Positioning;

- Management;

- Exposure;

- Charging;

- Security and privacy.

The study on Architecture support of Ambient power-enabled Internet of Things in Release 19 addressed the initial architectural enhancements to support security, identification, connection management and Ambient IoT inventory and command services.

However, in Release 19, while the study work considered both Topology 1 and Topology 2, the scope of the normative work has been limited to only Topology 1. Thus, the work in Release 20 should add support for Topology 2.

Additionally, multiple aspects remain to be addressed:

Ambient IoT device and reader association:

When an Ambient IoT device is within reach from multiple readers, it might respond to all of them when prompted, which can increase network load. To minimize this, Ambient IoT device and reader association can be considered to configure the device to respond to a single reader.

Additionally, the performance service requirements as defined in TS 22.369 include scenarios where the device can be moving at different speeds. In case of Topology 2, the UE reader can also move away from the Ambient IoT device. In that case, mechanisms are needed to update the device and reader association accordingly.

Roaming support:

Ambient IoT use cases described in TR 22.840 include logistics scenarios where the devices are used to track merchandise that is shipped across countries. The 5G system needs to support the architecture and procedures to enable reachability of Ambient IoT device when moving between one or more networks and countries.

AIoT device management and configuration:

As specified in TS 22.369, mechanisms are required to temporarily or permanently disable Ambient IoT devices. It is also necessary to support mechanisms to provide or update the configuration of one or more Ambient IoT devices to perform services.

Positioning:

Support location services for Ambient IoT devices (e.g., to locate Ambient IoT devices using absolute or relative positioning methods) for tracking.

Energy availability considerations:

The Ambient IoT devices are energy constrained and may receive different amounts of energy from different sources. At the same time, the Ambient IoT service requirements as defined in TS 22.369 include different message sizes, communication ranges and transfer intervals, which means that the amount of required energy to perform a task differs. Thus, mechanisms are needed to support the Ambient IoT services based on energy availability at the device.

# 3	Proposal

It is proposed to include the following WTs in the scope of Ambient IoT Phase 2:

- WT#1 Architecture and procedures for support of Ambient IoT services considering Topology 2.

- WT#2 Ambient IoT device and reader association.

  - WT#1.1 Study how to support Ambient IoT device association to a reader;

  - WT#1.2 Study how to support Ambient IoT device association update in case of mobility of the device or reader.

- WT#3 Architecture to support Ambient IoT device reachability and communication in roaming scenarios.

- WT#4 Architecture to support AIoT device management and configuration

  - WT#3.1 Study how to support temporary disablement and re-enablement of AIoT devices;

  - WT#3.2 Study how to support configuration provisioning and update of one or a group of AIoT devices;

- WT#5 Architecture to support AIoT device positioning.

- WT#6 Support Ambient IoT services based on energy availability;