3GPP TSG SA WG2#168	            S2-2502895

Goteborg, Sweden, 07 – 11 April 2025	(revision of S2-2501062)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0297 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SMF triggered I-SMF insertion or removal for Local Offloading Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For an ongoing PDU session, PCF may send updated PCC rule(s) which may include target DNAI or DNAI(s) for the PDU session by invoking Npcf_ SMPolicyControl_UpdateNotify to SMF based on either internal mechanisms or an AF request. Such procedure is missing for Local Offloading Management case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies SMF triggered I-SMF selection or removal for Local Offloading Management |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification related to Local Offloading Management. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 6.10.2.X	SMF triggered I-SMF insertion/change/removal for Local Offloading Management

For a PDU session supporting Local Offloading Management, handling of updated PCC rule(s) which may include target DNAI or DNAI(s) for the PDU session is executed same as the respective SMF triggered I-SMF selection or removal defined in clause 4.23.5.4 of TS 23.502 [3] with the following difference:

- At step 2, when the content of Nsmf_PDUSession_SMContextStatusNotify or Nsmf_PDUSession_StatusNotify message includes the target DNAI information, the existing I-SMF of the PDU session is removed only if:

  - AMF decides that the target DNAI is not supported by the existing I-SMF, or

  - AMF receives a target DNAI information without target DNAI.

The target DNAI, if included, is used for selecting I-SMF at step 3.

- At step 3,

  - the AMF selects a new I-SMF which serves the target DNAI, supports Local Offloading Management, serves the UE location and local offloading management service area, or

  - the AMF removes the I-SMF if the AMF receives a target DNAI information without including the target DNAI.

In case if the AMF cannot find a new I-SMF that supports Local Offloading Management and local offloading management service area, the AMF performs the procedure defined in clause 4.23.5.4 of TS 23.502 [3] from step 3 onwards and skips the remaining steps in this clause.

- At step 4, the AMF also includes Local Offloading Allowed indication to the new I-SMF as part of Nsmf_PDUSession_CreateSMContext Request.

- At step 5 and 6, the procedure defined in clause 6.10.2.4 for I-SMF insertion/change/removal during UE mobility is used with the following difference:

  - For I-SMF insertion or I-SMF change case, the I-SMF selects the I-UPF based on the target DNAI,

  - For I-SMF removal case, the SMF selects a new I-UPF based on the target DNAI.

End of CHANGES