3GPP TSG SA WG 2 Meeting SA2#168	                                                                  TDoc S2-2502907

Gothenburg, Sweden, 2025-04-07 – 2025-04-11

Title:	DRAFT - LS reply on support for multiple non-3GPP device identifiers

Response to:	LS C1-250822 on LS on support for multiple non-3GPP device identifiers from CT1

Release:	Rel-19

Work Item:	UIA_ARC

Source:	Ericsson (will be SA2)

To:	CT1

Cc:	CT3

Contact person:	Magnus Hallenstål

magnus.l.hallenstal@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR on TS 23.502, S2-250xxxx, CR#nnnn, and S2-250yyyy, CR#nnnn

CR on TS 23.503, S2-250zzzz, CR#nnnn

# 1	Overall description

In LS on support for multiple non-3GPP device identifiers from CT1 the following questions were asked:

Q1: Whether the multiple non-3GPP device identifiers are allowed in a single PDU SESSION MODIFICATION REQUEST message?

Q2: For the Non-3GPP device identifier not available scenario, whether the SMF rejects the Non-3GPP device identifier or rejects the whole PDU SESSION MODIFICATION REQUEST message?

Q3: For the scenario of removal of more than one non-3GPP device identifier from the subscription in the UDR, whether a single or multiple non-3GPP device identifiers should be permitted in a single PDU SESSION MODIFICATION COMMAND message?

Answers:

Q1: SA2 recognise the possibility that allowing only a single non-3GPP Device Identity in a PDU Session Modification may increase the signalling overhead. To rectify this issue, SA2 has agreed CR S2-250xxxx and S2-250zzzz, which allows the PDU Session Modification to have multiple non-3GPP Device Identifiers.

Q2: In light of Q1, since multiple non-3GPP Device Identities can be used, the PDU session modification will only be rejected if all non-3GPP Device Identities within the PDU session modification request are not available, else, the SMF will provide the result indicating success or no success including cause per non-3GPP Device Identity in the response for which QoS differentiation was requested in a  PDU session modification request. Please see CR S2-250xxxx and S2-250zzzz.

Q3: If more than one non-3GPP Device Identifiers are removed and this affects more than one non-3GPP Device that are active (have gotten QoS differentiation), the 5GC can provide all the non-3GPP Device Identities are removed and that are active in the PDU SESSION MODIFICATION COMMAND message. See CR S2-250yyyy and S2-250zzzz.

# 2	Actions

To CT1

ACTION: Based on the answers above, SA2 asks CT1 to take any needed action

# 3	Dates of next TSG SA WG 2 meetings

SA2#169		2025-05-19 – 2025-05-23 Fukuoka City, Japan

SA2#170		2025-08-25 – 2025-08-29 Gothenburg, Sweden