3GPP TSG SA WG2 Meeting #168	S2-2502919

# 7-11 April 2025, Goteborg, Sweden										revision (S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5387 | rev | - | Current version: | 19.3.0 |  |
| 0 |  |  |  |  |  |  |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Immediate Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current description of the Nnef_ImsEE_Subscribe service operation includes as an optional output a first event report of the subscribed event. This is incorrect since it is unlikely that the subscribed event takes place exectly at the same time of the subscribe request. Instead, this should refer to the Immediate Reporting Flag that the AF can include within the Event Reporting Information. This flag indicate to the NEF that the current status related to the requested event type shall be included in the response to the subscription request, if applicable for the event type and available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The optional output parameter related to the Immediate Reporting Flag is clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear semantics of the information in Subscription Response in relation to the Immediate Reporting Flag in the request. If the current status related to the specific event type subscribed is counted as a event reporting (i.e. event detected), it may cause interoperability issues for AFs requiring a one-time notification on event detection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.39.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.228 CR 2909 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

5.2.6.39.2.1	Nnef_ImsEE_Subscribe service operation

Service operation name: Nnef_ImsEE_Subscribe

Description: The NF consumer subscribes for a specific IMS event.

Inputs, Required: Event-ID(s) representing the event of interest, Event Reporting Information as defined in Table 4.15.1-1.

Inputs, Optional: List one or more IMS Public Subscriber identity, Event Filters, Notification Target address, Notification Correlation Information, Expiry time, Subscription Correlation ID (in the case of modification of the event subscription), AF Identifier.

IMS Public subscriber identity entry is only present for subscriber specific IMS events.

NOTE:	The semantics of the Event ID and Event Filters are transparent to the NEF.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can expire based on the operator's policy).

Outputs, Optional: If applicable to the event type, the current status related to the subscribed event type is included, if Immediate Reporting Flag is included in the request within Event Reporting Information and if available (see clause 4.15.1).

NOTE X: If the current status related to the subscribed event type is available and included in the response, it is not considered as an event report (i.e. it is not considered a one-time notification on event detection). That is, it is not deducted from the maximum number of reports if included in the request within the Event Reporting Information.

**** End of Changes ****