3GPP TSG SA WG2 Meeting #168	S2-2502920

# 7-11 April 2025, Goteborg, Sweden										revision (S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1610 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Event Status Retrieval Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Subscribe/Notify Framework enables third party AF and network functions to subscribe to and be notified about IMS events. The framework inherits the Event Reporting Information defined in 5GC by which the AF consumer can request e.g. the current status related to the subscribed event type in the response to the subscription request and a maximum number of event report notifications. However, the framework does not currently enable an explicit procedure to retrieve the current status related to the requested event type. This can be critical if the AF requires the knowledge of the current status related to an event type e.g. before taking the decision to subscribe to a that particular event type or when just the current status is relevant for the AF. This is a situation currently also applicable to the Event Exposure framework in 5GC and, in the lack of this explicit event status retrieval procedure, what it is observed in deployments is that the subscribe operation is misused to serve the purpose of a status retrieval operation. Subscribe request includes the Immediate Reporting Flag to receive the current status in the response of the subscribe request and maximum number of event report notifications is set to its minimum value of 1. Given that the consumer does not know whether the current status is available in 5GC at the moment of subscription for certain subscribed event types, if the consumer is interested in a one-time notification upon event detection, there is no deterministic way for the consumer to ensure that it will receive a first (and unique) event detection if additionally the current related status of the subscribed event is detected. While this can serve the purpose of the AF to become aware of the current status related to an event type, the use of the subscribe request/response in this way is far from optimal from a RESTful API design point of view. This is also creating interoperability issues as at least one notification is provided to the NEF/AF as requested by the maximum number of reports while some implementations interprete that the current status in the response of the subscribe request serves already as the only notification report requested. An explicit and optimized procedure for event status retrieval is therefore required to avoid the misuse of the subscribe operations and avoid interoperability issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An explicit and optimized procedure for event status retrieval is specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The subscribe request is misused for the purpose of event status retrieval. Unoptimal use of Subscribe API and related resources. Interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA. 2.1.1; AA. 2.1.5.X (NEW); AA.2.4.1; AA.2.4.5.X (NEW); AD.1; AD.2.3; AD.3.X (NEW); AD.3.Y (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 5388 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### AA.2.1.1	General

The following table shows the services exposed by an SBI capable HSS.

Table AA.2.1.1-1: IMS Services provided by an SBI capable HSS

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| ImsSubscriber Data | Get | Request/Response | S-CSCF, I-CSCF, AS, DCSF |
| Management (_ImsSDM) | Subscribe | Subscribe/Notify | S-CSCF, AS |
|  | Unsubscribe | Subscribe/Notify | S-CSCF, AS |
|  | Notification | Subscribe/Notify | S-CSCF, AS |
|  | Update | Request/Response | AS, DCSF |
| Ims UE Context | Registration | Request/Response | S-CSCF, IMS AS |
| Management | DeregistrationNotification | Subscribe/Notify | S-CSCF |
| (_ImsUECM) | Deregistration | Request/Response | S-CSCF, IMS AS |
|  | Authorize | Request/Response | I-CSCF |
|  | Update | Request/Response | S-CSCF |
|  | RestorationInfoGet | Request/Response | S-CSCF |
|  | RestorationInfoUpdate | Request/Response | S-CSCF |
| ImsUE Authentication | AsInfoGet | Request/Response | NEF |
| (_ImsUEAU) | Get | Request/Response | S-CSCF |
| ImsEventExposure | Subscribe | Subscribe/Notify | NEF, Trusted AF |
| (_ImsEE) | Unsubscribe |  | NEF, Trusted AF |
|  | GetStatus | Request/Response | NEF, Trusted AF |

**** Next Change ****

#### AA.2.1.5.X	Nhss_ImsEE_GetStatus service operation

Service operation name: Nhss_ImsEE_GetStatus

Description: The NF consumer requests the current status related to the requested subscriber specific IMS event type.

Inputs, Required: IMS Public Subscriber identity, Event-ID(s) representing the event of interest.

Event-ID represents one or multiple events as defined in clauses AD.2.4, and AD.2.5.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: Current status related to the requested IMS event type, if applicable to the event type and available.

**** Next Change ****

### AA.2.4.1	General

The following table shows the IMS AS Services and IMS AS Service Operations.

Table AA.2.4.1-1: NF services provided by the IMS AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_SessionEventControl | Subscribe | Subscribe/Notify | DCSF |
|  | Notify | Subscribe/Notify | DCSF |
| Nimsas_MediaControl | MediaInstruction | Request/Response | DCSF |
| Nimsas_ImsSessionManagement | Create | Request/Response | NEF, Trusted AF |
|  | Update | Request/Response | NEF, Trusted AF |
|  | Delete | Request/Response | NEF, Trusted AF |
|  | Notify | Subscribe/Notify | NEF, Trusted AF |
| Nimsas_ImsEventExposure (ImsEE) | Subscribe | Subscribe/Notify | NEF, HSS, Trusted AF |
|  | Unsubscribe | Subscribe/Notify | NEF, HSS, Trusted AF |
|  | Notify | Subscribe/Notify | NEF, Trusted AF |
|  | GetStatus | Request/Response | NEF, HSS, Trusted AF |
| Nimsas_ImsParameterProvision (ImsPP) | Create | Request/Response | NEF, Trusted AF |
|  | Update | Request/Response | NEF, Trusted AF |
|  | Delete | Request/Response | NEF, Trusted AF |

NOTE 1:	In this Release the IMS AS services is introduced to support Data Channel services in IMS further described in Annex AC.

NOTE 2:	There shall be no overlap between events supported in Nimsas_ImsEventExposure service vs events implicitly supported in Nimsas_SessionEventControl service.

**** Next Change ****

#### AA.2.4.5.X	Nimsas_ImsEE_GetStatus service operation

Service operation name: Nimsas_ImsEE_GetStatus

Description: The NF consumer requests the current status related to an IMS event type.

Inputs, Required: Event-ID(s) representing the related status of interest.

Event-ID represents one or multiple events as defined in clauses AD.2.4, and AD.2.5.

Inputs, Optional: For subscriber specific IMS events the IMS Public Subscriber identity is included.

Outputs, Required: Result.

Outputs, Optional: Current status related to the requested IMS event type, if applicable and available.

**** Next Change ****

# AD.1	General

This annex describes the Subscribe/Notify framework to support event monitoring in the IMS architecture. The framework enables third party AF and network functions to subscribe to IMS DC events.

IMS event exposure for event monitoring allows a consumer (e.g. a DC AS) to subscribe to and being notified about subscriber specific and non-subscriber specific IMS events and to invoke IMS services exposed by the IMS network via the NEF.

A subscriber specific event monitoring request includes the targeted subscribers. The targeted subscribers can be either one distinct subscriber or multiple subscribers indicated by their IMS public user identities. The events in the subscriber specific event monitoring request are common to all targeted subscribers. The NEF sends one event subscription request per targeted subscriber to the HSS serving the given subscriber. The NEF may determine the serving HSS based on configuration or based on HSS discovery as depicted in clause AA.3.3.

A non-subscriber specific event monitoring request does not contain any target subscriber. Non-subscriber specific event monitoring requests are sent by the NEF directly to all IMS AS instances which support monitoring of the requested IMS event. The NEF may be configured with available IMS AS instances, or the NEF can discover IMS AS instances via NRF.

Once an IMS subscriber successfully registers in IMS, and one of the IMS AS is assigned to the subscriber, the IMS AS which supports monitoring of IMS events registers its address in the HSS.

For subscriber specific event monitoring requests received from the AF (e.g. DC AS), the NEF uses Nhss_ImsEE service to manage the subscription requests (e.g. subscribe/unsubscribe) via HSS. HSS uses Nimsas_ImsEE service to manage the subscriber specific subscription requests with the IMS AS assigned to the subscriber.

The framework also supports the request of the current status related to an IMS event type. This allows a consumer to retrieve the current status e.g. before taking the decision to subscribe to a particular event or when just the current status is relevant for the AF. For subscriber specific events, the NEF retrieves the current status related to the IMS event type to the IMS AS via the HSS. For non-subscriber specific events, the NEF retrieves the current status related to the IMS event type directly from the IMS AS.

**** Next Change ****

## AD.2.3	Functional Entities

### AD.2.3.1	NEF

The NEF, in addition to its role as defined TS 23.502 [94], is the entry point for subscriptions to notifications for monitoring of IMS related events and/or retrieval of the status related to an event type, via N33 reference point using the Nnef_ImsEE_Subscribe service operation.

An incoming subscription and/or retrieval of the status related to an event type request to the NEF for a subscriber specific event may include a single IMS subscriber or a list of IMS subscribers. The NEF initiates an individual request to the HSS instance corresponding to each IMS subscriber included in the incoming request from the AF.

NEF initiates a subscription and/or retrieval of the status related to an event type request for subscriber specific events towards HSS via N91 reference point using the Nhss_ImsEE_Subscribe and/or Nhss_ImsEE_GetStatus service operations.

NEF initiates a subscription and/or retrieval of the status related to an event type request for non-subscriber specific events towards applicable IMS AS instances via N92 reference point using the Nimsas_ImsEE service and/or Nimsas_ImsEE_GetStatus service operations.

NEF receives notifications corresponding to subscribed IMS events from the applicable IMS AS instances via N92 reference point using the Nimsas_ImsEE_Notify service operation. NEF sends the received notifications to the requesting AFs via N33 reference point using the Nnef_ImsEE_Notify service operation.

### AD.2.3.2	HSS

HSS handles subscriptions to IMS DC events related to registered and unregistered IMS subscribers and ensures that a subscription is persistent when UE registers, deregisters and re-registers in IMS.

HSS receives subscription and/or retrieval of the status related to an event type requests for subscriber specific events from NEF via N91 reference point using the Nhss_ImsEE_Subscribe and/or Nhss_ImsEE_GetStatus service operations.

HSS stores the incoming subscription request and sends the subscription request to the IMS AS instance supporting the requested event and assigned to the IMS subscriber via N71 reference point, using the Nimsas_ImsEE_Subscribe service operation, or via Sh reference point.

HSS forwards the retrieval of the status related to an event type to the IMS AS instance supporting the requested event type and assigned to the IMS subscriber via N71 reference point, using the Nimsas_ImsEE_GetStatus service operation, or via Sh reference point.

The IMS AS assigned to an IMS subscriber is registered in HSS via N71 reference point using the Nhss_ImsUECM_Registration service operation, or via Sh reference point. If the IMS subscriber in the incoming subscription request is not registered, HSS ensures that when the IMS subscriber registers, any pending subscription request is sent to the applicable IMS AS instance assigned to the IMS subscriber. IMS AS rejects subscriptions to IMS DC events for a subscriber that is not subscribed to IMS DC service.

HSS ensures that corresponding notifications are directly sent to the NEF as subscribing entity, and not via HSS.

### AD.2.3.3	IMS AS

Once an IMS subscriber successfully registers in IMS, any DC supporting IMS AS allocated to the subscriber that supports monitoring of IMS events shall register in HSS its IMS AS instance via N71 reference point using the Nhss_ImsUECM_Registration service operation, or via Sh reference point.

The IMS AS receives subscription requests from HSS related to supported subscriber specific IMS DC events via the N71 reference point using the Nimsas_ImsEE_Subscribe service operation, or via the Sh reference point. IMS AS also receives subscription requests from NEF related to non-subscriber specific events via the N92 reference point using the Nimsas_ImsEE service operation.

IMS AS sends notification events directly to the NEF as subscribing entity included in the subscription request via N92 reference point using the Nimsas_ImsEE_Notify service operation.

The IMS AS receives request from HSS or NEF via N71 or N92 reference points respectively using the Nimsas_ImsEE_GetStatus service operation for retrieving the status of an IMS event type.

**** Next Change ****

## AD.3.X	Event Status Retrieval Procedure for Subscriber Specific IMS Events

Figure AD.3.X-1 depicts a signalling flow diagram for requesting the retrieval of the current status related to a subscriber specific IMS event type.

Figure AD.3.X-1: Event Status Request Procedure for Subscriber specific IMS event types

The steps in the call flow are as follows:

1.	UE performs initial IMS Registration as per clause AD.3.1.

2.	AF requests the current status related to a subscriber specific IMS event type to the NEF initiating the Nnef_imsEE_GetStatus Request. The AF may include one or more IMS subscriber IDs in the subscription request.

3.	NEF initiates for each IMS subscriber in the incoming event status request a separate retrieval of the status related to an event type request towards HSS via the Nhss_ImsEE_GetStatus Request service operation.

4.	HSS locates the IMS AS instance serving the UE based on the IMS AS registration in HSS executed in step 1. The HSS determines if the IMS AS instance for the IMS subscriber (IMPU) supports the related IMS event type based on the NF profile of the registered IMS AS instances stored in NRF.

If the UE is not IMS registered in HSS, steps 5 and 6 are skipped.

5.	HSS sends the retrieval of the status related to an event type request to the IMS AS instance serving the UE and supporting the requested IMS event, using Nimsas_ImsEE_GetStatus Request for the related IMS event including the IMPU. Alternatively, HSS can use Sh interface to send the retrieval request of the current status related to an event type to the IMS AS instance assigned to the UE.

6.	IMS AS returns to HSS the current status related to the requested subscriber specific IMS event type with the Nimsas_ImsEE_GetStatus Response or via Sh interface.

7.	HSS returns to NEF the Nhss_ImsEE_GetStatus Response.

8.	NEF returns to AF the Nnef_ImsEE_GetStatus Response.

**** Next Change ****

## AD.3.Y	Event Status Retrieval Procedure for Non-Subscriber Specific IMS Events

Figure AD.3.Y-1 depicts a signalling flow diagram for requesting the retrieval of the current status related to a non-subscriber specific IMS event type.

Figure AD.3.Y-1: Event Status Request Procedure for Non-Subscriber specific IMS event types

The steps in the call flow are as follows:

1.	AF requests the current status related to a non-subscriber specific IMS event type to the NEF initiating the Nnef_imsEE_GetStatus Request.

2.	NEF locates the IMS AS instances that support the related IMS event via NRF or by local configuration.

3.	For each IMS AS instance that supports the related IMS event, NEF initiates a separate retrieval of the status related to the event type request using Nimsas_ImsEE_GetStatus Request for the related IMS event.

4.	IMS AS returns to NEF the current status related to the requested non-subscriber specific IMS event type with the Nimsas_ImsEE_GetStatus Response.

5.	NEF returns to AF the Nnef_ImsEE_GetStatus Response.

**** End of Changes ****