3GPP TSG-SA WG2#168	S2-2502927

Goteborg, Sueden, 07-11 April 2025

Source:	Telefonica S.A., Verizon, Deutsche Telekom, KDDI, Ericsson, Nokia

Title:	Discussion paper on GSMA TSGNS AppToken use case

Document for:	Discussion

Agenda Item:	4.1

Work Item / Release:	TEI19/Rel. 19

Abstract of the contribution: the paper provides clarifications about various questions raised related to the AppToken functionality of GSMA TSGNS group.

# 1.	Introduction

SA2 received the LS from GSMA TSGNS group on this regard (App Token) (S2-2502797). The following figure is included in the LS:

# 2.	Discussion

There were some comments made in S2-2501497 and the intention of this discussion paper is to provide some clarifications related to them.

## 2.1 App token validity period

- As per GSMA TS.43 v12 section 14.2.1: “App token may be used only once, and once it is consumed by a device invoking the Get3ppAppInfo operation, it shall become useless (further operations shall return an error). Additionally, it should have associated a short expiration time (minutes range).”.

- This expiration time / validity period is an additional method to make the token useless.

- This expiration time / validity period has no relationship with URSP rule validity period (see next point), it is just the period of validity of the App token, for the OS to get the 3pAppInfo managed in the operator and properly identify an app.

- UE/OS does not need to manage App token validity period, this is done by the Entitlement Server and the app level (app is aware about the validity period).

- Once the OS has received the anonymized identity (YYY) associated with the App token from the MNO entitlement server, the app does not need to request a new token from the MNO to access the same connectivity service. The CR1041 for TS.62 proposes that the app token can still be used between the app and the OS to request access to such connectivity service (OS maintains a mapping between the app token, the application and the anonymous identifier).

## 2.2 URSP validity period

- URSP is defined in 3GPP TS 23.503 and TS 24.526 section 4.2 while URSP configuration in the UE is defined in TS 23.502 section 4.2.4.

- URSP validity period has no relationship with app token validity period.

- It is an MNO operational aspect when to provision or to delete a URSP rule for a device.

- If a URSP rule is matched, it is applied. If a URSP rule is not matched, it is skipped by using precedence.

## 2.3 Target UEs for URSP rules

## It is up to the MNO how to provision URSP rules, MNOs already have procedures in place to do this and this LS is asking for nothing new.

## The 3rd party establishes a trusted relationship with the MNO asking for App tokens.

## The interface between the 3rd party and the MNO is in GSMA scope.

## 2.4 Anonymized identity

- As per GSMA TS.43 v12 section 14.2.1, anonymized identity (YYY in the previous picture) does not contain any subscriber related information but anonymously identifies an application. It can be used in several URSP rules sent to several subscribers/UEs.

- Other mechanisms that allow applications to request a specific network connection based on DNN, Traffic Categories (Connection Capabilities) or FQDN traffic descriptor component, or allow the network to replace the network slice being used on an existing network connection do not address App token use case requirements. Specifically, these mechanisms don't allow an MNO to authenticate an application and authorize its access to a given network connection. In addition, these mechanisms don't meet the implicit GSMA requirement to be able to hide the actual identity of an application from the MNO.

# 3.	Conclusions

The proposed e2e flow partially uses a 3GPP defined procedure, i.e. URSP rules configuration, and the procedure is well applied.

Rest of procedures and interfaces involving the Operator Entitlement Server are not in scope of 3GPP, but in scope of GSMA. 3GPP assumption is that GSMA has already analysed its own procedures and interfaces in their scope. Based on the analysis performed in this paper, we propose the following answers to the GSMA LS questions:

1. Do the existing 3GPP specifications allow the network to use application identities that are generated by an application backend to populate URSP in step 2?

Application Function (AF) guidance on URSP allows AFs to provide URSP rules with any OS App Id; please refer clause 4.15.6.10 of TS 23.502 for details. SA2 has not defined how the application identities are generated. 3GPP SA2 assumes that the uniqueness of OS App Ids, if required, is the responsibility of the specific implementation.

2. Would 3GPP SA2 prefer an alternate mechanism to preserve end user privacy?

We recommend SA3 answering the user privacy aspect.

3. Consider the architectural implications of the proposed mechanism and provide feedback.

Please refer to the above answers of 1 and 2.

From SA2 specification perspective, the proposed mechanism has no impact on current 3GPP specifications.