3GPP TSG-SA2 Meeting #168	S2-2502937

# Goteborg, Sweden, 07 – 11 April 2025	(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0192 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on procedure for instructing aerial UEs for altitude reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3, TEI19 |  |  |  |  |  | Date: |  |  | 2025-03-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The LS on UAV regulation from RAN3 (S2-2502828/R3-250876) asks the following question to SA2. With regards to the “node-level signalling” solution configuration in clause 5.16.2, RAN3 would like to ask for further feedback from SA2 on whether the CN needs to indicate derived altitude thresholds and reporting periodicity for the aerial UE's altitude reporting. Therefore, the above aspect needs to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §5.16.2 - Clarify that altitude thresholds and reporting periodicity provided from AMF to NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In procedure for instructing aerial UEs for altitude reporting, information from AMF to NG-RAN may be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.16.2	Instructing aerial UEs to perform altitude reporting

Procedure shown in Figure 5.16.2-1 specifies how the 5GS instructs the aerial UEs (UAVs) to perform altitude reporting, considering requests from a USS/UTM for assistance with UAV's flight. This procedure can be used in conjunction with the NEF-assisted pre-flight planning, in-flight monitoring and USS changeover as specified in clauses 5.12.2, 5.12.3 and 5.13, respectively.

Figure 5.16.2-1: Procedure for instructing aerial UEs for altitude reporting

1.	UAS NF/NEF assists the USS/UTM with pre-fligh planning, in-flight monitoring and/or USS changeover based on the received request, which is either a Nnef_RetrieveInfoUAVFlight_Get or a Nnef_UAVFlightAssistance_Create, as specified in clauses 5.12 or 5.13.

2.	Based on the flight path information included in step 1 and other information obtained as a result of either of the procedures (i.e. NEF-assisted pre-flight planning specified in clause 5.12.2, NEF-assisted in-flight monitoring specified in clause 5.12.3, or USS changeover specified in clause 5.13.2), the UAS NF/NEF determines minimum and maximum values of altitudes (i.e. altitude range) for UAV's flight across each of the determined tracking areas (TAs) from the starting point to the destination point.

The determined minimum and maximum altitude values for each of the UAV UE is used by the UAS NF/NEF to derive altitude thresholds for altitude reporting events.

NOTE 1:	Event H1 and H2 for altitude-based reporting specified in clause 5.5.4.21 and clause 5.5.4.22 of TS 38.331 [23] can be used for altitude thresholds and event-based reporting.

Additionally, the UAS NF/NEF determines whether to request the aerial UE to report its altitude information periodically, threshold-based (e.g. altitude becomes a higher/lower than the provided threshold, multiple threshold values can be provided) or event-based (e.g. change of a tracking area/NG-RAN node).

3.	Delivering the UAS NF/NEF-derived altitude thresholds and, optionally, the indication about reporting periodicity (i.e. threshold-based, event-based, or periodic) for UE's altitude reporting to the aerial UE(s).

Option A: Application layer.

3a.	The UAS NF/NEF sends the derived altitude threshold values and provides indication about the required periodicity of the reporting (on demand, event-triggered, periodic, or per change of the altitude, or change of a specific geographical area) to the serving USS/UTM either in a Nnef_RetrieveInfoUAVFlight_Get response message or in a Nnef_UAVFlightAssistance_Notify message depending on the procedure that has triggered the UAS NF/NEF to derive the altitude thresholds for the aerial UE.

4a.	The USS/UTM sends the altitude reporting thresholds and, optionally, indication about reporting periodicity (and any additional information required to perform the reporting, e.g. a list of TA(s)/NG-RAN node identifier(s) for event-based reporting, periodicity value for periodic reporting) to the UAV over the application layer for the UAV's consumption.

NOTE 2:	Application layer reporting itself is outside of 3GPP scope.

Option B: Node-level signalling.

Editor's note:	This requires RAN to provide further enhancement of the procedure under RAN responsibility.

3b.	The UAS NF/NEF may send an Namf_EventExposure_Subscribe request to the AMF to get notified about events of UAV's deviation from the assigned trajectory (i.e. Event ID = "Assigned Trajectory") as specified in clause 5.2.2.3.1 of TS 23.502 [3]. Inside the subscription request, the UAS NF/NEF may include the derived altitude thresholds and indication about reporting periodicity for the UE's altitude reporting.

Additionally, the UAS NF/NEF may also include an indication to perform UE's altitude reporting for all aerial UEs in a specific TA(s) or NG-RAN node(s).

4b.	The AMF sends the corresponding N2 messages to all applicable NG-RAN node(s). For UE's altitude reporting, the AMF includes the altitude thresholds and/or reporting periodicity in the N2 messages.

5b.	The NG-RAN node(s) instructs the relevant UE(s) to perform altitude reporting as requested by the UAS NF/NEF (via AMF).

NOTE 3:	Aspects related to AMF informing NG-RAN regarding instructing UEs to perform altitude reporting are specified in TS 38.413 [24].

* * * * End of Changes * * * *